3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003786

Online, 20 – 30 March 2020

Agenda item:	7.2.4

Source:	Session Chair (Huawei)

Title:	Summary of SON/ANR open issues

WID:	NB_IOTenh3-Core

Document for:	Discussion and Decision

# 1	Introduction

This document contains a summary of SON/ANR documents from agenda item 7.2.4 as referenced in Section 4 in order to facilitate decision making at RAN2#109bis-e.

# 2	Summary

## 2.1	Summary of open issues related to FFS from previous meetings

The following proposals are covered in this section

| [4] | Huawei, HiSilicon | Proposal 1: Confirm the Working assumption that the ANR report is discarded after 96 hours. Proposal 2: ANR measurement report is discarded upon RAT change. Proposal 3: There is no need for a time indication of when the measurements were performed in the ANR measurement report. Proposal 4: The RLF report is discarded upon returning to idle if rlf-InfoAvailable has been reported. Proposal 5: RLF report is discarded upon RAT change. Proposal 6: The re-establishment cell ID is included in the RLF report. |
| --- | --- | --- |
| [5] | ZTE | Proposal 1: Confirm the working assumption that the validity timer is fixed to 96 hours. Proposal 2: It’s suggested to include timeSpent information in ANR report to indicate the elapsed time since the generation of ANR record. And the value range of timeSpent is suggested to be INTEGER (0..5760) with unit of minutes. Proposal 3: The re-establishment cell id need to be included in the RLF report as an optional IE. Proposal 4: UE can discard RLF in the following cases:  After 48 hours if not fetched.  Power off or detach.  Reporting rlf-InfoAvailable and returning to idle. |

### 2.1.1	Potential Easy agreements

Proposal 1 of [4] and [5] are the same.

[4] Proposal 1: Confirm the Working assumption that the ANR report is discarded after 96 hours.

[5] Proposal 1: Confirm the working assumption that the validity timer is fixed to 96 hours.

It is proposed to confirm, based on company input so far.

Proposal S1-1: Confirm the Working assumption that the ANR report is discarded after 96 hours.

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |

Proposal 2 of [4].

[4] Proposal 2: ANR measurement report is discarded upon RAT change.

It is proposed to confirm, based on company input so far.

Proposal S1-2: ANR measurement report is discarded upon RAT change.

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |

Proposal 6 of [4] and proposal 3 of [5] are the same:

Proposal 6: The re-establishment cell ID is included in the RLF report.

Proposal 3: The re-establishment cell id need to be included in the RLF report as an optional IE.

It is proposed to confirm, based on company input so far.

Proposal S1-3: Re-establishment cell ID is included in the RLF report.

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |

### 2.1.2	Needs further discussion

Proposal 3 of [4] and proposal 2 of [5] address the same issue but with opposite view.

[4] Proposal 3: There is no need for a time indication of when the measurements were performed in the ANR measurement report.

[5] Proposal 2: It’s suggested to include timeSpent information in ANR report to indicate the elapsed time since the generation of ANR record. And the value range of timeSpent is suggested to be INTEGER (0..5760) with unit of minutes.

Since there are opposing views, the following “positive” proposals are put forward for discussion.

Proposal S1-4a: [FFS] Include timeSpent information in ANR report to indicate the elapsed time since the generation of ANR record

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |

Proposal S1-4b: [FFS] timeSpent is defined as INTEGER (0..5760) with unit of minutes

Company views (to be completed during the meeting if you answered “yes” to S1-3a)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |

Proposals 4 and 5 of [4] and proposal 5 of [5] propose various cases in which RLF report is discarded.

[4] Proposal 4: The RLF report is discarded upon returning to idle if rlf-InfoAvailable has been reported.

[4] Proposal 5: RLF report is discarded upon RAT change.

[5] Proposal 4: UE can discard RLF in the following cases:

 After 48 hours if not fetched.

 Power off or detach.

 Reporting rlf-InfoAvailable and returning to idle.

One of the options was already agreed in the previous meeting, RAN2#109-e:

RLF report is discarded after 48 hours if not fetched.

Since there multiple additional options, the following “positive” proposal is put forward for discussion.

Proposal S1-5: [FFS] RLF report is discarded in the following cases:

- Reporting rlf-InfoAvailable and returning to idle.

- RAT change

- Power off or detach.

- (already agreed) after 48 hours if not fetched

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | (a) – yes/no (b) – yes/no (c) – yes/no |  |

## 2.2	Summary of new proposals

The following proposals are covered in this section

| [1] | Ericsson, Nokia, Nokia Shanghai Bell, ZTE Corporation | Proposal 1 Send a LS to RAN4 to verify that the ANR measurements specified by RAN2 would work fine. |
| --- | --- | --- |
| [2] | Ericsson | Proposal 1 UE stores the CE level of the Last Serving Cell for RLF Proposal 2 UE stores the CE level of serving cell when ANR measurement is being performed. Proposal 3 UE stores the change of CE level and corresponding RSRPs when ANR measurement is being performed. Proposal 4 UE stores the CE level of the target cell during ANR measurement when possible. |

### 2.2.1	Needs further discussion

The following proposal regarding sending an LS to RAN4 is made in [1]

Proposal 1 Send a LS to RAN4 to verify that the ANR measurements specified by RAN2 would work fine.

Proposal S2-1: [FFS] Send a LS to RAN4 to verify that the ANR measurements specified by RAN2 would work fine.

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |

The following proposals related to the storing of CE level information are made in [2]

Proposal 1 UE stores the CE level of the Last Serving Cell for RLF

Proposal 2 UE stores the CE level of serving cell when ANR measurement is being performed.

Proposal 3 UE stores the change of CE level and corresponding RSRPs when ANR measurement is being performed.

Proposal 4 UE stores the CE level of the target cell during ANR measurement when possible.

Proposal S2-1: [FFS] UE stores the following CE level information :

- CE level of the Last Serving Cell for RLF.

- CE level of serving cell when ANR measurement is being performed

- Change of CE level and corresponding RSRPs when ANR measurement is being performed

- CE level of the target cell during ANR measurement when possible

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments |
| --- | --- | --- |
|  | (a) – yes/no (b) – yes/no (c) – yes/no (d) – yes/no |  |

# 3	Conclusions (to be updated following offline)

Potential easy agreements:

Proposal S1-1: Confirm the Working assumption that the ANR report is discarded after 96 hours.

Proposal S1-2: ANR measurement report is discarded upon RAT change.

Proposal S1-3: Re-establishment cell ID is included in the RLF report.

Needs further discussion:

Proposal S1-4a: [FFS] Include timeSpent information in ANR report to indicate the elapsed time since the generation of ANR record

Proposal S1-4b: [FFS] timeSpent is defined as INTEGER (0..5760) with unit of minutes

Proposal S1-5: [FFS] RLF report is discarded in the following cases:

- Reporting rlf-InfoAvailable and returning to idle.

- RAT change

- Power off or detach.

- (already agreed) after 48 hours if not fetched

New proposals for further discussion:

Proposal S2-1: [FFS] Send a LS to RAN4 to verify that the ANR measurements specified by RAN2 would work fine.

Proposal S2-1: [FFS] UE stores the following CE level information :

- CE level of the Last Serving Cell for RLF.

- CE level of serving cell when ANR measurement is being performed

- Change of CE level and corresponding RSRPs when ANR measurement is being performed

- CE level of the target cell during ANR measurement when possible

# 4	List of referenced documents

- R2-2003131	To Verify ANR Measurements	Ericsson, Nokia, Nokia Shanghai Bell, ZTE Corporation	discussion	Rel-16

- R2-2003133	Logging of CE Level for RLF and ANR measurements	Ericsson	discussion	Rel-16

- R2-2003139	Draft LS to RAN4 on ANR Measurements	Ericsson [To be RAN2]	LS out	Rel-16	NB_IOTenh3-Core	To:RAN4

- R2-2003247	SON remaining issues	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core

- R2-2003291	Remaining FFSs for SON in NB-IoT	ZTE Corporation, Sanechips	discussion	Rel-16	NB_IOTenh3-Core