3GPP TSG-RAN WG2 Meeting #110-e	R2-2005731

Electronic, 1 – 12 June 2020

Agenda Item:	8.1

Source: 	Vice Chairman (Nokia)

Title:	Report on LTE legacy, LTE TEI16 and NR/LTE Rel-16 Mobility topics

General

RAN2 110 electronic has full decision power, i.e. full decision power to make agreements and approvals according to RAN WG2 terms of reference, without any need to ratify decisions at a later RAN2 or other meeting.

Scope

R17 will not be handled. R16 and earlier will be handled, all tdoc types, see also instructions for each agenda item.

The specific objectives of this meeting includes to finish all open Rel-16 Work Items, to finish the Rel-16 ASN.1 review, and conclude the Rel-16 UE capabilities work.

Specific methodology

R2 110e is expected to be conducted by email and by web conferences by GoToWebinar, in three parallel sessions. To facilitate easy treatment, some AIs may be summarized in summary tdoc.

Tdoc Limitation for some R16 items

Tdoc Limitation applies as indicated for an Agenda Item for all types of documents. As usual Rapporteur input (email discussion, WI rapporteur, TS rapporteur, assigned CR editor, assigned summary rapporteur etc) do not count. Corrections acknowledged but not addressed/resolved in email discussion, or acknowledged by TS rapporteur also do not count. For RRC, for accepted RIL issues, the proponent company may provide a discussion doc with annex TP (if needed) that do not count towards the tdoc limitation. Note that Contributions should be reserved for more complicated issued and minor issues are expected to be resolved in RRC email discussions or by CR rapporteur without any tdoc.

Endorsed or in-principle agreed CRs

CRs that were endorsed or in-principle agreed at previous bis-meeting, need to be provided for final agreement at this meeting

Rel-16 CRs

CRs for ongoing Rel-16 WIs, that were started last meeting, possibly endorsed, are expected to be updated to include agreements from R2-110-e, before final approval.

Note: Time Budget Comments remain in this document only for reference. They are not applicable for R2 110e.

List of offline email discussions:

NOTE: the email discussion deadlines are meant to allow at least all regions to have one day to comment (other than weekend) and also give rapporteurs time to update their proposals before the meeting)

Organizational

[AT110-e][200] Organizational Tero – LTE legacy, LTE Rel-16 and LTE/NR mobility

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs for presentation

Intended outcome (for LS discussion):

    - General information sharing about the sessions

Deadline for providing comments to LSs:

    - Deadline: Friday 2020-06-12 1000 UTC

LTE Legacy

[AT110-e#201][LTE] LTE Rel-15 TDD/FDD capability differentiation (Huawei)

Scope:

    - Discuss the matter of Rel-15 TDD/FDD capability differentiation as per CRs in R2-2005083 and R2-2005084 (late Tdoc R2-2005743 also submitted to further explain the details)

    - Determine what needs to be done and whether there are also earlier release capabilities for which differentiation is not clear.

    - Inform RAN1/4/P (exact groups TBD during discussion) about conclusions made on these.

Intended outcome:

    - Discussion summary in R2-2005741 (by email rapporteur)

    - If agreeable, LS to RANx (exact groups TBD) informing on the outcome of RAN2 in R2-2005742

    - Revised CRs (if agreeable, exact contents and release TBD during discussion)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005741):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

[AT110-e][202][LTE15] LTE non-contiguous CA capabilities (Nokia)

Scope:

    - Determine what can be agreed based on the Nokia CRs in R2-2005186, R2-2005187, R2-2005188, R2-2005189 and R2-2005190 and Huawei CRs in R2-2005481, R2-2005482, R2-2005483, R2-2005484, R2-2005485, R2-2005486 and R2-2005487

    - Determine from which release onwards a correction should be provided

Intended outcome:

    - Discussion summary in R2-2005744 (by email rapporteur)

    - Agreeable CRs

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005744):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

[AT110-e][203][LTE15] LTE legacy CRs (RAN2 VC)

Scope:

    - Discuss which CRs under 4.5.1 (that are not handled in [201] or [202]) are agreeable and whether modifications are needed..

Intended outcome:

    - Discussion summary in R2-2005747 (by email rapporteur).

    - Agreeable CRs (by each CR proponent)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005747):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

LTE Rel-16

[AT110-e#204][LTE] Handling of SA5 LS replies on QoE Measurement Collection (Ericsson)

Scope:

    - Discuss the LS replies received from SA5 in R2-2004381 and R2-2004382

    - Discuss the input documents in R2-2004623 and R2-2005385 to determine what RAN2 needs to do

    - Discuss whether to send reply LS to SA5 (CC: TBD) and, if agreeable, provide updated LS according to discussion in

Intended outcome:

    - Discussion summary in R2-2005748 (by email rapporteur)

    - If agreeable, LS to RANx (exact groups TBD) informing on the outcome of RAN2 in R2-2005749

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005741):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

[AT110-e#205][LTE] LTE contributions in AIs 7.6, 7.8 and 7.9 (RAN2 VC)

Scope:

    - Handle the contributions in AIs 7.6.0, 7.8 and 7.9

Intended outcome:

    - Discussion summary in R2-2005750 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline for companies' feedback:  Thursday 2020-06-04 10:00 UTC

    - Initial deadline for rapporteur's summary in R2-2005750:  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

LTE/NR ASN.1 review

[AT110-e][206][LTE ASN1] LTE general ASN.1 discussion (Samsung)

Scope:

    - Flag issues to be discussed online (including specifics of each issue)

Intended outcome:

    - Discussion summary (including list of flagged topics and proposed resolutions) in R2-2005752 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Wednesday 2020-06-03 11:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005752):  Thursday 2020-06-04 11:00 UTC

    - Whether to continue the discussion after this TBD during Thursday 2020-06-04 online session

NR Mobility

[AT110-e][207][NR MOB] ASN.1 review for NR mobility (Intel)

Scope:

    - Flag issues with proposed resolution to ASN.1 review issues as per R2-2004661 in and R2-2004672 for online discussion.

Intended outcome:

    - Discussion summary in R2-2005751 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - Deadline for flagging:  Tuesday 2020-06-02 08:00 UTC

    - Deadline for rapporteur's summary of flagging (in R2-2005751):  Tuesday 2020-06-02 13:00 UTC

LTE/NR Mobility

[AT110-e][208][LTE/NR MOB] User plane issues for DAPS (NN)

Scope:

    - Discuss issues remaining after DAPS UP session (TBD if needed)

Intended outcome:

    - Discussion summary in R2-2005753 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - TBD

[AT110-e][209][MOB] CHO and CPC issues (NN)

Scope:

    - Discuss the contributions R2-2005344, R2-2005682, R2-2005681, R2-2005380, R2-2005456 in AI 6.9.2 and the contributions R2-2005345, R2-2005381, R2-2005279 in AI 6.9.3

    - Determine what (if anything) can be agreed based on the handled contributions

Intended outcome:

    - Discussion summary in R2-2005754 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Friday 2020-06-05 10:00 UTC

    - Deadline for rapporteur's summary (in R2-2005754):  Monday 2020-06-08 16:00 UTC

CR finalization

[AT110-e][210][NR MOB] NR RRC CR (Intel)

Scope:

    - NR RRC CR capturing NR DAPS, NR CHO and CPC changes agreed in this meeting

Intended outcome:

    - Agreed 38.331 CR in R2-2005755

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

[AT110-e][211][LTE MOB] RRC CR (Ericsson)

Scope:

    - LTE RRC CR capturing LTE DAPS, LTE CHO and NR CPC changes agreed in this meeting

Intended outcome:

    - Agreed 36.331 CR for LTE and NR mobility in R2-2005757

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

[AT110-e][212][MOB] PDCP CRs for LTE and NR (Huawei)

Scope:

    - PDCP CRs for LTE and NR DAPS corrections agreed in this meeting

Intended outcome:

    - Agreed CR to 38.323 CR in R2-2005758 for NR PDCP changes agreed in this meeting

    - Agreed CR to 36.323 in R2-2005759 for LTE PDCP changes agreed in this meeting

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

[AT110-e][213][MOB] MAC CRs for LTE and NR (vivo)

Scope:

    - MAC CRs for LTE and NR DAPS corrections agreed in this meeting

Intended outcome:

    - Agreed CR to 38.321 CR in R2-2005760 for NR MAC changes agreed in this meeting

    - Agreed CR to 36.321 in R2-2005761 for LTE MAC changes agreed in this meeting

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

[AT110-e][214][MOB] UE capability CRs for NR mobility (Intel)

Scope:

    - 38.306 and 38.331 CRs for LTE capabilities based on agreements in this meeting

Intended outcome:

    - Agreed CR to 38.331 CR in R2-2005762 for NR UE capability signalling

    - Agreed CR to 38.306 in R2-2005763 for NR capability descriptions

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

[AT110-e][215][MOB] UE capability CRs for LTE mobility (China Telecom)

Scope:

    - 36.306 and 36.331 CRs for LTE capabilities based on agreements in this meeting

Intended outcome:

    - Agreed CR to 38.331 CR in R2-2005764 for LTE UE capability signalling

    - Agreed CR to 36.306 in R2-2005765 for LTE capability descriptions

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

[AT110-e][216][LTE]  LTE Stage-2 updates (China Telecom)

Scope:

    - Correct Stage-2 text to be according to agreements. Improvements over parts discussed online should also be considered.

Intended outcome:

    - Agreed CR to 36.300 CR in R2-2005756 for LTE UE capability signalling

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

Post-meeting email discussions

[Post110-e][LTE/NR] XXX CR (Company)

Intended outcome: Outcome

Deadline: 1-week

[Post110-e][LTE/NR] Title (Company)

Intended outcome: Outcome

Deadline: 1-week/Short/Long

Web conference schedule

Sessions handled by this document highlighted

| Time Zone UTC | Web Conference R2 NR RRC | Web Conference R2 NR Other | Web Conference R2 BO2 |
| --- | --- | --- | --- |
| Tuesday 2 |  |  |  |
| 13:30 – 15:00 | [6.9.5] NR & LTE mobility enhancements NR RRC (Tero) - LTE/NR mobility organizational [6.9.1, 7.3.1] - LTE/NR UE capability input from RAN1/4 [6.9.4, 7.3.3] - LTE/NR mobility ASN.1 review [ 7.3.4, 6.9.5] | Power saving [6.11.1] General, [6.11.2] UP and [6.11.4] RRM (Diana) | [6.4] NR V2X (Kyeongin) (can treat RRC as well) |
| Wednesd 3 |  |  |  |
| 13:30 – 15:00 | [6.2.3] NR-U CP RRC aspects (Diana) [6.11.3] PowSav CP RRC aspects (Diana) [6.13.3] 2-step CP RRC aspects (Diana) | TBD: [5] NR corrections (Johan) or [6.0.2] NR UE capabilities (Johan) | [6.9][7.3] NR & LTE mobility enhancements non-RRC (Tero) - DAPS UP [6.9.6, 7.3.2] - LTE mobility other [7.3.5] |
| Thursday 4 |  |  |  |
| 13:30 – 15:00 | [6.8.2.2] NR Pos RRC corrections, [6.21] On demand SI in connected (Nathan) | TBD: [6.1] IAB non-RRC (Johan) or [6.0.2] NR UE capabilities | [7.0.1] LTE ASN.1 review (Tero) [7.0.2] LTE UE features (if needed) |
| Friday 5 |  |  |  |
| 03:30-05:00 | [6.7.2.2] IIOT RRC [6.22.2] URLLC RRC (Johan) | [4.4][5.4][6.8][7.7][6.20] Positioning (Nathan) | [4.5][7] EUTRA misc (Tero) - LTE legacy and Rel-16 [4.5.1, 7.5.1, 7.6.1, 7.8, 7.9] |
| Tuesday 9 |  |  |  |
| 13:00 – 14:30 | [6.15.2][6.16.2][6.18.2] RRC aspects (Sergio) | [6.20] TEI16 (Johan), [6.19] other (Johan) if needed | [7.0.1] LTE ASN.1 review (Tero) |
| 14:30 – 16:00 | [6.9.5] NR & LTE mobility enhancements NR RRC (Tero) - UE capabilities for LTE/NR mobility [6.9.4, 7.3.3] - Any remaining ASN.1 review topics [7.3.4, 6.9.5] | TBD (Johan) | TBD [6.4] NR V2X |

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.5	Other LTE corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session.

### 4.5.0	In-principle agreed CRs

### 4.5.1	Other

By Email

Rel-15: TDD/FDD capabilty differentiation:

R2-2005743	[AT110-e#201][LTE] LTE Rel-15 TDD/FDD capability differentiation [Pre-meeting]	Huawei, HiSilicon	discussion	Rel-15	TEI15	Late

R2-2005083	Correction to the LTE Rel-15 TDD/FDD capability differentiation	Huawei, HiSilicon	CR	Rel-15	36.331	15.9.0	4304	-	F	TEI15

R2-2005084	Correction to the LTE Rel-15 TDD/FDD capability differentiation	Huawei, HiSilicon	CR	Rel-16	36.331	16.0.0	4305	-	A	TEI15

All of above handled in offline email discussion [202]

Offline email discussion [201] scope:

[AT110-e#201][LTE] LTE Rel-15 TDD/FDD capability differentiation (Huawei)

Scope:

    - Discuss the matter of Rel-15 TDD/FDD capability differentiation as per CRs in R2-2005083 and R2-2005084 (late Tdoc R2-2005743 also submitted to further explain the details)

    - Determine what needs to be done and whether there are also earlier release capabilities for which differentiation is not clear.

    - Inform RAN1/4/P (exact groups TBD during discussion) about conclusions made on these.

Intended outcome:

    - Discussion summary in R2-2005741 (by email rapporteur)

    - If agreeable, LS to RANx (exact groups TBD) informing on the outcome of RAN2 in R2-2005742

    - Revised CRs (if agreeable, exact contents and release TBD during discussion)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005741):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

By Web Conf (Friday June 5th)

R2-2005741	Summary of discussion [201] on missing TDD/FDD differentiation in LTE (Huawei)	Huawei	discussion	TEI15	Late

R2-2005742	Draft LS on missing TDD/FDD differentiation in LTE	Huawei	LS out	Rel-15	TEI15	To:RAN1, RAN4, RAN	Late

By Email

Rel-10/12: Non-contiguous Intra-band CA capabilities:

R2-2005186	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-12	36.331	12.18.0	4247	1	F	LTE_CA-Core, TEI12	R2-2003147

R2-2005187	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-13	36.331	13.15.0	4248	1	A	LTE_CA-Core, TEI12	R2-2003148

R2-2005188	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-14	36.331	14.14.0	4249	1	A	LTE_CA-Core, TEI12	R2-2003149

R2-2005189	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-15	36.331	15.9.0	4250	1	A	LTE_CA-Core, TEI12	R2-2003150

R2-2005190	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-16	36.331	16.0.0	4251	1	A	LTE_CA-Core, TEI12	R2-2003151

R2-2005481	Clarification on UE capability for intra-band non-continuous CA	Huawei, Hisilicon	CR	Rel-10	36.331	10.22.0	4327	-	F	LTE_CA-Core

R2-2005482	Clarification on UE capability for intra-band non-continuous CA	Huawei, Hisilicon	CR	Rel-11	36.331	11.19.0	4328	-	A	LTE_CA-Core

R2-2005483	Clarification on UE capability for intra-band non-continuous CA	Huawei, Hisilicon	CR	Rel-12	36.331	12.18.0	4329	-	F	LTE_CA-Core

R2-2005484	Clarification on UE capability for intra-band non-continuous CA	Huawei, Hisilicon	CR	Rel-13	36.331	13.15.0	4330	-	F	LTE_CA-Core

R2-2005485	Clarification on UE capability for intra-band non-continuous CA	Huawei, Hisilicon	CR	Rel-14	36.331	14.14.0	4331	-	A	LTE_CA-Core

R2-2005486	Clarification on UE capability for intra-band non-continuous CA	Huawei, Hisilicon	CR	Rel-15	36.331	15.9.0	4332	-	A	LTE_CA-Core

R2-2005487	Clarification on UE capability for intra-band non-continuous CA	Huawei, Hisilicon	CR	Rel-16	36.331	16.0.0	4333	-	A	LTE_CA-Core

All of above handled in offline email discussion [202]

Offline email discussion [202] scope:

[AT110-e][202][LTE15] LTE non-contiguous CA capabilities (Nokia)

Scope:

    - Determine what can be agreed based on the Nokia CRs in R2-2005186, R2-2005187, R2-2005188, R2-2005189 and R2-2005190 and Huawei CRs in R2-2005481, R2-2005482, R2-2005483, R2-2005484, R2-2005485, R2-2005486 and R2-2005487

    - Determine from which release onwards a correction should be provided

Intended outcome:

    - Discussion summary in R2-2005744 (by email rapporteur)

    - Agreeable CRs

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005744):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

By Web Conf (Friday June 5th)

R2-2005744	Summary of discussion [202] on non-contigous CA capabilities (Nokia)	Nokia	discussion	LTE_CA-Core	Late

By Email

Rel-12: Correction to T312:

R2-2005351	Correction on t312 timer information	ZTE Corporation, Sanechips	CR	Rel-12	36.331	12.18.0	4316	-	F	HetNet_eMOB_LTE-Core

R2-2005352	Correction on t312 timer information	ZTE Corporation, Sanechips	CR	Rel-13	36.331	13.15.0	4317	-	A	HetNet_eMOB_LTE-Core

R2-2005353	Correction on t312 timer information	ZTE Corporation, Sanechips	CR	Rel-14	36.331	14.14.0	4318	-	A	HetNet_eMOB_LTE-Core

R2-2005354	Correction on t312 timer information	ZTE Corporation, Sanechips	CR	Rel-15	36.331	15.9.0	4319	-	A	HetNet_eMOB_LTE-Core

R2-2005355	Correction on t312 timer information	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.0.0	4320	-	A	HetNet_eMOB_LTE-Core

Handled in offline email discussion [203]

Rel-13: HARQ-ACK codebook capabilities for more than 5CCs:

R2-2005191	Clarification on codebook-HARQ-ACK-r13 capability for CA with more than 5CCs	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-13	36.306	13.12.0	1747	1	F	LTE_CA_enh_b5C-Core	R2-2003152

R2-2005192	Clarification on codebook-HARQ-ACK-r13 capability for CA with more than 5CCs	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-14	36.306	14.11.0	1748	1	A	LTE_CA_enh_b5C-Core	R2-2003153

R2-2005193	Clarification on codebook-HARQ-ACK-r13 capability for CA with more than 5CCs	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-15	36.306	15.8.0	1749	1	A	LTE_CA_enh_b5C-Core	R2-2003154

R2-2005194	Clarification on codebook-HARQ-ACK-r13 capability for CA with more than 5CCs	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-16	36.306	16.0.0	1750	2	A	LTE_CA_enh_b5C-Core	R2-2003859

Handled in offline email discussion [203]

Rel-14: PDU generation for latency reduction:

R2-2005551	PDU generation for UL spatial multiplexing	ASUSTeK	discussion	Rel-15	LTE_LATRED_L2-Core, TEI14

R2-2005552	Correction on PDU generation for UL spatial multiplexing	ASUSTeK	CR	Rel-14	36.321	14.12.0	1480	-	F	LTE_LATRED_L2-Core, TEI14

R2-2005553	Correction on PDU generation for UL spatial multiplexing	ASUSTeK	CR	Rel-15	36.321	15.8.0	1481	-	A	LTE_LATRED_L2-Core, TEI14

R2-2005554	Correction on PDU generation for UL spatial multiplexing	ASUSTeK	CR	Rel-16	36.321	16.0.0	1482	-	A	LTE_LATRED_L2-Core, TEI14

Handled in offline email discussion [203]

Rel-15: SRB duplication:

R2-2004407	Correction on SRB duplication	OPPO, LG Electronics	CR	Rel-15	36.323	15.5.0	0280	1	F	LTE_HRLLC	R2-2002619

R2-2004408	Correction on SRB duplication	OPPO, LG Electronics	CR	Rel-16	36.323	16.0.0	0281	1	A	LTE_HRLLC	R2-2002620

Handled in offline email discussion [203]

Rel-15: Correction to AUL HARQ processes:

R2-2005678	Correction of AUL HARQ process	ASUSTeK	CR	Rel-15	36.331	15.9.0	4340	-	F	LTE_unlic-Core

Handled in offline email discussion [203]

Rel-16 shadow of above (if needed, according to conclusion of [203])

R2-2005745	Correction of AUL HARQ process	ASUSTeK	CR	Rel-16	36.331	16.0.0	XXXX	-	F	LTE_unlic-Core

Rel-15: Minor RRC corrections:

R2-2005283	Minor changes collected by Rapporteur	Samsung Telecommunications	CR	Rel-15	36.331	15.9.0	4314	-	F	MBMS_LTE_enh2-Core, TEI15	R2-2003233	Late

(moved from 4.5)

=> Revised in R2-2005995

R2-2005995	Minor changes collected by Rapporteur	Samsung Telecommunications	CR	Rel-15	36.331	15.9.0	4314	1	F	MBMS_LTE_enh2-Core, TEI15	Late

(moved from 4.5)

Handled in offline email discussion [203]

Rel-16 shadow of above (if needed, according to conclusion of [203])

R2-2005746	Minor changes collected by Rapporteur	Samsung Telecommunications	CR	Rel-15	36.331	16.0.0	XXXX	-	A	MBMS_LTE_enh2-Core, TEI15	R2-2003233	Late

Offline email discussion [203] scope:

[AT110-e][203][LTE15] LTE legacy CRs (RAN2 VC)

Scope:

    - Discuss which CRs under 4.5.1 (that are not handled in [201] or [202]) are agreeable and whether modifications are needed..

Intended outcome:

    - Discussion summary in R2-2005747 (by email rapporteur).

    - Agreeable CRs (by each CR proponent)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005747):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

By Web Conf (Friday June 5th)

R2-2005747	Summary of discussion [203] on LTE contributions in AI 4.5	Nokia (RAN2 VC)	discussion	HetNet_eMOB_LTE-Core, LTE_CA_enh_b5C-Core, LTE_LATRED_L2-Core, TEI14, LTE_HRLLC, LTE_unlic-Core, MBMS_LTE_enh2-Core, TEI15	Late

# 6	Rel-16 NR Work Items

## 6.9	NR mobility enhancements

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; target; Mar 20; WID: RP-192277). Documents in this agenda item will be handled in a break out session

No documents should be submitted to 6.9. Documents under 6.9 will be treated together with documents in 7.3.

A web conference may be used for handling some of the discussions in this WI, and summary document may be provided for some agenda items under 6.9.

### 6.9.1	Organisational

Including incoming LSs, running CRs, rapporteur inputs, etc.

Including outcome of [Post109bis-e][927][NR MOB] Stage-2 CR (Intel).

By Web Conf (Tuesday June 2nd)

R2-2004355	LS on Simultaneous reception of DL signals in intra-frequency DAPS HO (R1-2003058; contact: Intel)	RAN1	LS in	Rel-16	NR_Mob_enh-Core	To:RAN4	Cc:RAN2

=> Noted

R2-2004662	Corrections on NR mobility enhancements (109b-927)	Intel Corporation	CR	Rel-16	38.300	16.1.0	0230	-	F	NR_Mob_enh-Core

Email discussion [927] outcome

Agreed

Additional Stage-2 corrections:

R2-2004518	Corrections to Mobility Enhancements	Nokia, Intel Corporation (Rapporteurs)	CR	Rel-16	38.300	16.1.0	0211	2	F	NR_Mob_enh-Core	R2-2003857

- Nokia clarifies this is the IPA CR but with some additional clean-up – no functional changes.

Agreed

Updated version of IPA RRC from RAN2#109bis-e:

R2-2004670	Corrections on NR mobility enhancements	Intel Corporation	CR	Rel-16	38.331	16.0.0	1591	1	F	NR_Mob_enh-Core	R2-2003850

- Intel explains there are only some editorials compared to previous version.

Not treated

Text enhancements:

R2-2004914	Correction on CHO failure handling	OPPO	CR	Rel-16	38.300	16.1.0	0234	-	F	NR_Mob_enh-Core

(moved from 6.9.2)

By Email

Offline email discussion [210] scope:

[AT110-e][210][NR MOB] NR RRC CR (Intel)

Scope:

    - NR RRC CR capturing NR DAPS, NR CHO and CPC changes agreed in this meeting

Intended outcome:

    - Agreed 38.331 CR in R2-2005755

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

R2-2005755	Corrections to Rel-16 NR mobility enhancement	Intel Corporation	CR	Rel-16	38.331	16.0.0	XXXX	F	NR_Mob_enh-Core

### 6.9.2	Conditional handover

This AI jointly addresses corrections to NR and LTE CHO.

All RRC-related corrections to CHO should be submitted to ASN.1 review agenda items in 6.9.5 (NR RRC) and 7.3.4 (LTE RRC).

Tdoc Limitation per company: 1 tdoc.

By Email

Stage-2 corrections, including CHO evaluation condition stopping during legacy HO:

R2-2005344	On stopping evaluating execution condition once triggering the legacy HO	ZTE Corporation, Sanechips	discussion	Rel-16	NR_Mob_enh-Core

R2-2005682	CHO Evaluating Handling during Legacy HO	LG Electronics Inc.	discussion	Rel-16	NR_Mob_enh-Core

(moved from 6.9.6)

R2-2005681	Stage 2 CR for CHO Evaluating Handling during Legacy HO	LG Electronics Inc.	CR	Rel-16	38.300	16.1.0	0242	-	F	NR_Mob_enh-Core

Handled in email discussion [209]

Other topics:

R2-2005380	Discussion on leftovers for CHO	Huawei, HiSilicon	discussion	Rel-16	LTE_feMob-Core, NR_Mob_enh-Core	R2-2003577

R2-2005456	Further consideration on CHO in MR-DC operation	CMCC	discussion	Rel-16	NR_Mob_enh-Core

Handled in email discussion [209]

Offline email discussion [209] scope:

[AT110-e][209][LTE/NR MOB] CHO and CPC issues (Nokia)

Scope:

    - Discuss the contributions R2-2005344, R2-2005682, R2-2005681, R2-2005380, R2-2005456 in AI 6.9.2 and the contributions R2-2005345, R2-2005381, R2-2005279 in AI 6.9.3

    - Determine what (if anything) can be agreed based on the handled contributions

Intended outcome:

    - Discussion summary in R2-2005754 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Friday 2020-06-05 10:00 UTC

    - Deadline for rapporteur's summary (in R2-2005754):  Monday 2020-06-08 16:00 UTC

By Web Conf (Tuesday June 9th)

R2-2005754	Summary of discussion [209] on CHO/CPC	Nokia	discussion	Late

### 6.9.3	Conditional PSCell change for intra-SN

Including corrections for CPC.

Including outcome of [Post109bis-e][929][NR MOB] Stage-2 CR for CPC (CATT)

Tdoc Limitation per company: 1 tdoc

By Web Conf (Tuesday June 9th)

Outcome of [Post109bis-e][929][NR MOB] Stage-2 CR for CPC (CATT)

R2-2005071	Introduction of Conditional PSCell Change for intra-SN without MN involvement	CATT	draftCR	Rel-16	37.340	16.1.0	F	NR_Mob_enh-Core	Late

Email discussion [929] outcome

By Email

Miscellaneous issues for CPC:

R2-2005345	Remaining issues for CPC	ZTE Corporation, Sanechips	discussion	Rel-16	NR_Mob_enh-Core

Handled in email discussion [209]

Optimizations requiring reversal or change in previous agreements:

R2-2005381	Discussion on leftovers for CPC	Huawei, HiSilicon	discussion	Rel-16	LTE_feMob-Core, NR_Mob_enh-Core

R2-2005279	Corrections on procedure for CPC complete	Futurewei	discussion	Rel-16	NR_Mob_enh-Core

Handled in email discussion [209]

### 6.9.4	UE capabilities for conditional handover, fast handover failure recovery and conditional PSCell change

This AI jointly addresses UE capabilities for features in the NR mobility WI (i.e. DAPS, CHO, CPC, T312). Any input on UE capabilities from RAN1/4 will be handled in this agenda item.

Including outcome of [Post109bis-e][930][NR MOB] UE capabilities for NR mobility (Intel).

Tdoc Limitation per company: 1 tdoc

By Email

Offline email discussion [214] scope:

[AT110-e][214][MOB] UE capability CRs for NR mobility (Intel)

Scope:

    - 38.306 and 38.331 CRs for LTE capabilities based on agreements in this meeting

Intended outcome:

    - Agreed CR to 38.331 CR in R2-2005762 for NR UE capability signalling

    - Agreed CR to 38.306 in R2-2005763 for NR capability descriptions

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

R2-2005762	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation	CR	Rel-16	38.331	16.0.0	XXXX	B	NR_Mob_enh-Core

R2-2005763	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation	CR	Rel-16	38.306	16.0.0	XXXX	B	NR_Mob_enh-Core

By Web Conf (Wednesday June 3rd or Tuesday June 9th)

Outcome of [Post109bis-e][930][NR MOB] UE capabilities for NR mobility (Intel):

R2-2004663	[109b#930] UE capabilities for NR mobility	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core

DAPS-related proposals in the general UE capability discussion [963]:

R2-2005311	Report of email discussion [Post109bis-e][963][NR16] UE capabilities	Intel Corporation, NTT DoCoMo	discussion	Rel-16	NR_UE_pow_sav, NR_IAB-Core, NR_eMIMO-Core, NR_IIOT-Core, NR_2step_RACH-Core, 5G_V2X_NRSL-Core, NR_Mob_enh-Core, NR_pos-Core, NR_unlic-Core, LTE_NR_DC_CA_enh-Core, NR_SON_MDT-Core, NR_CLI_RIM, NG_RAN_PRN-Core, TEI16, NR_L1enh_URLLC-Core	Late

- Intel clarifies these are mainly due to RAN1/4 decisions.

- Intel also now thinks P12 is not correct for number of TAGs – since we only have source/targe PCell, no need for additional TAGs. UE needs to support source and target TAGs (=2 TAGs) but not more. If UE supports only one TAG, it only supports collocated case.

- Qualcomm thinks the TAG has to be mandatory for all UEs supporting DAPS. This was not clear in LTE CA, which caused problems.

- MediaTek thinks we don’t need single- and multi-UL capabilities. Intel clarifies RAN2 allowed single UL earlier, this intends to allow multiple UL in addition. LGE wonders if the same applies for sync and async DAPS – is sync the baseline and async optional?

- Huawei agrees single UL and sync can be the default UE capabilities.

- Ericsson thinks we could just reuse legacy TAG capability.

- Intel clarifies we agreed TAG support is mandatory, but RAN4 wants different capabilities for intra- and inter-frequency DAPS.

- OPPO wonders if we ever discussed collocated case for intra-frequency DAPS? Thinks UE would always support two TAGs for intra-frequency case. Intel clarifies TAG=1 means TA value is the same for source and target. This comes from RAN4. OPPO thinks typical mobility scenario is between two nodes. Intel thinks 2 TAGs is mandatory with IOT bit.

- vivo wonders why the intra-frequency is per band, per BC, could be per band. Intel clarifies per BC is needed since different combinations use different BW classes. It’s also more future-proof.

Agreements (NR)

12a introduce separate capabilities for intraFreq and interFreq as below:

Per Band/per BC (for intraFreq capabilities), I.e. put under BandParameters-v16xy:

intraFreqDiffSCS-DAPS-r16;

intraFreqAsyncDAPS-r16

intraFreqMultiUL-TransmissionDAPS-r16

Per BC (for interFreq capabilities), i.e. put under CA-ParametersNR-v16xy:

interFreqDiffSCS-DAPS-r16

interFreqAsyncDAPS-r16

interFreqMultiUL-TransmissionDAPS-r16.

12b	All UEs supporting DAPS support these capabilities (can discuss signalling details and naming):

SyncDAPS-r16

SingleUL-TransmissionDAPS-r16

intraFreqTwoTAGs-DAPS-r16  (with 2 TAGs)

(for interFreq since RAN2 agreed to “Reuse CA capability “supportedNumberTAG” for DAPS handover.)

8a	Remove UplinkPowerSharingDAPS-HO

8b	Add separate capabilities for 21-2, 21-2a, 21-2b as semiStaticPowerSharingDAPS-Mode1, semiStaticPowerSharingDAPS-Mode2 and dynamicPowersharingDAPS.

8c	RAN2 thinks that these apply only for multiple UL supporting UEs,

10	Remove pdcch-BlindDetectionSource and pdcch-BlindDetectionTarget from RAN2 agreed capabilities.

11	Add syncDAPS and simultaneous UL transmission based on RAN4 latest capability table.

13	Introduce separate capabilities for intraFreq and interFreq for power sharing capabilities.

- Wait for RAN1 conclusion on ul-TransCancellationDAPS.

R2-2004664	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation	draftCR	Rel-16	38.331	16.0.0	F	NR_Mob_enh-Core

R2-2004665	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation	draftCR	Rel-16	38.306	16.0.0	F	NR_Mob_enh-Core

NR DAPS capabilities:

R2-2005061	Discussion on UE capabilities for NR DAPS	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

Proposal 1: remove singleUL-TransmissionDAPS-r16 in baseline TP.

Proposal 2: specify separate UE capabilities for all three power sharing modes.

Proposal 3: remove pdcch-BlindDetectionDAPS-r16 in baseline TP.

Proposal 4: Add separate supportedNumberTAG UE capability for intra-frequency DAPS handover.

Additional input on CHO and CPC capabilities:

R2-2005160	UE capabilities for Mobility Enhancements WI	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_Mob_enh-Core

R2-2005457	Discussion on the maxinum CPC candidates	CMCC	discussion	Rel-16	NR_Mob_enh-Core

(moved from 6.9.3)

R2-2004917	Discussion on UE capability for CHO and CPC	OPPO	discussion	Rel-16	NR_Mob_enh-Core

R2-2005684	Consideration on Conditional mobility capability	LG Electronics Inc.	discussion	Rel-16	NR_Mob_enh-Core	R2-2002902

### 6.9.5	ASN.1 review of mobility WIs for NR RRC

This agenda item focuses on NR RRC aspects of NR mobility W – LTE RRC aspects of both LTE and NR mobility WIs should be submitted to 7.3.4. Do not submit contributions on WI-specific open issues that are not captured in the current NR RRC to this agenda item.

All ASN.1 issues should be raised in RILs first – contributions where no RIL issue exists may not be treated.

Including contributions/TPs on RRC corrections based on review issues. For these, no individual company CRs should be submitted: please consult with the rapporteur of NR RRC CR first (yi.guo@intel.com).

By Email

Offline email discussion [207] scope:

[AT110-e][207][NR MOB] ASN.1 review for NR mobility (Intel)

Scope:

    - Flag issues with proposed resolution to ASN.1 review issues as per R2-2004661 in and R2-2004672 for online discussion.

Intended outcome:

    - Discussion summary in R2-2005751 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - Deadline for flagging:  Tuesday 2020-06-02 08:00 UTC

    - Deadline for rapporteur's summary of flagging (in R2-2005751):  Tuesday 2020-06-02 13:00 UTC

By Web Conf (Tuesday June 2nd)

R2-2005751	Summary of discussion [207] on NR mobility ASN.1 review	Intel	discussion	Late

3.1	Proposal on Phase 1 class 2 RIL issues:

To approve:

Bulk agreement

I103: Agree I103 to add “2> release source PCell configuration;”in 5.3.5.3.

I107: Agree I107, to combine the conditions “If any DAPS bearer is configured:” and “2>	for each SRB:” together in 5.3.5.6.3:

I109: Agree I109, to remove “the S-KgNB key, the S-KeNB key,” from 5.3.5.8.3:

I111: Agree I111 to add the field description for configRestrictInfoDAPS: “Includes fields for which souce cell explictly indicates the restriction to be observed by target cell during DAPS handover.”

Online discussion:

I101:

- LGE would like to leave the existing text as it is. Intel clarifies that procedural text already captures this. Nokia agrees.Huawei thinks it’s not clear that UE performs CHO failure handling if all entries have been removed.

I105:

- CATT thinks the text clarified that this applies to PCell only, not CPC. Intel thinks network only configures the attemptCondReconfig is CHO is configured, so there’s no ambiguity. LGE agres wuith CATT. OPPO thinks the sentence could be reworded.

Agreements

I101: Agree I101 to remove the note “This step is performed so the UE only performs conditional reconfiguration execution while timer T311 is running once for a given failure detection .” from 5.3.5.3.

I105: Agree I105, to move the NOTE3 together with Note 1, 2  in 5.3.5.5.2:

I100: Change to ConcReject.

Rest are discussed later on

????

Proposal on I104: to discuss whether to clarify same configuration is the configuration from the same DRB in 5.3.5.5.2:

3>	establish an RLC entity or entities for the target, with the same configurations of the same DRB  as for the source;

3>	establish the logical channel for the target PCell, with the same configurations of the same DRB  as for the source;

Proposal on S303: Agree S303:Use Cond PCell for field attemptCondReconfig-r16 with

Alt 1 “The field is optional present, need N, if conditionalReconfiguration is added for CHO. Otherwise the field is not present.” Or

Al2 “The field is optional present in Reconfiguration message embedded in condRRCReconfig that concerns PCell, need N. Otherwise the field is not present.”

3.2	Proposal on Phase 2 class 2/3 RIL issues:

Bulk Agreement

Proposal on Z274: PropAgree2. To capture the changes based on Z274.

Proposal on E231: PropAgree2.

Proposal on M201: follow RRC Rapporteur’s view, i.e. PropAgree2.

Proposal on Z275: PropAgree2. To capture the changes based on Z275.

Proposal on H458: PropReject2.

Proposal on Z276: PropAgree2. But double check the proposed changes.

Proposal on G103: PropReject2.

Proposal on J031: PropReject2.

Proposal on B105: PropReject2.

Proposal on E038: PropReject2.

Proposal on B107: Not related to MOB WI.

Proposal on H455: PropReject2.

Proposal on Z278: CPC cannot be configure in PSCell change command.

Proposal on C210: leave the discussion to SON/MDT WI.

Proposal on S309: PropAgree2.

Proposal on S305: PropReject2.

Proposal on E234: PropAgree2.

Proposal on O201: PropReject2.

Proposal on H462: PropAgree2.

Proposal on X007: PropReject2.

Proposal on S308: PropReject2.

Online discussion:

Proposal on J030: PropReject2. R2-2005430

Proposal on G104: PropReject2. R2-2005529

Proposal on I113: Remove the below EN, and keep current CR as it is.

Editor's note: It is FFS if the whole handling on release of spcellConfig, MCG SCells, etc shall be moved to under 1>	else: in 5.3.7.3, i.e. release when reestablishment is triggered.

Proposal on S304: DiscMeet2. R2-2005668

Proposal on Z277: PropAgree2. R2-2005347

Proposal on I112: When resume SRB upon DAPS HO failure, the RLC entities of RRC bearers are re-established.

Proposal on Z273: partially PropAgree2. Do not introduce the definition of Non-DAPS bearer, and change the definition of DAPS bearer as

“DAPS bearer: a bearer whose radio protocols are located in both the source SpCell gNB and the target SpCell gNB during DAPS handover to use both source SpCell gNB and target SpCell gNB resources” R2-2005997 (LTE CR)?

Proposal on E232: DiscMeet2.

Proposal on J033: DiscMeet2.

Proposal on I114: DiscMeet2.

No document:

Proposal on J032: PropReject2.

Proposal on S307: PropAgree2.

Proposal on X004: PropReject2.

Proposal on X005: PropReject2.

Proposal on H460: PropReject2.

Proposal on S306: PropReject2.

By Web Conf (Tuesday June 2nd)

Phase 1 issue resolution:

R2-2004661	Phase 1 class 2 issues on MOB WI (I101, I103, I104, I105, I107, I109, I100, S303, I111)	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core

Proposal on I101: Agree I101 to remove the note “This step is performed so the UE only performs conditional reconfiguration execution while timer T311 is running once for a given failure detection .” from 5.3.5.3.

Proposal on I103: Agree I103 to add “2> release source PCell configuration;”in 5.3.5.3.

Proposal on I104: to discuss whether to clarify same configuration is the configuration from the same DRB in 5.3.5.5.2:

3>	establish an RLC entity or entities for the target, with the same configurations of the same DRB  as for the source;

3>	establish the logical channel for the target PCell, with the same configurations of the same DRB  as for the source;

Proposal on I105: Agree I105, to move the NOTE3 together with Note 1, 2  in 5.3.5.5.2:

Proposal on I107: Agree I107, to combine the conditions “If any DAPS bearer is configured:” and “2>	for each SRB:” together in 5.3.5.6.3:

Proposal on I109: Agree I109, to remove “the S-KgNB key, the S-KeNB key,” from 5.3.5.8.3:

Proposal on I100: Agree I100, to remove “which the reconfigurationWithSync is included in the masterCellGroup” from 5.3.7.3:

Proposal on S303: Agree S303:Use Cond PCell for field attemptCondReconfig-r16 with “The field is optional present, need N, if conditionalReconfiguration is added for CHO. Otherwise the field is not present.”

Proposal on I111: Agree I111 to add the field description for configRestrictInfoDAPS: “Includes fields for which souce cell explictly indicates the restriction to be observed by target cell during DAPS handover.”

Phase 2 issue resolution

R2-2004672	Phase 2 MOB RIL issues	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core	Late

Proposed for bulk agreement:

Agreements???

Proposal on Z273: partially PropAgree2. Do not introduce the definition of Non-DAPS bearer, and change the definition of DAPS bearer as

“DAPS bearer: a bearer whose radio protocols are located in both the source SpCell gNB and the target SpCell gNB during DAPS handover to use both source SpCell gNB and target SpCell gNB resources”

Proposal on Z274: PropAgree2. To capture the changes based on Z274.

Proposal on S306: PropReject2.

Proposal on J032: PropReject2.

Proposal on E231: PropAgree2.

Proposal on M201: DiscMail2.

Proposal on M201: follow RRC Rapporteur’s view, i.e. PropAgree2.

Proposal on G104: PropReject2.

Proposal on Z275: PropAgree2. To capture the changes based on Z275.

Proposal on I112: When resume SRB upon DAPS HO failure, the RLC entities of RRC bearers are re-established.

Proposal on H458: PropReject2.

Proposal on I113: Remove the below EN, and keep current CR as it is.

Editor's note: It is FFS if the whole handling on release of spcellConfig, MCG SCells, etc shall be moved to under 1>	else: in 5.3.7.3, i.e. release when reestablishment is triggered.

Proposal on Z276: PropAgree2. But double check the proposed changes.

Proposal on G103: PropReject2.

Proposal on J031: PropReject2.

Proposal on B105: PropReject2.

Proposal on X004: PropReject2.

Proposal on J030: PropReject2.

Proposal on E038: PropReject2.

Proposal on X005: PropReject2.

Proposal on B107: Not related to MOB WI.

Proposal on H455: PropReject2.

Proposal on Z277: PropAgree2.

Proposal on Z278: CPC cannot be configure in PSCell change command.

Proposal on C210: leave the discussion to SON/MDT WI.

Proposal on S309: PropAgree2.

Proposal on H460: PropReject2.

Proposal on S307: PropAgree2.

Proposal on S305: PropReject2.

Proposal on E234: PropAgree2.

Proposal on O201: PropReject2.

Proposal on H462: PropAgree2.

Proposal on X007: PropReject2.

Proposal on S308: DiscMail2.

Proposal on E232: DiscMeet2.

Proposal on J033: DiscMeet2.

Proposal on S304: DiscMeet2.

Proposal on I114: DiscMeet2.

[J033] RoHC handling without key changes:

R2-2005512	[J033] RoHC handling with and without key change at the UE	SHARP Corporation	discussion	Rel-16	LTE_feMob-Core	R2-2003665

[H223] TAG configuration:

R2-2004427	Clarification on tag-Config for DAPS (subject to [H223])	Samsung	discussion	Rel-16	NR_Mob_enh-Core

S350: Reconfiguration procedure in DAPS and I112: RLC re-establishment upon fallback:

R2-2004666	Phase 1 open issue on DAPS CP (S350, I112)	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core

R2-2005064	[I112] discussion on RLC re-establishment upon fallback	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2005708	[S350] Discussion on radio bearer handling during DAPS	Samsung Electronics	discussion	NR_Mob_enh-Core

R2-2005062	[S350] Discussion on reconfiguration procedure in DAPS	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

Z255: Handling of stored CPC configuration:

R2-2004668	Phase 1 Open issue on  CPC (Z255)	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core

R2-2005348	[Z255] Further discussion on the handling of stored CPC configuration	ZTE Corporation, Sanechips	discussion	Rel-16	NR_Mob_enh-Core

R2-2004620	Remaining issues for conditional PSCell change	Ericsson	discussion	NR_Mob_enh-Core

(moved from 6.9.3)

I113: Field release during re-establishment procedure:

R2-2004667	Phase 1 open issue on CHO (I113)	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core

R2-2005065	[I113] Discussion on handling CHO candidate cells upon RRC re-establishment	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2004619	Re-establishment initiation and CHO	Ericsson	discussion	NR_Mob_enh-Core

(moved from 6.9.2)

[O201]: Restricting DAPS + CHO and DAPS + CPC:

R2-2004915	[O201] Correction on dapsConfig	OPPO	discussion	Rel-16	NR_Mob_enh-Core

R2-2005349	Clarification on not supporting CHO+DAPS	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_feMob-Core

(moved from 7.3.2)

[E232, Z258] Generic RRC text enhancements for DAPS procedure:

R2-2004693	[E232] Source and target entities at DAPS HO	Ericsson	discussion	Rel-16	NR_Mob_enh-Core

R2-2005997	TP on DAPS terminology related ASN.1 review issues (ao Z258)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	TEI16

[S304]: Identification of cell according to PCI or SSB?

R2-2005668	[S304] Clarification on applicable cell in CHO	Samsung R&D Institute UK	discussion

C003: Aligning T310 and T312 descriptions:

R2-2005382	[C003] T312 discussion	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

[XXXX]: Release of CPC when SCG is released:

R2-2005683	Draft CR for Clarification to release CPC when SCG Release	LG Electronics Inc.	draftCR	Rel-16	38.331	16.0.0	F	NR_Mob_enh-Core

(moved from 6.9.3)

[XXXX] Disabling IioT duplication of >2 legs:

R2-2004649	Disabling multi-leg RB for DAPS	vivo	discussion	Rel-16	LTE_feMob-Core

(moved from 7.3.2)

Not treated (unless flagged)

[J030, J031, G103, G104, B105, H458]: PropReject in R2-2004672:

R2-2005430	[J030, J031] UE DAPS configuration release upon RLF	SHARP	discussion	Rel-16	NR_Mob_enh-Core

R2-2005529	[G104] Clarification on DAPS handover failure while the T310 is running	Google Inc.	discussion	38.331	NR_Mob_enh-Core

R2-2005134	[B105] TP for DAPS handover with fast MCG link recovery	Lenovo, Motorola Mobility	discussion	Rel-16

R2-2005383	[H458] Triggering quantity discussion	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2005511	[G103] Clarification on CHO handling during RRC connection re-establishment procedure	Google Inc.	draftCR	Rel-16	38.331	16.0.0	F	NR_Mob_enh-Core

[Z276, Z277]: PropAgree in R2-2004672:

R2-2005346	[Z276] Discussion on UE configuration release in RRC re-establishment	ZTE Corporation, Sanechips	discussion	Rel-16	NR_Mob_enh-Core

R2-2005347	[Z277] Discussion on stopping conditional reconfiguration evaluation during fast MCG recovery	ZTE Corporation, Sanechips	discussion	Rel-16	NR_Mob_enh-Core

### 6.9.6	Other

Only corrections not fitting other agenda items.

Including DAPS aspects that are NR-specific without equivalent LTE impacts: Do not use this AI for any DAPS topics that can be discussed jointly for LTE and NR - Contributions on DAPS that apply for both LTE and NR are treated jointly in under 7.3.2.

Tdoc Limitation per company: 1 tdoc.

By Web Conf (Wed, June 3rd)

Correction to MAC on random access procedure:

R2-2005612	Draft CR on 38.321 for NR mobility enhancement	LG Electronics	draftCR	Rel-16	38.321	16.0.0	F	NR_Mob_enh-Core

(moved from 7.3.5)

- vivo thinks that it’s already clear the RA is for DAPS HO, so it can only be the target MAC entity so the second change is not needed. First change is also not needed for the same reason. LGE thinks this impacts source MAC entity even when not configured with DAPS HO. vivo thinks source MAC doesn’t know about target MAC – there’s no coordination.

- Intel thinks that LGE’s point is that MAC would have to indicate something to the upper layers regardless of DAPS HO. So the change is needed. CATT agrees.

- vivo thinks it was commentecd that the terminology was considered not clear earlier, but should be consistent.

=> Intent of the first change is correct. Exact text can be discussed.

=> Second change is clarification but its intent is correct.

Agreements

=> Consider whether “target MAC entity” is a good way to indicate and use it consistently.

=> Discuss in the MAC CR email discussion how to capture this.

# 7	Rel-16 LTE Work Items

Documents in these agenda items will be handled in break out sessions

## 7.0	LTE Rel-16 General

### 7.0.1	ASN.1 review

Including documents related to LTE ASN.1 review.

Including outcome of [Post109bis-e][932][LTE/NR/ASN.1]  Resolution to review issues S003, S005, B002, S046 (Samsung/Ericsson)

A web conference may be used for handling some of the discussions in this agenda item.

By Email

Offline email discussion [206] scope:

[AT110-e][206][LTE ASN1] LTE general ASN.1 discussion (Samsung)

Scope:

    - Flag issues to be discussed online (including specifics of each issue)

Intended outcome:

    - Discussion summary (including list of flagged topics and proposed resolutions) in R2-2005752 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Wednesday 2020-06-03 11:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005752):  Thursday 2020-06-04 11:00 UTC

    - This discussion will continue until Tuesday 9th

    - Whether to continue the discussion after this TBD during Thursday 2020-06-04 online session

By Web Conf (Thursday June 4th, Tuesday June 9th)

Flagged issues as per [206]:

R2-2005752	[AT110-e][206][LTE ASN1] R16 LTE RRC coordination (Samsung)	Samsung Telecommunications	discussion	Rel-16	Late

Critical extension of FailureInformation message & associated general principles (S004)

Proposal Conclusion 1:

•	Do not introduce general requirements concerning ASN.1 comprehension for network but decide on a case by case basis

•	Create a regular critical extension of the FailureInformation message i.e. re-use the existing name and ASN.1 section

•	Endorse the related parts of R2-2005282 with the following changes

o	Do not introduce changes to Annex F

o	Add the following note

NOTE:      The UE may apply the FailureInformation-r16 message to report a failure defined in REL-15, but only if it is configured with a feature incorporating a failure that can only be reported by the FailureInformation-r16 message

Agreements

•	Do not introduce general requirements concerning ASN.1 comprehension for network but decide on a case by case basis

•	Create a regular critical extension of the FailureInformation message i.e. re-use the existing name and ASN.1 section

•	Endorse the related parts of R2-2005282 with the following changes

o	Do not introduce changes to Annex F

o	Add the following note

NOTE:      The UE may apply the FailureInformation-r16 message to report a failure defined in REL-15, but only if it is configured with a feature incorporating a failure that can only be reported by the FailureInformation-r16 message

Avoiding critical extension for ULInformationTransfer (S006)

Proposal Conclusion 2:

•	Given the limited input, some discussion seems required to take a final decision. I.e. to chose between:

o	Option A: Change to using a non-critical extension approach, as reflected by the TP in R2-2005282

o	Option B: Continue using the critical extension approach and clarify when UE is allowed to use the R16 message version for signalling legacy fields by adding the following note:

NOTE:      The UE may apply the ULInformationTransfer-r16 message to transfer pre-REL-16 information, but only if it is configured to report F1AP information (i.e. configured with a feature involving transfer of information that can only be carried by the ULInformationTransfer-r16 message)

Discussion

- Ericsson would prefer to stick to existing version. Chair wonders if F1AP and NAS information need to be sent together. Samsung is not sure this can happen.

- Ercisson thinks “pre-Rel-16 information” may not be accurate. Huawei wonders if only IAB-DU and IAB-Donor are involved, so nothing is required from UE.

Agreements

Continue using the critical extension approach and clarify when UE is allowed to use the R16 message version for signalling legacy fields. Can discuss wording of note offline in [206] continuation.

Handling of spares as per R2-2005996

Proposal Conclusion 4:

•	Keep the spares defined for establishmentCause in RRCConnectionRequest-5GC-NB

Discussion

- QC thinks this applies also to non-5GC cases. Lenovo thinks we agreed that we normally avoid spares in UL messages. Need to have specified behaviour for handling received UL spare values. QC clarifies there is specified behaviour for E-UTRAN to not reject the connection due to unknown values for establishmentCause.

- Samsung wonders if we need to mention the UL spare handling in agreement.

Agreements

Keep the spares as defined for establishmentCause in current RRC version where we have defined behaviour for E-UTRAN on handling unknown values.

Encoding of 5G indicator (S191)

Proposal Conclusion 6:

•	Revise the CR to avoid per PLMN information for the case of no sharing or if the same EN-DC bands apply for all PLMNs by adopting 0 as lower bound for the list size

•	Do not adopt the alternative signalling structure as proposed in R2-2005292

Agreements

•	Revise the CR to avoid per PLMN information for the case of no sharing or if the same EN-DC bands apply for all PLMNs by adopting 0 as lower bound for the list size

•	Do not adopt the alternative signalling structure as proposed in R2-2005292

Approach for extension of failure types introduced in R16

Proposal Conclusion 3:

•	As the views have not really converged some further discussion seems required to conclude

•	There seem to be two primary options to chose between:

•	Option 1:

o	Introduce a value other/ unspecified within the legacy field

o	Use spares if defined and undefined code point otherwise

o	Include all new R16 values in an –v16xy extension

o	When signalling the –v16xy extension, the UE will set the legacy field to other/ unspecified

•	Option 2:

o	Do no introduce a value other/ unspecified

o	Use the legacy field to add new R16 values, as long as spares or undefined code points are available

•	The main advantage of option 1 is that it enables networks to configure an R16 features even if MN may not comprehend the extension, as long as it is upgraded to comprehend value other/ unspecified. I.e. it allows some additional network flexibility, but implies that any spares available in the legacy field may not be used

Discussion

- QC thinks option 2 doesn’t solve the problem. Lenovo thinks option 2 is enough as all nodes would support this but is fine with option 1 if network vendors require it. Not sure if all features require the flexibility from network. Samsung thinks the main case is for DC when the flexibility is useful. CATT supports option 1 due to the network flexibility for Rel-16 features. Ericsson also thinks option 1 is better. Samsung is fine with option 1 if necessary but this will require using undefined codepoints for the “other”. Network needs to allow this for any of the features.

- Lenovo wonders if this will also apply for NR. Ericsson thinks this is for both LTE and NR.

- Chair wonders if we will leave spares unused. Ericsson thinks this applies case-by-base – if legacy field is optional, we can still fill in the spare values. QC wonders if this can really apply to NR.

Agreements (for LTE and NR)

1	For extension of failure types (which have mandatory R15 field) introduced in R16:

- Introduce a value other/ unspecified within the legacy field; Use spares if defined and undefined code point otherwise

- Include all new R16 values in an –v16xy extension

- When signalling the –v16xy extension, the UE will set the legacy field to other/ unspecified

Agreements

Merge the CR in R2-2005292 with the changes suggested by Lenovo to the R2-2005768 (done under [206])

Agreements

1	B100: In addition, add the R16 extensions to the paging record (accessType, mt-EDT) by a parallel list (include in ASN1 review CR)

2	Capture the same additional change in NB-IoT CR

=> Inform the decision to eMTC session.

Discussion

- QC indicates this was a late comment in NB-IoT session but is fine with the approach. Should we include this in eMTC or ASN.1 CR? Huawei indicates this can be done also to the NB-IoT CR.

ASN.1 review file, RIL and class0/1 issues:

R2-2005284	ASN.1 Review file (LTE, Word)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	TEI16	R2-2003234	Late

This document is endorsed and the generic ASN.1 impacts according to the issue resolutions will be captured in R2-2005768. Impacts affecting other CRs (e.g. eMTC, NB-IoT) will be captured in the corresponding CRs.

Update newly found issues from other LTE sessions in the file (by each WI RRC CR rapporteur) as part of [206]

Update to capture the progress on the issues so far in this meeting in R2-2005770

R2-2005285	ASN.1 Review RIL (LTE, Excel)	Samsung Telecommunications	report	Rel-16	TEI16	R2-2003827	Late

This document is endorsed and the generic ASN.1 impacts according to the issue resolutions will be captured in R2-2005768. Impacts affecting other CRs (e.g. eMTC, NB-IoT) will be captured in the corresponding CRs.

Update to reflect updated R2-2005770 in R2-2005771

R2-2005286	LTE Rel-16 ASN.1 Review, Class 0 and Class 1 issues	Samsung Telecommunications	report	Rel-16	TEI16	R2-2003235	Late

- Huawei thinks there is an error between use of NB-IoT vs. eMTC in issue 23.

- QC thinks issue 91 (and possibly 92) are not captured and should be.

Correct issue 23 to be captured in NB-IoT CR (CR4287)

Capture issue 91 in the CR

With the above changes, this document is endorsed and the generic ASN.1 impacts according to the issue resolutions will be captured in R2-2005768. Impacts affecting other CRs (e.g. eMTC, NB-IoT) will be captured in the corresponding CRs.

Generic ASN.1 aspects:

R2-2005287	General changes resulting from ASN.1 review for LTE RRC REL-16	Samsung Telecommunications	CR	Rel-16	36.331	16.0.0	4315	-	F	TEI16	Late

- Qualcomm has some comments on this CR on missing changes to changes to wrong places.

Continue discussion under [206]

Revised in R2-2005768

R2-2005768	General changes resulting from ASN.1 review for LTE RRC REL-16	Samsung Telecommunications	CR	Rel-16	36.331	16.0.0	4315	-	F	TEI16	Late

R2-2005292	Adding guidelines for SetupRelease paramterised  type (S008)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	TEI16	Late

R2-2005281	General ASN.1 issues for 36.331 Rel-16 (S004, S006, B102, Q604, B103, X002)	Samsung Telecommunications	discussion	Rel-16	TEI16	R2-2003231	Late

Revised in R2-2005996

R2-2005996	General ASN.1 issues for 36.331 Rel-16 (S004, S006, B102, Q604, B103, X002)	Samsung Telecommunications	discussion	Rel-16	TEI16	Late

Proposal 1	Agree the general principle that, when network supports a critical extension for an UL DCCH message/ IE for one feature, it should also support for this critical extension receipt of legacy values of another feature it supports (i.e. impose additional requirements on network, alike imposed on UE for early implementation)

Proposal 2	Create a regular critical extension of the FailureInformation message i.e. re-use the existing name and ASN.1 section

Proposal 3	Decide which solution to apply for each failure type introduced in R16. I.e. RAN2 is requested to discuss and conclude whether

- If, regardless whether suitalble legacy values exist, it is anyhow fine to use OAM to avoid avoid a legacy node acting as MN receives value an unsupported extension

- If so (i.e. solution 1a/ 3 for all)

  - While available, use an undefined code points for the R16 extensions (solution 1a)

  - Otherwise: use –v16xy and state that network only considers –v16xy i.e. ignores legacy field (solution 3)

- If not (i.e. decide per case):

  - If a suitable legacy value exist for a case: use–v16xy and specify for each case the value to be set in legacy field (solution 1b)

  - Else: solution 1a/ 3 (see previous bullet)

R2-2005282	TP for general ASN.1 issues for 36.331 REL-16 (General ASN.1 issues for 36.331 Rel-16 (S004, S006, B102, Q604, B103, X002)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	TEI16	Late

Revised in R2-2005766

R2-2005766	TP for general ASN.1 issues for 36.331 REL-16 (General ASN.1 issues for 36.331 Rel-16 (S004, S006, B102, Q604, B103, X002)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	TEI16	Late

Outcome of Email discussion [Post109bis-e][932][LTE/NR/ASN.1]  Resolution of review issues S003, S005, B002, S046 (Samsung/Ericsson)):

R2-2005288	Report of [Post109bis-e][932][LTE/NR/ASN.1]  Resolution of review issues S003, S005, B002, S046 (Samsung/Ericsson))	Samsung Telecommunications	report	Rel-16	5G_V2X_NRSL-Core	Late

Proposal 1:	RAN2 is requested to agree the following approach:

  - UL DCCH: one LTE message/ procedure for transfer of NR UL DCCH messages. The procedural handling is completely re-using what is already specified in NR. Statements will be added to indicate that network only includes particular NR SL related info. The same applies for the reverse direction

  - DL DCCH: NR information is added to the concerned LTE procedure (Reconfiguration), by an octet string carrying the particular NR message (Reconfiguration). The procedural handling is completely re-using what is already specified in NR. Statements will be added to indicate that network only includes particular NR SL related info. The same applies for the reverse direction

Proposal 2:	RAN2 is requested to endorse the text proposals as reflected in the following draft CRs:

  - R2-2005178 CR to NR RRC on Correction on crossRAT signalling for NR V2X (Ericsson)

  - R2-2005289 CR to LTE RRC on V2X IRAT signalling (resolution of S003, S005, B002, S046)	(Samsung)

Summary of concerns that were expressed with option 1:

         For UL DCCH information, option 1 seems inconsistent i.e. sometimes using the concerned LTE procedure (CBR measurements) and otherwise using a new procedure (UE Assistance Information, Sidelink UE information)

         Why introduce specific procedures in LTE even if there is no specific procedural handling specified in LTE (UE Assistance Information, Sidelink UE information)

         Is the approach really future proof i.e. does it mean that we will end up introducing a new LTE message/ procedure for any further case we may come across, although all could simply be covered by a single procedure (as shown in option 3)

         For DL DCCH information, why use LTE encoding/ carry specific NR IEs. I.e. it seems much better/ more future proof to carry NR messages. I.e. this avoids problems when small extensions are added in future. I.e. when referring to NR messages such extensions would not require any LTE changes

Summary of concerns concerns were raised regarding option 2/ 3:

         Some companies indicated that use of embedding is strange/ may introduce problems. Why so i.e. we use this approach all from the start of NR e.g. with EN-DC

         Some companies indicated there may be problems related to triggering. Why so i.e. we use this approach all from the start of NR e.g. with transfer of Reconfiguration and Measurements

         Some companies indicate that in implementation it is the LTE RRC module that produces/ receives the NR encoded information. To me this seems merely an implementation matter that we don’t need to worry about. I.e. we merely need to focus on the specification aspects

Discsu

R2-2005289	V2X IRAT signalling (resolution of S003, S005, B002, S046)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	5G_V2X_NRSL-Core	Late

- Huawei wonders what change is to be done to this? Samsung clarifies that one sentence in 5.3.3.2 (“-	receiving NR RRCReconfiguration message that includes an embedded RRCConnectionReconfiguration message; ”) will be removed

Agree to go according to the intent of this CR

To be discussed with V2X chair how/whether to merge this to existing V2X CR (or somewhere else, e.g. ASN.1 CR)

LTE ASN.1 session view is that this could be merged to the V2X LTE RRC CR

Revised in R2-2005767

CB Tuesday

R2-2005767	V2X IRAT signalling (resolution of S003, S005, B002, S046)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	5G_V2X_NRSL-Core	Late

R2-2005178	[Post109bis-e][932][LTE-NR-ASN.1] Correction on crossRAT signalling for NR V2X	Ericsson	CR	Rel-16	38.331	16.0.0	1658	-	F	5G_V2X_NRSL-Core	Late

- Ericsson thinks one RAN1 parameter was erroneously deleted and needs to be added again

Agree to go according to the intent of this CR

To be discussed with V2X chair how/whether to merge this to existing V2X CR (or somewhere else, e.g. ASN.1 CR)

LTE ASN.1 session view is that this could be merged to the V2X NR RRC CR

Revised in R2-2005769

CB Tuesday

R2-2005769	[Post109bis-e][932][LTE-NR-ASN.1] Correction on crossRAT signalling for NR V2X	Ericsson	CR	Rel-16	38.331	16.0.0	1658	1	F	5G_V2X_NRSL-Core	Late

Specific issue resolutions:

R2-2004626	[Q502] [Z302] Merging issues in TS 36.331 subclause 5.3.3.4a	Qualcomm Incorporated	discussion

Agreements

1.	Update status of Z302 to ConcNoAct.

2.	Update status of Q502 to ConcAgree WI-CR.

3.	Adopt the changes shown in section 2.2 to DCCA WI-CR to TS 36.331.

R2-2005290	Encoding of 5G indicator (S191)	Samsung Telecommunications	draftCR	Rel-16	36.331	16.0.0	TEI16

(moved from 7.6.1)

To be discussed together with the other 5G indicator contributions under 6.20.1

LTE ASN.1 specific topics discussed under [206]

### 7.0.2	Features and UE capabilities

Including documents related to LTE UE capabilities based on RAN1/4 input. WI-specific capability contributions should be submitted to the individual WI agenda items.

A web conference may be used for handling some of the discussions in this agenda item.

By Web Conf (Thursday June 4th if needed)

R2-2004357	LS on updated Rel-16 RAN1 UE features lists for LTE (R1-2003070;; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core, 5G_V2X_NRSL-Core, TEI16	To:RAN2	Cc:RAN4

Rapporteur of each WI is expected to provide per-WI CR for capturing capabilities (if not done yet)

Noted

R2-2004362	LS on Rel-16 RAN4 UE features lists for LTE and NR (R4-2005192; contact: NTT DOCOMO)	RAN4	LS in	Rel-16	To:RAN2	Cc:RAN1

Rapporteur of each WI is expected to provide per-WI CR for capturing capabilities (if not done yet)

Noted

## 7.3	Even further mobility enhancement in E-UTRAN

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; target; Mar 20; WID: RP-190921)

No documents should be submitted to 7.3. Documents under 7.3 will be treated together with documents in 6.9.

A web conference may be used for handling some of the discussions in this WI, and summary document may be provided for some agenda items under 7.3.

### 7.3.1	Organizational

Including incoming LSs and rapporteur inputs (if any).

Including outcome of [Post109bis-e][928][LTE MOB] Stage-2 CR (China Telecom)

By Web Conf (Tuesday June 2nd)

Outcome of [Post109bis-e][928][LTE MOB] Stage-2 CR (China Telecom):

R2-2005214	Corrections to even further mobility enhancement in E-UTRAN	China Telecommunications	CR	Rel-16	36.300	16.1.0	1284	-	F	LTE_feMob-Core

- Samsung wonders what non-DAPS DRB means. Is PDCP re-establishment only triggered upon RA completion at target?

- Intel clarifies that DRB not configured with DAPS is non-DAPS DRB. No PDCP re-establishment triggering upon RA completion was agreed last time.

- Samsung clarifies that “Upon successful DAPS handover, UE establishes target cell non-DAPS DRB by re-establishing PDCP and RLC entities.” could be interpreted wrong as DAPS handover completion occurs after RA completion. vivo clarifies for non-DAPS DRBs, legacy behaviour is followed so re-establishment occurs at HO command reception.

- Ericsson thinks “UE fallbacks” could be “UE falls back”.

Email discussion [928] outcome

Some updates needed, go for offline discussion

Check if the same issue(s) exists for NR CR

[AT110-e][216][LTE]  LTE Stage-2 updates (China Telecom)

Scope:

    - Correct Stage-2 text to be according to agreements. Improvements over parts discussed online should also be considered.

Intended outcome:

    - Agreed CR to 36.300 CR in R2-2005756 for LTE UE capability signalling

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

R2-2005756	Corrections to even further mobility enhancement in E-UTRAN	China Telecommunications	CR	Rel-16	36.300	16.1.0	1284	1	F	LTE_feMob-Core	R2-2005214	Late

By Email

Offline email discussion [21] scope:

[AT110-e][211][LTE MOB] RRC CR (Ericsson)

Scope:

    - LTE RRC CR capturing LTE DAPS, LTE CHO and NR CPC changes agreed in this meeting

Intended outcome:

    - Agreed 36.331 CR for LTE and NR mobility in R2-2005757

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

R2-2005757	Corrections to Rel-16 LTE mobility enhancement	Ericsson Corporation	CR	Rel-16	38.331	16.0.0	XXXX	F	LTE_feMob-Core

### 7.3.2	Reduction in user data interruption during DAPS handover

This AI jointly addresses corrections to NR and LTE DAPS.

Including corrections to control and user plane for DAPS HO. All RRC-related corrections to DAPS should be submitted to ASN.1 review agenda items in 6.9.5 (NR RRC) and 7.3.4 (LTE RRC).

Tdoc Limitation per company: 2 tdocs

By Web Conf (Tuesday June 2nd)

MAC CRs:

R2-2004644	CR on 36.321 for LTE feMob	vivo	CR	Rel-16	36.321	16.0.0	1474	-	B	LTE_feMob-Core

R2-2004645	CR on 38.321 for NR mobility enhancement	vivo	CR	Rel-16	38.321	16.0.0	0744	-	B	NR_Mob_enh-Core

Email discussion [AT109bis-e][214] outcome from last meeting

PDCP CRs:

R2-2005058	CR on 38.323 for NR mobility enhancement	Huawei, HiSilicon, Mediatek Inc., LG Electronics	CR	Rel-16	38.323	16.0.0	0045	2	C	LTE_feMob-Core	R2-2003853

R2-2005059	CR on 36.323 for LTE feMob	Huawei, HiSilicon, Mediatek Inc., LG Electronics	CR	Rel-16	36.323	16.0.0	0282	2	C	LTE_feMob-Core	R2-2003854

Email discussion [AT109bis-e][213] outcome from last meeting

By Email

Offline email discussion [212] and [213] scopes:

[AT110-e][212][MOB] PDCP CRs for LTE and NR (Huawei)

Scope:

    - PDCP CRs for LTE and NR DAPS corrections agreed in this meeting

Intended outcome:

    - Agreed CR to 38.323 CR in R2-2005758 for NR PDCP changes agreed in this meeting

    - Agreed CR to 36.323 in R2-2005759 for LTE PDCP changes agreed in this meeting

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

PDCP CRs:

R2-2005758	CR on 38.323 for NR mobility enhancement	Huawei, HiSilicon, Mediatek Inc., LG Electronics	CR	Rel-16	38.323	16.0.0	0045	3	C	LTE_feMob-Core	R2-2003853

R2-2005759	CR on 36.323 for LTE feMob	Huawei, HiSilicon, Mediatek Inc., LG Electronics	CR	Rel-16	36.323	16.0.0	0282	3	C	LTE_feMob-Core	R2-2003854

[AT110-e][213][MOB] MAC CRs for LTE and NR (vivo)

Scope:

    - MAC CRs for LTE and NR DAPS corrections agreed in this meeting

Intended outcome:

    - Agreed CR to 38.321 CR in R2-2005760 for NR MAC changes agreed in this meeting

    - Agreed CR to 36.321 in R2-2005761 for LTE MAC changes agreed in this meeting

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

MAC CRs:

R2-2005760	CR on 36.321 for LTE feMob	vivo	CR	Rel-16	36.321	16.0.0	1474	1	B	LTE_feMob-Core

R2-2005761	CR on 38.321 for NR mobility enhancement	vivo	CR	Rel-16	38.321	16.0.0	0744	1	B	NR_Mob_enh-Core

By Web Conf (Wed, June 3rd)

UE actions upon DAPS failure and other cases

R2-2004699	Open issues for control plane aspects of DAPS handover	Ericsson	discussion	Rel-16	LTE_feMob-Core

Proposal 1	At DAPS handover failure, upon fallback to source cell, for each SRB, the UE discards any PDCP SDUs along with the PDCP data PDUs.

Proposal 2	At DAPS handover failure, upon fallback to source cell the UE performs RLC re-establishment for each SRB.

Proposal 3	If the UE receives MobilityFromEUTRACommand (LTE) or MobilityFromNRCommand (NR) after a DAPS handover but before the UE has released the source cell connection, the UE behaviour can be left unspecified.

Proposal 4	Add a note in the inter-RAT handover procedure to say that the UE behaviour is unspecified in this case.

Proposal 5	When UE enters RRC_INACTIVE,  it releases the previous source cell resources, when applicable.

Proposal 6	When UE triggers RRC connection re-establishment,  it releases the previous source cell resources, when applicable.

P1+2

- MediaTek supports both proposals. These are just clarifications how we have already agreed. Intel agrees. Samsung agrees but thinks all buffer data will be discarded. SDU discarding is not needed. vivo thinks we can just use PDCP re-establishment.

- OPPO thinks PDCP re-establishment will reset the COUNT value and could cause COUNT reuse. We can just discard the PDUs and SDUs. QC agrees.

- For P2, LGE thinks RLC re-establishment is not needed as network knows which RLC PDUs are outdated.

- Intel wonders what additional changes P1 would bring to RRC? Ericsson thinks some indication to trigger is sufficient. Intel clarifies that PDCP handles the discard based on RRC indication. OPPO thinks the indication also discards SDUs.

P3+4

- Intel thinks we agreed last time that this can’t happen for intra-RAT handover, so same could be done for inter-RAT. Huawei agrees. Nokia agrees. OPPO agrees.

- Ericsson clarifies this was the intention. Intel clarifies that we already capture something on this on intra-RAT handover.

P5+6

- OPPO thinks similar treatment as in P3 can apply. Intel thinks P6 is not yet agreed.

- Nokia wonders if there is no new behaviour. Ericsson thinks it’s not yet captured.

Agreements

1	At DAPS handover failure, upon fallback to source cell, for each SRB, the UE discards any PDCP SDUs along with the PDCP data PDUs.

2	At DAPS handover failure, upon fallback to source cell the UE performs RLC re-establishment for each SRB.

3	Inter-RAT handover during DAPS handover before release of source cell is not allowed in Rel-16.

4	Releasing UE to INACTIVE via RRCRelease during DAPS handover before release of source cell is not allowed in Rel-16.

5	When UE triggers RRC connection re-establishment, it releases the previous source cell resources, when applicable.

R2-2004896	Discussion on old stored RRC message handling upon DAPS HO failure	OPPO	discussion	Rel-16	NR_Mob_enh-Core

R2-2005513	Remaining issues on fallback from DAPS handover failure	SHARP Corporation	discussion	Rel-16	LTE_feMob-Core

R2-2005060	Discussion on DAPS CP remaining issue	Huawei, HiSilicon	discussion	Rel-16	LTE_feMob-Core

PDCP/RLC re-establishment for source cell SRBs:

R2-2004648	Handling of the source SRB at DAPS failure	vivo	discussion	Rel-16	LTE_feMob-Core

R2-2005497	Handling of RLC for SRBs	LG Electronics Inc.	discussion	NR_Mob_enh-Core, LTE_feMob-Core

RoHC feedback for source cell:

R2-2004697	RoHC feedback to source cell after UL transmission switch	Ericsson	discussion	Rel-16	LTE_feMob-Core

Proposal 1	Apply text proposal to TS 38.323 (as included in appendix) for transmission of RoHC feedback in the source cell. A similar change is also needed for TS 36.323.

- Huawei thinks this is already captured in PDCP specifications so this is not needed. LGE agrees.

=>Intent is agreeable, but is already captured in PDCP CR. Can discuss if something needs to be clarified further.

RoHC handling when security key doesn’t change during DAPS handover (related to [J033] discussed under 6.9.5):

R2-2004878	Compromised solutions for ROHC related security issue	Samsung	discussion	LTE_feMob-Core

Proposal 1. The security key is always updated for Rel-16 DAPS handover.

Proposal 2. If Proposal 1 is not agreeable, the following compromised solution is considered:

-	One common ROHC instance is used for the source and the target, based on an indicator.

-	PDCP data recovery-like retransmission is performed based on an indicator upon uplink data switching.

P1

- Ericsson thinks we shouldn’t force key change. QC agrees. Nokia agrees.

- LGE supports P1. Other options may take a long time. Huawei agrees since DAPS allows for 0ms interruption. OPPO supports this option. Intel thinks we only wanted to avoid delay in Rel-15.

P2

- Ericsson thinks this could be agreeable. Huawei thinks this would require third type of PDCP entity, which would require quite some effort. Nokia agrees.

- QC thinks this is typical intra-CU scenario and would like to support that. NEC supports this. CATT supports this as it improves performance.

- MediaTek agrees with Huawei and QC: While this is useful, it’s a big change.

Agreements

1	Do not specify any special handling for RoHC when security key is not updated in DAPS handover in Rel-16. (This means that changing security key ensures no problems occur, but it’s up to network implementation.) Add a NOTE to RRC/PDCP specification on this.

R2-2004563	ROHC Handling for DAPS Handover without Key Change	MediaTek Inc.	discussion

R2-2004788	Solutions for security issue in case of DAPS without key change	NEC	discussion	Rel-16	LTE_feMob-Core

R2-2005500	ROHC handling for DAPS HO without security key change	LG Electronics Inc.	discussion	NR_Mob_enh-Core, LTE_feMob-Core

R2-2004916	Discussion on ROHC handling in DAPS HO	OPPO	discussion	Rel-16	LTE_feMob-Core

R2-2004947	DAPS handover UP remaining key issues	Qualcomm India Pvt Ltd	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

R2-2004698	RoHC handling during DAPS handover without key change	Ericsson	discussion	Rel-16	NR_Mob_enh-Core	R2-2002589

(moved from 6.9.6)

R2-2005056	Discussion on ROHC handling in DAPS HO without key change	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

(moved from 6.9.6)

IR context maintenaince in target cell:

R2-2005057	Discussion on transmitting ROHC IR packets in target during DAPS HO	Huawei, HiSilicon, Vivo, Oppo, Apple, China Telecom, Samsung, LG Electronics, CATT, CMCC, Mediatek Inc., LG Uplus	discussion	Rel-16	LTE_feMob-Core

Observation 1: according to RAN3 BL CR, all downlink SDUs should be forwarded to target from source, and source can inform discarding of already successfully transmitted SDUs, which helps target to refresh storage buffer and determine which SDUs should be sent to UE.

Observation 2: RAN2 agree to introduce a PDCP status report for DAPS DRB, it can also help target determine the first SDU which should be sent to UE.

Observation 3: due to transmission delay of PDCP status report and continuous downlink transmission of data from source, the first several PDCP PDUs will be discarded, which means this duplicate discarding makes ROHC context totally missing in UE side.

Observation 4: for U-mode and O-mode ROHC compressor can enter into FO state from IR state without any ACK received, so specification intervention is still needed.

Observation 5: if all PDCP PDUs need to be decompressed before duplicate discarding, there should be three reordering functions in DAPS PDCP entity. It is too late to introduce this fundamental modification.

Observation 6: it depends on UE ROHC implementation whether and when to send a ROHC acknowledgement to target cell.

Observation 7: Even if this acknowledgement is received, we also need to specify a new ROHC compressor behavior, i.e. to allow compressor to operate in higher compression state only after compressor receives an acknowledgement from decompressor.

Proposal 1: RAN2 confirm to specify “For downlink, the ROHC protocol of the target cell maintains the IR state if operating in U-mode and O-mode during DAPS handover.”

Proposal 2: If companies don’t prefer to get specific ROHC mode involved, RAN2 to specify “For downlink, the ROHC protocol of the target cell maintains the IR state during DAPS handover.”

R2-2005161	Target cell’s ROHC behaviour for DAPS handover	Nokia, Nokia Shanghai Bell, Ericsson, Intel Corporation, NEC	discussion	Rel-16	LTE_feMob-Core

Observation 1: It is possible to minimise the ROHC failures by different network implementation options - including maintaining the IR state until source cell release after DAPS handover. Restricting network behaviour for specific implementation for this issue is not needed.

Proposal 1: For downlink, maintaining the header compression protocol IR state in U-mode and O-mode during DAPS handover is up to target cell. A corresponding note can be added to the specification (as shown in the Annex).

- MediaTek wonders that if we don’t specify this, UE behaviour will be different depending on what target cell RoHC does, i.e. whether it maintains the IR state or not. Is this acceptable to networks? FutureWei thinks UE behaviour is clear: If the RoHC context is known, UE decompresses, otherwise there’s RoHC decompression error. Nokia agrees – network just doesn’t know whether UE has the RoHC context. Ericsson agrees and thinks we don’t specify network behaviour.

- QC thinks we need to specify network behaviour as UE will receive many duplicate packets. UE might lose RoHC context. Network should only send IR packets.

- Nokia thinks that inter-node messages may resolve the IR packets.

Agreements

1	For downlink, the ROHC protocol of the target cell should maintain the IR state if operating in U-mode and O-mode during DAPS handover. This can be captured in a NOTE in PDCP specification.

Miscellaneous:

R2-2005448	Discussion of remaining issues for DAPS HO	CMCC	discussion	Rel-16	LTE_feMob-Core

R2-2004787	Handling of expiry of DataInactivityTimer for DAPS	NEC	discussion	Rel-16	LTE_feMob-Core

By Email (After Wednesday June 4th)

Offline email discussion [208] scope:

[AT110-e][208][LTE/NR MOB] User plane issues for DAPS (NN)

Scope:

    - Discuss issues remaining after DAPS UP session (TBD if needed)

Intended outcome:

    - Discussion summary in R2-2005753 (by email rapporteur).

Deadline for providing comments and for rapporteur inputs:

    - TBD

R2-2005753	Summary of discussion [208] on DAPS UP	NN	discussion	Late

Discussion is cancelled (not needed, no more known UP open issues)

### 7.3.3	UE capabilities for conditional handover and DAPS

Including UE capability aspects of LTE mobility WI. Any input on UE capabilities from RAN1/4 will be handled in this agenda item.

Including outcome of [Post109bis-e][931][LTE MOB] UE capabilities for NR mobility (China Telecom)

Tdoc Limitation per company: 1 tdoc.

By Web Conf (Wednesday June 3rd or Tuesday June 9th)

Outcome of [Post109bis-e][931][LTE MOB] UE capabilities for NR mobility (China Telecom):

R2-2005216	report of [Post109bis-e][931][LTE MOB] UE capabilities for NR mobility (China Telecom)	China Telecommunications	discussion	Late

Proposal 1: the conclusions on CHO in NR for the maximum candidate cells that the CHO capable UE must support and whether introducing additional capability on the support of 2 trigger events for same execution condition will be introduce into LTE.

Proposal 2: Not to introduce the UE power sharing and UL transmission cancellation capabilities for LTE in RAN2.

Proposal 3: Remove pdcch-BlindDetectionSource and pdcch-BlindDetectionTarget from RAN2 agreed capabilities.

Proposal 4: The following capabilities are introduced into LTE, which is aligned with NR,

-asyncDAPS-r16

-interFreqDAPS-r16

-interFreqDiffSCS-DAPS-r16

-singleUL-TransmissionDAPS-r16

-supportedNumberTAG-DAPS(it is not needed for interFreq since RAN2 agreed to “Reuse CA capability “supportedNumberTAG” for DAPS handover.)

-MultiUL-TransmissionDAPS

-syncDAPS.

Agreements (LTE)

12a introduce separate capabilities for intraFreq and interFreq as below:

Per Band/per BC (for intraFreq capabilities), I.e. put under BandParameters-v16xy:

intraFreqAsyncDAPS-r16

intraFreqMultiUL-TransmissionDAPS-r16

Per BC (for interFreq capabilities), i.e. put under CA-ParametersNR-v16xy:

interFreqAsyncDAPS-r16

interFreqMultiUL-TransmissionDAPS-r16.

12b	All UEs supporting DAPS support these capabilities (can discuss signalling details and naming):

SyncDAPS-r16

SingleUL-TransmissionDAPS-r16

intraFreqTwoTAGs-DAPS-r16  (with 2 TAGs)

(for interFreq since RAN2 agreed to “Reuse CA capability “supportedNumberTAG” for DAPS handover.)

10	Remove pdcch-BlindDetectionSource and pdcch-BlindDetectionTarget from RAN2 agreed capabilities.

11	Add syncDAPS and simultaneous UL transmission based on RAN4 latest capability table.

13	Introduce separate capabilities for intraFreq and interFreq for power sharing capabilities.

Discuss further about P1 over email (discussion [215])

Wait for RAN1 conclusion on ul-TransCancellationDAPS.

R2-2005217	UE Capability for Rel-16 LTE even further mobility enhancement	China Telecommunications	CR	Rel-16	36.331	16.0.0	4306	-	B	LTE_feMob-Core

R2-2005218	UE Capability for Rel-16 LTE even further mobility enhancement	China Telecommunications	CR	Rel-16	36.306	16.0.0	1763	-	B	LTE_feMob-Core

Remaining UE capability issues LTE:

R2-2004691	Open issues on UE capabilities at DAPS HO	Ericsson	discussion	Rel-16	LTE_feMob-Core

R2-2005685	Consideration on DAPS Capability	LG Electronics Inc.	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core	R2-2002905

R2-2005063	Discussion on UE capabilities for LTE DAPS	Huawei, HiSilicon	discussion	Rel-16	LTE_feMob-Core

(moved from 7.3.4)

By Email

Offline email discussion [215] scope:

[AT110-e][215][MOB] UE capability CRs for LTE mobility (China Telecom)

Scope:

    - 36.306 and 36.331 CRs for LTE capabilities based on agreements in this meeting

Intended outcome:

    - Agreed CR to 36.331 CR in R2-2005764 for LTE UE capability signalling

    - Agreed CR to 36.306 in R2-2005765 for LTE capability descriptions

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

R2-2005764	UE Capability for Rel-16 LTE even further mobility enhancement	China Telecommunications	CR	Rel-16	36.331	16.0.0	4306	1	B	LTE_feMob-Core

R2-2005765	UE Capability for Rel-16 LTE even further mobility enhancement	China Telecommunications	CR	Rel-16	36.306	16.0.0	1763	1	B	LTE_feMob-Core

### 7.3.4	ASN.1 review of mobility WIs for LTE RRC

This agenda item focuses on LTE RRC aspects of both LTE and NR mobility WIs – NR RRC aspects of both LTE and NR mobility WIs should be submitted to 6.9.5. Do not submit contributions on WI-specific open issues that are not captured in the current LTE RRC to this agenda item.

All ASN.1 issues should be raised in RILs first – contributions where no RIL issue exists may not be treated.

Including contributions/TPs on RRC corrections based on review issues. For these, no individual company CRs should be submitted: please consult with the rapporteur of LTE RRC CR first (cecilia.eklof@ericsson.com).

By Web Conf (Tuesday June 2nd)

R2-2004621	Updates for R16 LTE Mobility Enhancements and LTE updates for R16 NR Mobility Enhancements	Ericsson	CR	Rel-16	36.331	16.0.0	4290	1	F	LTE_feMob-Core	R2-2003852

R2-2004695	[E928][I114] Condition for setting statusReportRequired for RLC UM	Ericsson, Intel Corporation	discussion	LTE_feMob-Core

Observation 1	For RLC AM radio bearers the statusReportRequired field is configured at setup of the DRB (PDCP entity) and can then later be reconfigured if needed.

Observation 2	The statusReportRequired field is typically configured when the radio bearer is setup and then used at the related events.

Observation 3	For RLC UM radio bearers the statusReportRequired field can currently only be configured within the (DAPS) HO Command message.

Observation 4	The size of the HO Command message should be kept as small as possible to avoid negative impacts on HO success rate.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The condition for inclusion of the statusReportRequired field for RLC-UM radio bearers should be changed so that it is not restricted to when the bearer is configured for DAPS. It should instead be restricted to when the UE supports DAPS.

Proposal 2	In 36.331, if the statusReportRequired field has not been configured for an RLC-UM radio bearer it should by default have the value FALSE. It should then be optional, need ON.

Proposal 3	In 36.331 the name of the struct “rlc-AM”, which includes the statusReportRequired field, should be changed to “rlc-AM-UM”.

Proposal 4	Include the Text Proposal in Annex A.1 to TS 38.331.

Proposal 5	Include the Text Proposal in Annex A.2 to TS 36.331.

R2-2005350	[Z263] Discussion on UE configuration release in RRC re-establishment	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_feMob-Core

Observation 1: According to the current RRC CR, the UE shall perform MR-DC release and release some configured UE configuration (e.g. uplinkDataCompression, UE configuration included in RadioResourceConfigDedicated and UE configuration included in otherConfig except for delayBudgetReportingConfig and overheatingAssistanceConfig) upon initiation of RRC re-establishment, if the UE is not configured with conditionalReconfiguration.

Observation 2: In case the UE selects a CHO candidate cell whose matching configuration includes the SCG delta configuration, releasing the SCG configuration upon initiation of RRC re-establishment may cause the reconfiguration failure when applying the stored CHO configuration.

Observation 3: Similar to delayBudgetReportingConfig and overheatingAssistanceConfig, releasing UE configuration included in otherConfig, RadioResourceConfigDedicated, and uplinkDataCompression during the RRC re-establishment initiation phase may cause the mismatching of configuration between the UE side and the target side if the UE selects a CHO candidate cell.

Proposal 1: The UE does not perform MR-DC release and does not release UE configuration if the UE was configured with conditionalReconfiguration and the selected cell during re-establishment is a CHO candidate cell.

Proposal 2: Remove the description about release of UE configuration upon initiation of RRC re-establishment in section 5.3.7.2, and then capture the corresponding description in section 5.3.7.3 in case the selected cell is not a CHO candidate cell.

### 7.3.5	Other

Only corrections not fitting other agenda items.

Including CHO aspects that are LTE-specific without equivalent NR impacts: Do not use this AI for any item that can be discussed jointly for LTE and NR - Contributions on conditional handover that apply for both LTE and NR are treated jointly in under 6.9.3.

Tdoc Limitation per company: 1 tdoc.

R2-2004692	Power coordination at DAPS HO in LTE	Ericsson	discussion	Rel-16	LTE_feMob-Core

Observation 1	RAN1 are not working on DAPS HO for LTE in Rel-16. DAPS HO is thus not specified in TS 36.213.

Proposal 1	No power coordination information parameters should be specified for DAPS handover in 36.331 (in Rel-16).

Proposal 2	No capability for uplink power sharing at DAPS handover should be specified in 36.306 (in Rel-16).

Proposal 3	In case of overlapping UL transmissions at DAPS handover in LTE, the UE shall only transmit in the target cell.

R2-2005384	Discussion on LTE specific CHO issues	Huawei, HiSilicon	discussion	Rel-16	LTE_feMob-Core

Proposal: It is proposed to agree on the following clarification to TS 36.300 so that LTE CHO is not supported in LTE-5GC:

In the text and figure(s) in the following clauses (except for 10.1.2.1a), intra-E-UTRA HO description is applicable for both intra-EPC and intra-5GC cases.

## 7.4	Further performance enhancement for LTE in high speed scenario

(LTE_high_speed_enh2-Core; leading WG: RAN4; REL-16; started: Jun 18; target; Sep 19; WID: RP-181482)

Including documents related to WI-specific ASN.1 review issues.

A web conference may be used for handling some of the discussions in this agenda item.

## 7.5	Other LTE Rel-16 WIs

This agenda item is to be used for LSs and documents relating to Rel-16 LTE but for which there is no existing RAN WI/SI (e.g. LSs from CT/SA requesting RAN2 action) or for which there is no allocated RAN2 time.

A web conference may be used for handling some of the discussions in this WI.

### 7.5.0	In-principle Agreed CRs

### 7.5.1	Other

By Email

SA5 LSs for QMC:

R2-2004381	LS on Reply on QoE Measurement Collection (S5-202304; contact: Ericsson)	SA5	LS in	Rel-16	QOED	To:SA4, CT1, RAN2, RAN3

(moved from 7.5)

R2-2004382	LS on Reply on QoE Measurement Collection (S5-202305; contact: Ericsson)	SA5	LS in	Rel-16	QOED	To:RAN2, RAN3	Cc:CT1, SA4

(moved from 7.5)

Handled in offline email discussion [204]

Discussion on SA5 LSs on QMC

R2-2004623	Handling of incoming LS on QoE Measurement Collection	Ericsson	discussion	TEI16

R2-2005385	Discussion on QMC regarding incoming SA5 LS	Huawei, HiSilicon	discussion	Rel-16	LTE_QMC_Streaming-Core

(moved from 7.6.1)

Handled in offline email discussion [204]

Draft CR based on LSs:

R2-2004624	QoE Measurement Collection additions	Ericsson	CR	Rel-16	36.331	16.0.0	4297	-	C	TEI16

Handled in offline email discussion [204]

Draft LS replies:

R2-2004625	Draft LS Reply on QoE Measurement Collection	Ericsson	LS out	Rel-16	TEI16	To:SA5	Cc: RAN3, SA4, CT1

R2-2005386	Draft reply LS to R2-2004381	Huawei	discussion	Rel-16	LTE_QMC_Streaming-Core

(moved from 7.6.1)

R2-2005387	Draft reply LS to R2-2004382	Huawei	discussion	Rel-16	LTE_QMC_Streaming-Core

(moved from 7.6.1)

Handled in offline email discussion [204]

Offline email discussion [204] scope:

[AT110-e#204][LTE] Handling of SA5 LS replies on QoE Measurement Collection (Ericsson)

Scope:

    - Discuss the LS replies received from SA5 in R2-2004381 and R2-2004382

    - Discuss the input documents in R2-2004623 and R2-2005385 to determine what RAN2 needs to do

    - Discuss whether to send reply LS to SA5 (CC: TBD) and, if agreeable, provide updated LS according to discussion in

Intended outcome:

    - Discussion summary in R2-2005748 (by email rapporteur)

    - If agreeable, LS to RANx (exact groups TBD) informing on the outcome of RAN2 in R2-2005749

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  Thursday 2020-06-04 10:00 UTC

    - Initial deadline (for rapporteur's summary in R2-2005741):  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

By Web Conf (Friday June 5th)

R2-2005748	Summary of discussion [204] on Handling of SA5 LS replies on QoE Measurement Collection (Ericsson)	Ericsson	discussion	TEI16, LTE_QMC_Streaming-Core	Late

R2-2005749	Draft LS Reply on QoE Measurement Collection	Ericsson	LS out	Rel-16	TEI16, LTE_QMC_Streaming-Core	To:SA5	Cc: RAN3, SA4, CT1

## 7.6	LTE TEI16 enhancements

Small Technical Enhancements to LTE. TEI should be predominantly within a single WG and fully completed within the same quarter in all affected WGs. RAN2 impact of RAN1/4-led TEI shall be limited to RRC signalling of configuration parameters and UE capabilities (no MAC impact, no RRC procedural impact, etc). Please also see RP-191602 endorsed at RAN#84.

Including documents related to TEI16 ASN.1 review issues.

New TEI16 proposals are discouraged and may be deprioritized in this meeting.

A web conference may be used for handling some of the discussions in this agenda item.

### 7.6.0	In-principle Agreed CRs

By Email

R2-2004818	CR on RLC out-of-order delivery configuration	Samsung, LG Electronics Inc., Nokia, Nokia Shanghai Bell, Intel, Apple	CR	Rel-15	36.323	15.5.0	0283	1	F	TEI15, LTE_HRLLC-Core	R2-2003860

R2-2004820	CR on RLC out-of-order delivery configuration	Samsung, LG Electronics Inc., Nokia, Nokia Shanghai Bell, Intel, Apple	CR	Rel-16	36.323	16.0.0	0284	1	A	TEI16	R2-2003861

R2-2004826	CR on RLC out-of-order delivery configuration	Samsung, LG Electronics Inc., Nokia, Nokia Shanghai Bell, Intel, Apple	CR	Rel-15	36.331	15.9.0	4288	1	F	TEI15, LTE_HRLLC-Core	R2-2003862

R2-2004827	CR on RLC out-of-order delivery configuration	Samsung, LG Electronics Inc., Nokia, Nokia Shanghai Bell, Intel, Apple	CR	Rel-16	36.331	16.0.0	4240	2	F	TEI16	R2-2003863

Handled in offline email discussion [205]

### 7.6.1	Other

By Email

Offline email discussion [205] scope:

[AT110-e#205][LTE] LTE contributions in AIs 7.6, 7.8 and 7.9 (RAN2 VC)

Scope:

    - Handle the contributions in AIs 7.6.0, 7.8 and 7.9

Intended outcome:

    - Discussion summary in R2-2005750 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline for companies' feedback:  Thursday 2020-06-04 10:00 UTC

    - Initial deadline for rapporteur's summary in R2-2005750:  Friday 2020-06-05 03:00 UTC

    - Whether to continue the discussion after this TBD during Friday 2020-06-05 online session

By Web Conf (Friday June 5th)

R2-2005750	Summary of discussion [205] on LTE contributions in AIs 7.6, 7.8 and 7.9	Nokia (RAN2 VC)	discussion	LTE_HRLLC-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core 	Late

## 7.8	DL MIMO efficiency enhancements for LTE

(LTE_DL_MIMO_EE-Core; leading WG: RAN1; REL-16;target; March-20; WID: RP-182901)

Including documents related to WI-specific ASN.1 review issues.

A web conference may be used for handling some of the discussions in this agenda item.

By Email

UE capabilities based on latest RAN1 LS:

R2-2005488	Introduction of UE capabilities for DL MIMO efficiency enhancement	Huawei, Hisilicon	CR	Rel-16	36.331	16.0.0	4334	-	B	LTE_DL_MIMO_EE-Core

R2-2005489	Introduction of UE capabilities for DL MIMO efficiency enhancement	Huawei, Hisilicon	CR	Rel-16	36.306	16.0.0	1770	-	B	LTE_DL_MIMO_EE-Core

Handled in offline email discussion [205]

## 7.9	LTE-based 5G Terrestrial Broadcast

(LTE_terr_bcast-Core; leading WG: RAN1; REL-16; target; March-20; WID: RP-182924)

Including documents related to WI-specific ASN.1 review issues.

A web conference may be used for handling some of the discussions in this agenda item.

By Email

Corrections to subframe allocation:

R2-2004429	Correction on the configuration of subframe #0 and #5 for MCH in MBMS dedicated cell	Qualcomm Incorporated	CR	Rel-16	36.331	16.0.0	4259	2	F	LTE_terr_bcast-Core	R2-2003866

Handled in offline email discussion [205]

Corrections to MCCH configuration:

R2-2005490	Clarification on MCCH configuration for 0.37kHz SCS	Huawei, Hisilicon	CR	Rel-16	36.331	16.0.0	4335	-	F	LTE_terr_bcast-Core

Handled in offline email discussion [205]

UE capabilities based on latest RAN1 LS:

R2-2005224	MBMS UE capabilities per band for subcarrier spacing of 2.5 kHz and 0.37 kHz	Qualcomm Technologies Int	CR	Rel-16	36.331	16.0.0	4307	-	F	LTE_terr_bcast-Core

R2-2005227	MBMS UE capabilities per band for subcarrier spacing of 2.5 kHz and 0.37 kHz	Qualcomm Technologies Int	CR	Rel-16	36.306	16.0.0	1764	-	F	LTE_terr_bcast-Core

Handled in offline email discussion [205]