3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [011][NR15] Inter-Node RRC

Agenda Item:	5.4.1.4

Document for:	Discussion and decision

# Introduction

This document contains a list of TDocs covered in the following offline discussion:

- [AT110e][011][NR15] Inter-Node RRC (Huawei)

Scope: Treat R2-2004337, R2-2005182, R2-2005235, R2-2005236, R2-2005237, R2-2005167, R2-2005168, R2-2005574, R2-2005576, (proponents are responsible to explain and drive)

Part 1: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

Companies are invited to share their views on each TDoc submitted.

# Discussion

## R2-2005236, R2-2005237 Handover preparation with SN terminated bearers

Problem statement:

The following is excerpted from 37.340 Chapter 10.2.1:

The Secondary Node Addition procedure is initiated by the MN and is used to establish a UE context at the SN to provide resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the first cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). Figure 10.2.1-1 shows the Secondary Node Addition procedure.

The highlighted part suggests that “SN radio bearer configuration” and “SCG configuration” are two concepts.

However, in the text related to handover procedures, it is unclear whether “SN radio bearer configuration” is provided, only “SCG configuration” is mentioned.

Solution: Add radio bearer configuration into Stage 2 spec.

Q1: Do you agree with the changes in R2-2005167/R2-2005168?

| Company | YES/NO | Comments |
| --- | --- | --- |

## R2-2004337 Reply LS on handover without SN configuration query (contact: Huawei)

Q2: Do you think any RAN2 action is needed for this LS? If YES, please clarify.

| Company | Any action needed? (YES/NO) | Comments |
| --- | --- | --- |

## R2-2005182 Handover without fetching source SN config

Proposal 1: RAN2 confirms that, for handover in EN-DC, when MN initiates handover while up-to-date source SN information is not available, legacy IEs sourceSCG-ConfiguredNR and sourceRB-ConfigSN-NR (with drb-ToAddModList) in 36.331 HandoverPreparationInformation message can be used by source MN to indicate the SN terminated DRB list to target MN.

Proposal 2: RAN2 confirms that, for handover in NGEN-DC, NR-DC and NE-DC, when MN initiates handover while up-to-date source SN information is not available, legacy IEs sourceSCG-Configured and sourceRB-SN-Config (with drb-ToAddModList) in 38.331 HandoverPreparationInformation message can be used by source MN to indicate the potential SN terminated DRB list to target MN.

Q3: Do you agree with Proposal 1?

| Company | YES/NO | Comments |
| --- | --- | --- |

Q4: Do you agree with Proposal 2?

| Company | YES/NO | Comments |
| --- | --- | --- |

## R2-2005167, R2-2005168 Measurement coordination in MR-DC

Q5: Do you agree with the changes in R2-2005167/R2-2005168?

| Company | YES/NO | Comments |
| --- | --- | --- |

## R2-2005574, R2-2005576 Introduction of p-MaxUE-FR1 in the inter-node message

Q6: Do you agree with the changes in R2-2005574, R2-2005576?

| Company | YES/NO | Comments |
| --- | --- | --- |

# Conclusion

To be added.