3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [011][NR15] Inter-Node RRC

Agenda Item:	5.4.1.4

Document for:	Discussion and decision

# Introduction

This document contains a list of TDocs covered in the following offline discussion:

- [AT110e][011][NR15] Inter-Node RRC (Huawei)

Scope: Treat R2-2004337, R2-2005182, R2-2005235, R2-2005236, R2-2005237, R2-2005167, R2-2005168, R2-2005574, R2-2005576, (proponents are responsible to explain and drive)

Part 1: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

Companies are invited to share their views on each TDoc submitted.

# Discussion

## R2-2005236, R2-2005237 Handover preparation with SN terminated bearers

Problem statement:

The following is excerpted from 37.340 Chapter 10.2.1:

The Secondary Node Addition procedure is initiated by the MN and is used to establish a UE context at the SN to provide resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the first cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). Figure 10.2.1-1 shows the Secondary Node Addition procedure.

The highlighted part suggests that “SN radio bearer configuration” and “SCG configuration” are two concepts.

However, in the text related to handover procedures, it is unclear whether “SN radio bearer configuration” is provided, only “SCG configuration” is mentioned.

Solution: Add radio bearer configuration into Stage 2 spec.

Q1: Do you agree with the changes in R2-2005236/R2-2005237?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | No | When this topic was discussed in RAN2#108, the understanding was that this was a kind of optimization rather to fix something that is broken. According to the reply we got from RAN3, it seems that they have also the same understanding. In fact, in the LS is stated: ““a reasonable implementation would try to keep all the bearers during mobility and take appropriate measures, like triggering the handover preparation sufficiently early. In case a source node would nevertheless skip the SN Configuration Query procedure, the target side would need to go for full configuration, which does not quite fit into a scenario of an urgent handover.” According to this, we prefer to not pursue any change in current specification. [Huawei] The changes in R2-2005236, R2-2005237 are not closely related to the reply LS from RAN3. |
| ZTE | No | We updated the question accordingly (hope it is correct). One question for clarification, we understand the CRs and corresponding discussion paper R2-2005235 relate to the below discussion of LS and R2-2005182? First, we disagree with the proposals in R2-2005235. Because the proposed solution is complex and results in signalling overhead between MN and SN. Second, regarding changes in R2-2005236/R2-2005237, as explained by the rapporteur, seems it is intended to solve another issue. We understand the term “MCG configuration” and “SCG configuration” mentioned in TS37.340 can include both radio configuration and radio bearer configuration. Thus the highlight sentence may not very accurate. Different from what proposed in the CRs (involves too many changes), we suggest to simply revise the highlight sentence as below: This procedure can also be used to configure an SN terminated MCG bearer (where no SCG radio configuration is needed). [Huawei] Our concern is that, this way a new concept of “SCG radio configuration” is introduced, which may make the spec less readable. |
| vivo | Yes | We agree this is an optimization, but the specification impact is not so major, so think it is not a bad idea to have it. |
| Nokia | No | As pointed out during earlier meetings, Stage 2 doesn’t have to be perfectly mirroring every single Stage-3 signalling, so we don’t see anything as needed here. [Huawei] It is not merely misalignment between Stage 2 and Stage 3 specs, the Stage 2 spec itself has some misalignment. We would prefer to fix this to avoid confusion. |
| Huawei | Yes | As the proponent of the CR, we need to clarify that the changes in R2-2005236, R2-2005237 are not closely related to the reply LS from RAN3. Rather, they are due to some mistakes in the stage 2 CR. |

## R2-2004337 Reply LS on handover without SN configuration query (contact: Huawei)

Q2: Do you think any RAN2 action is needed for this LS? If YES, please clarify.

| Company | Any action needed? (YES/NO) | Comments |
| --- | --- | --- |
| Ericsson | No |  |
| ZTE | No | We prefer to do nothing, from network perspective, one (rough) solution is that the target MN can put all DRB IDs in addition to used MCG DRB IDs in the toReleaseList. If majority company want to introduce more perfect solution, then the proposed solution in R2-2005182 looks simpler and acceptable to us. |
| vivo | Yes with comments | I agree with ZTE, the solution considered in R2-2005182 is quite acceptable for us. |
| Nokia | Yes with comments | In the case handover without SN configuration query, we understand the key point is target MN want to use endc-ReleaseAndAdd for SCG full configuration while the UE will not release the SN terminated radio bearer configuration when receiving this IE. Source MN need to indicate target MN a list of DRBs which are in use or potentially in use by the source SN, to enable target MN release source SN terminated DRBs. In RAN2#108 meeting, RAN2 confirms the expected behaviour and send LS to ask RAN3 solution. R2-1915570 Summary of E-mail discusion [107bis#33][NR R15] Exchanging used IDs of SN terminated DRBs Huawei, HiSilicon discussion Rel-15 NR_newRAT-Core Late => RAN2 confirms that for handover, there is a need to allow the source MN to indicate to the target MN that up-to-date information is not available for a list of DRBs, which are in use or potentially in use by the SN.. We think above RAN2 conclusion is still valid. Considering RAN3 does not apply any specification change in reply LS to support this scenario, we think RAN2 INM solution is needed. Otherwise, it may cause SN configuration mismatch between UE and NW (e.g. PDCP configuration of SN terminated DRB). |
| Huawei | Yes with comments | The solution in R2-2005182 is acceptable to us, however, to avoid inter-operability issues, we suggest to add a new DRB list to serve the purpose instead of reusing the existing IE. |

## R2-2005182 Handover without fetching source SN config

Proposal 1: RAN2 confirms that, for handover in EN-DC, when MN initiates handover while up-to-date source SN information is not available, legacy IEs sourceSCG-ConfiguredNR and sourceRB-ConfigSN-NR (with drb-ToAddModList) in 36.331 HandoverPreparationInformation message can be used by source MN to indicate the SN terminated DRB list to target MN.

Proposal 2: RAN2 confirms that, for handover in NGEN-DC, NR-DC and NE-DC, when MN initiates handover while up-to-date source SN information is not available, legacy IEs sourceSCG-Configured and sourceRB-SN-Config (with drb-ToAddModList) in 38.331 HandoverPreparationInformation message can be used by source MN to indicate the potential SN terminated DRB list to target MN.

Q3: Do you agree with Proposal 1?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | No | This topic was discussed already in RAN2#108 and we agreed to not pursue any solution in the inter-node RRC message. This was also the motivation for having sent the LS to RAN3. Further, when this topic was discussed in RAN2#108, the understanding was that this was a kind of optimization rather to fix something that is broken. According to the reply we got from RAN3, it seems that they have also the same understanding. In fact, in the LS is stated: ““a reasonable implementation would try to keep all the bearers during mobility and take appropriate measures, like triggering the handover preparation sufficiently early. In case a source node would nevertheless skip the SN Configuration Query procedure, the target side would need to go for full configuration, which does not quite fit into a scenario of an urgent handover.” According to this, we prefer to not pursue any change in current specification. |
| ZTE | Yes with comments | As replied to Q2, we prefer to do nothing, if majority companies pursue more perfect solution, then proposal 1 is acceptable to us. |
| vivo | See Q2 |  |
| Nokia | Yes | We are the proponent company for this. As replied in Q2, we think solution is needed and we should avoid any Rel-15 ASN.1 impact if possible. In EN-DC, as the MN knows the SN terminated DRB IDs at all time, it is feasible to use these legacy IEs to indicate SN terminated bearers which should be released. |
| Huawei | Yes with comments | The solution in R2-2005182 is acceptable to us, however, to avoid inter-operability issues, we suggest to add a new DRB list to serve the purpose instead of reusing the existing IE. |

Q4: Do you agree with Proposal 2?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | No | This topic was discussed already in RAN2#108 and we agreed to not pursue any solution in the inter-node RRC message. This was also the motivation for having sent the LS to RAN3. Further, when this topic was discussed in RAN2#108, the understanding was that this was a kind of optimization rather to fix something that is broken. According to the reply we got from RAN3, it seems that they have also the same understanding. In fact, in the LS is stated: ““a reasonable implementation would try to keep all the bearers during mobility and take appropriate measures, like triggering the handover preparation sufficiently early. In case a source node would nevertheless skip the SN Configuration Query procedure, the target side would need to go for full configuration, which does not quite fit into a scenario of an urgent handover.” According to this, we prefer to not pursue any change in current specification. |
| ZTE | Yes with comments | As replied to Q2, we prefer to do nothing, if majority companies pursue more perfect solution, then proposal 2 is acceptable to us. And we understand the “potential SN terminated DRB list” reflects the DRB ID range allocated to source SN. [Nokia]: Yes, we have the same understanding for “potential SN terminated DRB list”. |
| vivo | See Q2 |  |
| Nokia | Yes | We are the proponent company for this. In NGEN-DC, NR-DC and NE-DC, as source MN don’t know exactly source SN terminated bearers without fetching SCG config, we prefer simple solution without impact to Rel-15 ASN.1 to only indicate potential SN terminated DRB list. Otherwise the network can either fetch latest source SN configuration during handover or trigger full configuration in both MN and SN. |
| Huawei | Yes with comments | The solution in R2-2005182 is acceptable to us, however, to avoid inter-operability issues, we suggest to add a new DRB list to serve the purpose instead of reusing the existing IE. |

## R2-2005167, R2-2005168 Measurement coordination in MR-DC

Q5: Do you agree with the changes in R2-2005167/R2-2005168?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | Yes | We are the proponent company for this. The reason why we brought these CR is because we are doing an exercise to list in section 11.2.3 of the inter-node message all those fields that do not need to signal all the time if their value to not change. For the case of the new signalling introduced for the measurement identities coordination, the understanding is that, once the MN signals the restrictions to the SN, the MN does not need to signal those restriction again unless their values are not changed. For these reasons, we believe that the two fields maxInterFreqMeasIdentitiesSCG and maxIntraFreqMeasIdentitiesSCG should be part of the list in section 11.2.3. |
| ZTE | No | In fact the meaning when the two fields are absent is already stated in the field descriptions. The intention is that SN needs this value no matter of the gNB version, otherwise, SN is unclear how many measIDs can be configured by SN. And assuming “0” definitely causes more inter-operability issues. So there’s no need to add maxInterFreqMeasIdentitiesSCG and maxIntraFreqMeasIdentitiesSCG fields to the list. However, for exchanged serving frequency list (“servFrequenciesMN-NR”, “scellFrequenciesSN-EUTRA” and “scellFrequenciesSN-NR”), we think they can be added to that list, and receiving node can assume the (serving cell) configuration is not changed when the field is absent. |
| vivo | No | Agree with ZTE, the corresponding field description have already addressed, the case where they are absent: “If the field is absent, the SCG is allowed to configure inter-frequency measurements up to the maximum value” and “If the field is absent, the SCG is allowed to configure intra-frequency measurements up to the maximum value on each serving frequency”. So we do not think the CR is needed |
| Nokia | No | In maxIntraFreqMeasIdentitiesSCG / maxInterFreqMeasIdentitiesSCG field description, the field absence means the SCG is allowed to configure intra-freq/inter-freq measurements up to the maximum value. So, if MCG want SCG to maintain previous signed value, it is not possible to use IE absence. Thus, these IEs should be re-signalled in INM and cannot be omitted. For exchanged serving frequency list (“servFrequenciesMN-NR”, “scellFrequenciesSN-EUTRA” and “scellFrequenciesSN-NR”), we have similar view as ZTE. |
| Huawei | No | We think the CR will cause inter-operability issues. |

## R2-2005574, R2-2005576 Introduction of p-MaxUE-FR1 in the inter-node message

Q6: Do you agree with the changes in R2-2005574, R2-2005576?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | No | We think the CRs are not need. During RRC re-establishment and RRC resume, this power is explicitly released by the UE as well as in the case of handover. Further, this field in 36.331 is Need OR and thus if the target node does not signal it, this means that the UE should release this field. |
| ZTE | No | We have checked the reason of adding p-MaxUE-EUTRA to AS-Config (R2-1902711 cr3937). Seems purely for signalling optimization. (The p-MaxUE-EUTRA is defined as “Need ON” in Uu interface) However, the p-MaxUE-FR1 field is defined as “Need OR” in Uu interface, which means it anyway can not support delta configuration, thus the benefit of forwarding it during handover is gone. |
| vivo |  | We this is an optimization, not critical |
| Nokia | No | As p-MaxUE-FR1 is “Need OR”, it seems no need to transfer to target for delta configuration. |
| Huawei | No | Target eNB can configure a new value to the UE, so the CR looks not essential. |

# Conclusion

To be added.