3GPP TSG-RAN WG2 #110e	Tdoc R2-20xxxxx

Electronic meeting, June 1st – 12th 2020

Agenda Item:	6.0.3

Source:	Ericsson (Rapporteur)

Title:	Outcome of [AT110-e][062][NR16] MAC updates

Document for:	Discussion, Decision

1	Introduction

This document pertains to the following e-mail discussion:

[AT110-e][062][NR16] MAC updates (Ericsson)

Scope: Treat R2-2005328, R2-2005501, R2-2005502, R2-2005562. Multi-WI MAC corrections.

Wanted Outcome: Agreed CR

Deadline for first round: June 5, 0900 UTC

Deadline for second round: June 11, 0700 UTC

During the first round the intention is to decide which CRs to pursue for the second round. The second round will be used for updating and merging the CRs which continue from the first round.

The rapporteur invites companies to provide input well in advance of the deadline in order for a productive discussion to take place.

2	First round of discussion

2.1	Discussion

R2-2005501	38321 CR Clarification on eLCID	LG Electronics Inc., MediaTek	CR	Rel-16	38.321	16.0.0	0752	-	F	TEI16

Companies are invited to state their opinion on the CR above (R2-2005501).

| Company | Opinion |
| --- | --- |
| LG | Support the CR. This CR is produced based on the outcome of the e-mail discussion at the last meeting [AT109bis-e][060][NR16] MAC eLCID and RACH stopping. |

R2-2005562	Handling of unexpected eLCID values.	ASUSTeK	discussion	Rel-16	38.321

Companies are invited to state their opinion on the CR above (R2-2005562).

| Company | Opinion |
| --- | --- |
| LG | Support the CR. |

R2-2005328	Alignment of SR clause	Ericsson, Samsung	CR	Rel-16	38.321	16.0.0	0732	1	F	NR_unlic-Core, NR_eMIMO-Core	R2-2003833

Companies are invited to state their opinion on the CR above (R2-2005328).

| Company | Opinion |
| --- | --- |
| LG | Agree with the intention. We need more time to check the actual changes. |

R2-2005502	Stopping ongoing Random Access procedure	LG Electronics Inc.	discussion	Rel-16	TEI16

Companies are invited to state their opinion on the CR above (R2-2005502).

| Company | Opinion |
| --- | --- |
| LG | Support the TP in R2-2005502. At least, it is asked for RAN2 to take some exercise to simplify the text on the UE optional behavior. |

2.2	Conclusion

TBD