3GPP TSG-RAN2 Meeting #110-e	R2-2005765

# Online, , 1st Jun 2020 - 12th Jun 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1763 | rev | -1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Capability for Rel-16 LTE even further mobility enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture capabilities for LTE mobility enhancement into stage 3 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To capture below capabilities: DAPS: intraFreqDAPS-r16; intraFreqAsyncDAPS-r16 intraFreqMultiUL-TransmissionDAPS-r16 interFreqDAPS-r16 interFreqAsyncDAPS-r16 interFreqMultiUL-TransmissionDAPS-r16 intraFreqTwoTAGs-DAPS-r16 uplinkPowerSharingDAPS-r16 ul-TransCancellationDAPS-r16 (Editor's note: ul-TransCancellationDAPS is FFS and may need update on RAN1 conclusion.) CHO: cho-FDD-TDD-r16 cho-r16 cho-Failure-r16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capability part for LTE moiblity enhancement is missing in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4, 4.3.5, 4.3.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------[Changes Start]-----------------------------------------------------

### 4.3.4	Physical layer parameters

------------unchanged part omitted-------------

#### 4.3.4.191	widebandPRG-Slot-r16, widebandPRG-Subslot-r16, widebandPRG-Subframe-r16

This field indicates whether the UE supports wideband precoding resource block group size for slot/subslot/subframe PDSCH operation as specified in TS 36.213 [22].

#### 4.3.4.x1	ul-TransCancellationDAPS-r16

This field indicates support of cancelling UL transmission to the source PCell.

#### 4.3.4.x2	uplinkPowerSharingDAPS-r16

This field indicates whether the UE supports UL power sharing during DAPS handover.

-----------------------------------------[Next Change]-----------------------------------------------------

### 4.3.5	RF parameters

#### 4.3.5.3	multipleTimingAdvance

This field defines whether multiple timing advances are supported for each band combination supported by the UE. It is mandatory for UEs of this release of the specification to support this capability for band combinations having an UL on multiple FDD bands as specified in TS 36.101 [6]. If the band combination comprised of more than one band entry (i.e., inter-band or intra-band non-contiguous band combination), the field indicates that different timing advances on different band entries are supported. If the band combination comprised of one band entry (i.e., intra-band contiguous band combination), the field indicates that different timing advances across component carriers of the band entry are supported. This field is mandatory for UEs supporting DAPS handover.

--------------unchanged part omitted---------------------

#### 4.3.5.x1	intraFreqDAPS-r16

This field indicates whether UE supports DAPS handover in source PCell and intra-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell.

#### 4.3.5.x2	intraFreqAsyncDAPS-r16

This field indicates whether the UE supports asynchronous DAPS handover in source PCell and intra-frequency target PCell.

#### 4.3.5.x3	intraFreqMultiUL-TransmissionDAPS

This field indicates that the UE supports simultaneous UL transmission in source PCell and intra-frequency target PCell.

#### 4.3.5.x4	interFreqDAPS-r16

This field indicates whether the UE supports DAPS in source PCell and inter-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell.

#### 4.3.5.x5	interFreqAsyncDAPS-r16

This field indicates whether the UE supports asynchronous DAPS handover in source PCell and inter-frequency target PCell.

#### 4.3.5.x6	interFreqMultiUL-TransmissionDAPS

This field indicates that the UE supports simultaneous UL transmission in source PCell and inter-frequency target PCell.

#### 4.3.5.x9	intraFreqMultiTAG-DAPS-r16

This field indicates whether the UE supports different timing advance groups in source PCell and intra-frequency target PCell. It is mandatory for intraFreqDAPS capable UE.

#### 4.3.5.x10	ul-TransCancellationDAPS-r16

This field indicates support of cancelling UL transmission to the source PCell. It is mandatory for intraFreqDAPS and interFreqDAPS capable UE.

Editor's note: ul-TransCancellationDAPS is FFS and may need update on RAN1 conclusion.

-----------------------------------------[Next Change]-----------------------------------------------------

-------------unchanged part omitted--------------------

### 4.3.30	Mobility enhancement parameters

#### 4.3.30.1	makeBeforeBreak-r14

This field defines whether the UE supports Make-Before-Break handover and, if the UE supports DC, Make-Before-Break SeNB change, as specified in TS 36.331 [5].

#### 4.3.30.2	rach-Less-r14

This field defines whether the UE supports RACH-less handover and, if the UE supports DC, RACH-less SeNB change, as specified in TS 36.213 [22] and TS 36.331 [5].

#### 4.3.30.x1	cho-r16

This field indicates whether the UE supports conditional handover including execution condition and candidate cell configuration.

#### 4.3.30.x2	cho-Failure-r16

This field indicates whether the UE supports conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover.

#### 4.3.30.x3	cho-FDD-TDD-r16

This field indicates whether the UE supports conditional handover between FDD and TDD cells.

-----------------------------------------[ Changes End]-----------------------------------------------------