| Channel condition | Frequency | NW configures below criteria and when fulfilled | Measurement approach (or with relaxation) | Use case ID |
| --- | --- | --- | --- | --- |
| For inter-frequency / inter-RAT frequency measurement, this part is deduced from RAN4 LS |  |  |  |  |
| When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (According to following proposal 5, not-at-cell-edge criterion is fulfilled always, if it is configured.) | Higher priority carriers | Low mobility (i.e. when NW only configures low mobility, and it is fulfilled) | FFS: If highPriorityMeasRelax is configured: Option 1: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. Option 2: UE can stop high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. Option 3: UE can stop high priority inter-freq/inter-RAT measurements. No minimum measurement time interval applies in that case. | A-1 |
|  |  |  | If highPriorityMeasRelax is not configured: Option 1: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. (i.e. legacy behavior) | A-2 |
|  |  | Not at cell edge (i.e. when NW only configures not-at-cell-edge, and it is fulfilled) | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. Note: this criterion is always fulfilled, if it is configured. (i.e. legacy behavior) | B |
|  |  | When NW configure both while combineRelaxedMeasCondition is not configured, not at cell edge is fulfilled but low mobility is not fulfilled | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. (?) (i.e. legacy behavior) | B-2 |
|  |  | Low mobility and Not at cell edge (i.e. when NW configure both, and both are fulfilled, regardless whether combineRelaxedMeasCondition is configured ) | FFS: Option 1: UE can stop high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. Option 2: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. Option 3: UE can stop high priority inter-freq/inter-RAT measurements. No minimum measurement time interval applies in that case. | C |
|  | Equal/Lower priority carriers | All scenario | UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. (According to current TS 38.304) (i.e. legacy behavior) | D |
| When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Higher priority carriers, Equal/Lower priority carriers | Low mobility including: case 1) when NW only configures low mobility, and it is fulfilled; case 2) When NW configure both while combineRelaxedMeasCondition is not configured, low mobility is fulfilled but not at cell edge is not fulfilled; | UE may choose to perform relaxed measurements with longer intervals. | E |
|  |  | Not at cell edge including: case 1) when NW only configures not at cell edge, and it is fulfilled; case 2) When NW configure both while combineRelaxedMeasCondition is not configured, not at cell edge is fulfilled but low mobility is not fulfilled; | UE may choose to perform relaxed measurements with longer intervals. | F |
|  |  | Low mobility and Not at cell edge (i.e. when NW configure both, and both are fulfilled, regardless whether combineRelaxedMeasCondition is configured ) | FFS for higher priority: Option 1: UE can stop high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. Option 2: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. Option 3: UE can stop high priority inter-freq/inter-RAT measurements. No minimum measurement time interval applies in that case. For equal/lower priority frequency: UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. (Note: for higher/equal/lower carriers) | G |
| For intra-frequency measurement, this part is deduced from RAN2/RAN4 conclusion |  |  |  |  |
| Srxlev > SIntraSearchP and Squal > SIntraSearchQ | N/A | All scenarios | UE may choose not to perform intra-frequency measurements. (According to current TS 38.304) (i.e. legacy behavior) | H |
| Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ | N/A | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | I |
|  | N/A | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | J |
|  | N/A | Low mobility and Not at cell edge | UE can stop intra-frequency measurement with a minimum measurement time interval of 1 hour. | K |