3GPP TSG-RAN WG2 Meeting #110-e	R2-2004426

# Online, 1–12 June 2020

Agenda item:	6.2.2

Source:	Samsung

Title:	Clarification on the variable LBT_COUNTER

Document for:	Discussion and Agreement

# 1	Introduction

From the RAN2 discussion, we understand that the variable LBT_COUNTER is per Serving Cell, which is not clear from the current specification [1]. The contribution discusses how to clarify it.

# 2	Discussion

From the discussion in the past, and according to the procedure, we understand that LBT_COUNTER is per Serving Cell. However, this is not clear from the specification.

Proposal 1:	RAN2 to confirm that the variable LBT_COUNTER is per Serving Cell

If it is agreeable, RAN2 can clarify all the instances of 'LBT_COUNTER' to 'LBT_COUNTER for this Serving Cell', which might be excessive. Hence the following changes would provide clarify while minimizing the changes.

## 5.12 MAC Reset If a reset of the MAC entity is requested by upper layers, the MAC entity shall: … 1> reset all LBT_COUNTERs. … ### 5.21.2 LBT failure detection and recovery procedure The MAC entity may be configured by RRC with a consistent LBT failure recovery procedure. Consistent LBT failure is detected per UL BWP by counting LBT failure indications, for all UL transmissions, from the lower layers to the MAC entity. RRC configures the following parameters in the lbt-FailureRecoveryConfig: - lbt-FailureInstanceMaxCount for the consistent LBT failure detection; - lbt-FailureDetectionTimer for the consistent LBT failure detection; The following UE variable is used for the consistent LBT failure detection procedure: - LBT_COUNTER (per Serving Cell): counter for LBT failure indication which is initially set to 0. …

Proposal 2:	Update the running CR as proposed above.

Note that same thing can be done for BFI_COUNTER for eMIMO, which can be done by eMIMO rapporteur.

# 3	Conclusion

Proposal 1:	RAN2 to confirm that the variable LBT_COUNTER is per Serving Cell

Proposal 2:	Update the running CR as proposed above.

# 4	References

[1]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".