3GPP TSG-RAN WG2 Meeting #110	electronic							       R2-2004744

E-Meeting, 1th Jun – 12th Jun, 2020

Source: 	vivo

Title:  	Discussion on manual CAG selection

Agenda Item:	6.18.3

Document for:	Discussion and Decision

Introduction

Based on the received CT1 LS R2-2004178, we will discusses the issue on interaction of NAS and AS to make manual CAG selection more clearly.

Discussion

  - CT1 LS R2-2004178

In RAN2#109bis-e Meeting, a CT1 LS R2-2004178 is received. In this LS, CT1 ask RAN2 to provide any feedback on the Editor’s note “confirm whether there will be modification in the radio interface in order to enable a CAG cell's broadcasting that the PLMN allows a user to manually select a CAG-ID supported by the CAG cell”.

Along with this LS, there is an agreed CR C1-282062, which shows the MS will present the CAG ID to the user under the following case:

In i to v, if the MS supports CAG and is provisioned with a non-empty "CAG information list", for each PLMN/access technology combination of NG-RAN access technology, the MS shall present to the user: a) the PLMN/access technology combination and a list of CAG-IDs composed of one or more CAG-IDs such that for each CAG-ID: 1) there is an available CAG cell which broadcasts the CAG-ID for the PLMN; and 2) the following is true: i) there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" of the entry; or ii) the available CAG cell broadcasting the CAG-ID for the PLMN also broadcasts that the PLMN allows a user to manually select the CAG-ID.

RAN2 is working on introducing a indication to support CT1’s agreement. Even though RAN2 is still unclear that whether the indication that the PLMN allows a user to manually select a CAG-ID supported by the CAG cell but outside the UE’s allowed CAG list is per PLMN or per CAG ID. Based on the above description, it is certain that the indication that the PLMN allows a user to manually select the CAG ID shall be provided to assisting NAS to determine whether a CAG ID shall be present to the user. As this indication is only useful for manual CAG selection, there is no need to provide to NAS automatically. Same with HRNN, upon request by NAS, AS provide the indication of allowing manual CAG selection together with available CAG ID(s) and their HRNN (if broadcast) and PLMN(s) to the NAS.

Proposal: For manual CAG selection, upon request by NAS, the AS shall report the indication of allowing manual CAG selection to NAS.

Conclusions

Based on the analysis given above, we have the following proposal:

Proposal: For manual CAG selection, upon request by NAS, the AS shall report the indication of allowing manual CAG selection to NAS.

Reference

- R2-2004178, LS on manual CAG selection.