3GPP TSG-RAN WG2 Meeting #110-e	R2-2004979

# Electronic, 1st June – 12th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0323 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of IAB capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the support for IAB capabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Included RAN2-specific IAB capabilities, and RAN1-specific capabilities agreed in RAN1#100-bis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not be able to indicate support for IAB capabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.xx (new), 4.2.xx.1 (new), 4.2.xx.2 (new), 4.2.xx.3 (new), 4.2.xx.4 (new), 4.2.xx.5 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR 1642 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.2.xx	IAB-MT parameters

The following clauses define the IAB-MT radio access capability parameters and mandatory features. All radio access capability parameters in this specification, not listed in clause 4.2.xx.1 are optional to the IAB-MT.

#### 4.2.xx.1	Mandatory features without radio access capability parameters

| Features | Feature group | Components |
| --- | --- | --- |
| 1. PDCP | Basic PDCP procedures | 1) (de)Ciphering on SRB 2) Integrity protection on SRB 3) Timer based SDU discard 4) Re-ordering and in-order delivery 6) Duplicate discarding |
| 2. RLC | Basic RLC procedures | 1) RLC TM 2) RLC AM with 18bits SN 3) SDU discard |
|  | NR RLC SN size for SRB | NR RLC SN size for SRB |
| 3. MAC | Basic MAC procedures | 1) RA procedure on PCell 2) IAB-MT initiated RA procedure (including for beam recovery purpose) 3) NW initiated RA procedure (i.e. based on PDCCH) 4) Support of ssb-Threshold and association between preamble/PRACH occasion and SSB 5) Preamble grouping 6) UL single TA maintenance 7) HARQ operation for DL and UL 8) LCH prioritization 9) Prioritized bit rate 10) Multiplexing 11) SR with single SR configuration 12) BSR 13) PHR 14) 8bits and 16bits L field |
| 4. RRC | RRC buffer size | Maximum overall RRC configuration size |
|  | RRC processing time | 1) RRC connection establishment 2) RRC connection reconfiguration without SCell addition/release 3) RRC connection re-establishment. 4) RRC connection reconfiguration with sync procedure 5) RRC connection reconfiguration with SCell addition/release 6) Initial security activation 7) Counter check 8) UE capability transfer |

#### 4.2.xx.2	BAP parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF |
| --- | --- | --- | --- |
| bap-BasicProcedures Indicates whether the IAB-MT supports the BAP layer and its basic procedures such as routing, bearer mapping, IP assignment over RRC. It shall be set to supported. | IAB-MT | Yes | No |
| flowControl-BH-RLC-Channel-Based Indicates whether IAB-MT supports hop-by-hop flow control based on backhaul RLC channel. | IAB-MT | No | No |
| flowControl-Routing-ID-Based Indicates whether IAB-MT supports hop-by-hop flow control based on routing ID. | IAB-MT | No | No |
| rlf-Handling Indicates whether IAB-MT supports reception of BAP Control messages for RLF handling. | IAB-MT | No | No |

#### 4.2.xx.3	MAC parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| preEmptiveBSR Indicates whether the IAB-MT supports transmission of pre-emptive BSR. | IAB-MT | No | No | No |
| lcid-Extension Indicates whether the IAB-MT supports the 2-byte LCID-extension | IAB-MT | No | No | No |

#### 4.2.xx.4		Physical layer parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| dci-Format2-5 Indicates whether the IAB-MT supports monitoring of DCI Format 2_5 scrambled by AI-RNTI for indication of soft resource availability to an IAB node. | IAB-MT | No | No | No |
| discoveryAndMeas Indicates whether the IAB-MT supports measurement timing configuration for SS corresponding to IAB-MT. | IAB-MT | No | No | No |
| dynamicUlFlexibleDlSlotFormat Indicates whether the MT supports dynamic indication of TDD UL/DL configuration. | IAB-MT | No | No | No |
| guardSymbols Indicates whether the IAB-MT supports reception of Provided Guard Symbol MAC CE and transmission of Desired Guard Symbol MAC CE see TS 38.321 [8]. | IAB-MT | No | No | No |
| ota-TimingAlignment Indicates whether the IAB-MT supports reception of Timing Delta MAC CE see TS 38.321 [8]. | IAB-MT | No | No | No |
| rach-MT Indicates whether the IAB-MT supports random access parameters for IAB. | IAB-MT | No | No | No |
| semiStaticUlFlexibleDlSlotFormat Indicates whether the MT supports semistatic TDD UL/DL configuration. | IAB-MT | No | No | No |

#### 4.2.xx.5		Inter-RAT parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| f1apOverLTE Indicates whether the IAB-MT supports F1AP signalling over LTE leg for EN-DC. | IAB-MT | No | No | No |

END OF CHANGES