3GPP TSG-RAN2 Meeting#110-e                                R2-2005416

Online, 1st June – 12nd June 2020

Agenda Item:	6.12.4

Source: 	Samsung

Title: 	Correction on MDT Configuration [S959]

Document for:	Discussion and decision

Introduction

In RAN2#107bis-e, RAN2 made the following agreement:

Introduce the logged MDT measurement frequencies and cell IDs (i.e. PCI) for neighbour cell measurement in the area configuration. UE only need to log and report measurement results for the configured frequencies if the results available.

From the agreement, RAN2 made a new IE, InterFreqTargetList for MDT configuration.

This contribution will discuss if the IE needs to be included into IE AreaConfiguration.

Discussion

From the relevant agreement, the intended behaviour is as follows:

- AreaConfig: If configured, the UE performs measurement logging only if the serving cell is part of AreaConfig (i.e. outside this area, UE performs no logging)

- InterFreqTargetList: If configured, the IE further restricts the neighbouring cells for which UE performs measurement logging. I.e. the UE shall perform logging

  - Of frequencies that are indicated both in SIB4 and InterFreqTargetList

  - Of cells on the concerned frequency indicated by the cellist field of InterFreqTargetList

The intention of the two IE AreaConfig and InterFreqTargetList is quite different, i.e. the IE AreaConfig is used to indicate the area where UE shall perform MDT operation, but the IE InterFreqTargetList is used to indicate the neighbouring frequencies to be logged.

In other words, the IE InterFreqTargetList is not information related to any area. Accordingly, it could result in confusion when IE InterFreqTargetList is included in the IE AreaConfiguration.

Proposal: IE InterFreqTargetList moves out of IE AreaConfiguration.

Suggested TP

The content of the IE InterFreqTargetList is still under discussion while considering to remove the frequency band list. Therefore, please refer to our intention from the following TP.

–    LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16           OPTIONAL,  --Need R

interFreqTargetList-r16                        SEQUENCE(SIZE (1..maxFreq)) OF InterFreqTargetInfo-r16             OPTIONAL

plmn-IdentityList-r16                       PLMN-IdentityList2-r16          OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need R

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need R

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need R

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16

}

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

InterFreqTargetInfo-r16  ::=   SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

cellList                         SEQUENCE (SIZE (1..32)) OF  PhysCellId  OPTIONAL

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

–    AreaConfiguration

The AreaConfiguration indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in plmn-IdentityList stored in VarLogMeasReport.

AreaConfiguration information element

-- ASN1START

-- TAG-AREACONFIGURATION-START

AreaConfiguration-r16 ::=        SEQUENCE {

areaConfig-r16         AreaConfig-r16,

interFreqTargetList-r16       InterFreqTargetList-r16              OPTIONAL

}

AreaConfig-r16 ::=     CHOICE {

cellGlobalIdList-r16             CellGlobalIdList-r16,

trackingAreaCodeList-r16         TrackingAreaCodeList-r16,

trackingAreaIdentityList-r16     TrackingAreaIdentityList-r16

}

InterFreqTargetList-r16 ::=   SEQUENCE {

dl-CarrierFreq                    ARFCN-ValueNR,

frequencyBandList                MultiFrequencyBandListNR,

cellList                         SEQUENCE (SIZE (1..32)) OF  PhysCellId  OPTIONAL

}

CellGlobalIdList-r16 ::=         SEQUENCE (SIZE (1..32)) OF CGI-Info-Logging-r16

TrackingAreaCodeList-r16 ::=     SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

TrackingAreaIdentityList-r16 ::= SEQUENCE (SIZE (1..8)) OF TrackingAreaIdentity-r16

TrackingAreaIdentity-r16 ::=     SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

trackingAreaCode-r16             TrackingAreaCode

}

-- TAG-AREACONFIGURATION-STOP

-- ASN1STOP

Conclusion

It is suggested that

Proposal: IE InterFreqTargetList moves out of IE AreaConfiguration.