3GPP TSG-RAN WG2 Meeting #110 electronic			     R2-2005664

Online, 1st – 12th June 2020

Agenda Item:	6.1.4 (NR_IAB-Core)

Source: 	LG Electronics Inc.

Title:         	Consideration on reserved value in two byte eLCID space

Document for: 	Discussion and Decision

Introduction

In the last meeting, RAN2 discussed whether to keep the reserved values in the two-byte eLCID space. However, this is related to RAN3 decision and eventually it was decided to wait RAN3 conclusion for this issue as shown below. This contribution discusses this point based on RAN3 agreements.

R2-2003829 Summary of IAB User Plane open issues and corrections Samsung Electronics GmbH report Late DISCUSSION Proposal 2: Reserved values in the two-byte eLCID space are kept. - LG would like to use the reserved bits for LCID range. - Nokia would like that as well, and think that the reserved bits are not used. - Samsung think that 14bits are acc to R3. QC think R3 re-discusses, and thikn that we discussed recently 20 bit flow label, and we should have 16 bits. Nokia think it is in any case better to have reserved values rather than reserved bits. ZTE also support 16bits. - Chair wonder if it matters whether 14-16bits. Samsung think from use case prespective it doesn’t matter. - Ericsson think we can align w R3, we should respect R3 decision. - Huawei thikn we should stick to previous agreement. - QC indicates that R3 decision was based on a mistake by the WI rapporteur. - CHAIR: OK, we wait for R3 to rediscuss then, if there was no reason to change the earlier decided 16bits should be applied. Come bck later.

Discussion

In the discussion, the main reason by the opponents are to keep aligning with RAN3 agreement, i.e., 14bits for BH RLC channel IDs and this reserved value will give more flexibility for future proof. However, considering that reserved LCID values by one byte LCID extension is introduced in Rel-16 and the original intention of this reserved LCID value is just to prepare lack of LCID values for future use, we think that 256 reserved values by one-byte LCID extension are quite sufficient for future proof and reserved value by two-byte LCID extension may not be used. Furthermore, as shown in below RAN3 agreement, RAN3 finally reverted there previous agreement for maximum number of BH RLC channel and decided to increase up to 65536, i.e., 216.

OTHER REMAINING ISSUES, CLEANUPS

Use the following parameters: maxnoofBHRLCChannels: 65536 BH RLC CH ID: BIT STRING(16) maxnoofChildIABNodes: 1024 maxnoofServedCellsIAB: 512

As explained above, main arguments by opponents are gone and there would be no more clear reasons to still keep the reserved values in the two-byte eLCID space. Therefore, it is better to remove the reserved LCID values for two bytes LCID extension and use this reserved value for extended logical channel ID. The text proposal is suggested based on the IAB MAC rapporteur’s CR in the Annex.

Proposal 1. The reserved LCID values for two bytes LCID extension is removed and used for extended logical channel IDs.

Proposal

Based on the above discussions, we present the following proposals and the TP in the Annex:

Proposal 1. The reserved LCID values for two bytes LCID extension is removed and used for extended logical channel IDs.

- Annex

## 6.2	Formats and parameters

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits. If the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 1:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one–octet eLCID field) |
| 35 | Reserved |
| 36 | SP Positioning SRS Activation/Deactivation |
| 37 | Duplication RLC Activation/Deactivation |
| 38 | Absolute Timing Advance Command |
| 39 | CC list-based SRS Activation/Deactivation |
| 40 | PUSCH Pathloss Reference RS Activation/Deactivation |
| 41 | SRS Pathloss Reference RS Activation/Deactivation |
| 42 | AP SRS spatial relation Indication |
| 43 | Enhanced PUCCH spatial relation Activation/Deactivation |
| 44 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 45–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 253 | 64 to 317 | reserved |
| 254 | 318 | Number of Provided Guard Symbols |
| 255 | 319 | Timing Delta |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33 | Extended logical channel ID field (two–octet eLCID field) |
| 34 | Extended logical channel ID field (one–octet eLCID field) |
| 35–39 | Reserved |
| 40 | Sidelink Configured Grant Confirmation |
| 41 | Truncated Sidelink BSR |
| 42 | Sidelink BSR |
| 43 | Multiple Entry Configured Grant Confirmation |
| 44 | LBT failure (four octets) |
| 45 | LBT failure (one octet) |
| 46 | SCell BFR (four octets Ci) |
| 47 | SCell BFR (one octet Ci) |
| 48 | Truncated SCell BFR (four octets Ci) |
| 49 | Truncated SCell BFR (one octet Ci) |
| 50–51 | Reserved |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Table 6.2.1-2a Values of two-octet eLCID for UL-SCH

| Codepoint/IIndex | LCID values |
| --- | --- |
| 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-2b Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 253 | 64 to 317 | reserved |
| 254 | 318 | Number of Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |

NOTE 2:	For the two-octet eLCID space, the 16-bit codepoint 000…00 (all zeros) corresponds to the index value of 320, while the 16-bit codepoint 111…11 (all ones) corresponds to the index value of 216 + 319.