3GPP TSG-RAN WG2 #111e	Tdoc R2-2007660

Electronic meeting, August 17th – 28th 2020

Agenda Item:	8.4.2

Source:	Ericsson

Title:	Scenarios of topology adaptation for IAB network

Document for:	Discussion, Decision

# 1	Introduction

This paper provides insights on what challenges need to be solved for topology adaptation in IAB Rel-17 WI.

# 2	Discussion

In our view, topology adaptation and mobility are two different concepts, where IAB node mobility is excluded from Rel-17 WI. There are at least two main differences between mobility and topology adaptation. Firstly, in mobility, the reconfiguration procedure for handover is time-sensitive, which means that if the procedure is not completed fast, the UE could lose the connection to the network. Conversely, for topology adaptation, reconfigurations are not time-critical. Topology adaptation should be commonly seen/used as a mechanism for keeping the network stable by monitoring the network status and then adjusting the topology gradually in a non-time-critical manner. Short-term traffic peaks are not meant to trigger topology adaptations – these occurrences are rather temporary and can be addressed by congestion mitigation mechanisms, for example.

The second difference between topology adaptation and mobility is that, in mobility, if an inter-donor CU migration is needed, the connection to the first donor CU is typically released after the connection to the second node is established. For topology adaptation, since the goal is to move traffic, the result of a topology adaptation is that a only fraction of the traffic served by a migrating IAB node needs to be moved, for example, at peak hours. This implies that a part of the traffic served by such a migrating IAB node will still be carried over the first donor and another part of the traffic over the second donor. Since the network nodes do not move, the reconfigurations will always affect a very limited set of nodes.

Hence, we believe that the inter-CU migration should be mainly dictated by load balancing purposes, in which the serving CU may decide to offload some traffic to other less loaded cells served by a different CU.

Unlike mobility, inter-CU migration (i.e. topology adaptation) is not time-critical.

Topology adaptation will mainly be used for load balancing, which means only a fraction of the traffic will be carried by another/new donor-node.

Consequently, the connection to the first CU will typically be maintained even after connected to another CU.

However, even if in most common cases only a fraction of the overall traffic served by a migrating IAB node needs to be moved, it is important, from a network design point of view, that topology adaptation in IAB networks does not result in a considerable amount of signalling and reconfigurations as this may lead to traffic interruptions, buffering, delays, retransmissions, and, in extreme cases, packet losses. Thus, inter-CU migration (for load balancing) should incur minimum possible signalling or, at the very least, it should be possible to distribute the reconfiguration signalling load over a relatively long period.

Inter-CU migration solutions should be transparent as far as possible and should not incur massive simultaneous reconfigurations.

Topology adaptation will also handle error cases such as radio link failures. For wide-area IAB networks, good and robust network planning and deployment should be considered as the primary mechanism to avoid radio link failures. Thus, radio link failures are expected to be very rare events. However, it might not be the same situation for other types of IAB-nodes that are not under operator control and/or deployed in an uncoordinated/unplanned manner. For such IAB-nodes, failure cases may be more common, and solutions could be customized exclusively for them. However, migration as a consequence of RLF may be subject to less signalling overhead at least over the air interface.

RAN2 should consider load balancing of some of the traffic served by an IAB node as the primary use case for the Rel-17 inter-CU migration

Inter-CU migration solutions should avoid massive reconfigurations and minimize the signalling load.

## 2.1	The approaches for inter-CU topology adaptation

#### Approach 1: Inter-donor migration – the “classic” case

In this case, an IAB node (herein referred to as the top-level node) and a number of its directly and indirectly served IAB nodes and UEs are “completely” handed over to a different donor gNB or to a parent IAB node served by a different donor gNB. This means that 100% of the traffic carried over that node is moved to another donor. The “completely” above means that all the RRC/F1AP connections with the old donor are removed in conjunction with the migration towards the target donor CU, similar to the legacy handover for UEs. We believe that this scenario might not be very likely to occur in practice, since load balancing typically operates on a device level, to properly balance the load in the various cells. Therefore a scenario in which a huge number of devices are suddenly subject to load balancing operations from the network less likely to occur. This scenario is captured in Figure 1.

A “classic handover” on a top-level node and its descendant nodes and UEs means that 100% of the traffic carried over that node is moved to another donor.

Figure 1: Example of the classic case for inter-CU migration – all RRC and F1 connections are migrated to the target donor

This case is more suitable for failure cases or mobility use cases than for a load balancing scenario, since the mobility/RLF inherently prohibits long-term simultaneous connections to the source and target. Therefore, as argued above, this is not a reasonable approach for network-controlled topology adaptation, which is triggered by decisions based on monitoring over a certain period of time, where the goal is to keep the network stable and traffic well balanced across cells.

This scenario also imposes several challenges. For example, since the gNB-donor is changed, all the nodes in the network including IABs and UEs will have to be reconfigured:

First, the security keys will necessarily need to be changed.

Second, the target donor gNB may support different features than the source gNB.

This may result in that the network, i.e. the target CU, will need to:

For each migrating UE and IAB node, assess their capabilities and the corresponding current configurations

Perform access control for all these IAB nodes and UEs

Perform reconfiguration of the accepted IAB nodes and UEs, if applicable.

Even if the network performs some parallel processes and reconfigurations, it is not realistic to expect that the network will parallelize all computational processes and reconfigure all the IABs and UEs at the same time. Processing and reconfigurations will take time, especially if there are hundreds or thousands of devices under the IAB nodes and its child nodes. Besides, this will incur huge signalling overhead within the network and between the network and the UEs/IABs.

It will be extremely challenging for a network to execute, in parallel, the handover and reconfigurations of many migrating IAB nodes and served UEs.

Since this is not the typical topology adaptation use case, the “complete” group handover case should be de-prioritized among the approaches for inter-donor migration. Nevertheless, if RAN3 decides to support the “classic” approach, the solutions should focus on ways to limit the impact to as few nodes as possible and to avoid a massive number of reconfigurations.

RAN2 to de-prioritize the “classic” approach for inter-donor migration, where the migrating IAB nodes and UEs are completely handed over to the target concurrently, and the connections to the source are torn down.

#### Approach 2: Gradual and partial inter-donor migration

As argued above, in a controlled topology adaptation, a part of the load may be offloaded gradually to a second node. This implies that a connection to a second node is created and then, reconfigurations are performed gradually, in a controlled manner, where part of the traffic traversing the top-level node is offloaded to the new donor. This means that for the top-level IAB node, the connections to both the source and the target donor gNB are maintained. Eventually, the connection of the top-level IAB node in either the source or the target donor gNB could be released. Nevertheless, considering that topology adaption will be mainly for load balancing, it is likely that, when the load in the network becomes normal again, the connections are moved back to the first/source node, i.e. the target donor gNB connections would be released.

This approach has at least two flavours, as shown in Figure 2 and Figure 3. In Figure 2, Donor 1 requests the Donor 2 to establish a connection towards top-level node.  A new connection is then established towards such top-level node (in addition to the existing one towards Donor 1), and, when this new connection is established, the network can reconfigure the UEs or other IAB nodes served by such IAB node. In this way, the load is gradually moved towards Donor 2. Once the corresponding nodes are reconfigured and transferred completely to Donor 2, then the connections from these UEs and nodes towards the first donor can be removed. This implies that nodes will have to support, for a period of time, two protocol stacks.

Figure 2: An example of inter-CU migration – load gradually moved to target CU.

Figure 3: An example of inter-CU migration: IAB-node  connected to two different donor gNB and target CU acting as a proxy for some traffic.

Another option is shown in Figure 3: Donor 1 requests the Donor 2 to establish a connection towards top-level IABa node in a “tunnel” mode, where Donor 2 would act as a proxy for traffic between some of the migrated nodes and UEs and Donor 1. This means that the control remains in the CU_1.  In other words, a new tunnel connection is then established towards the top-level IAB node and part of the traffic can then be routed through CU_2 which allows offloading the Uu interface of the top-level IAB node in the CU_1.  Part of the traffic would then be carried over CU_2 and tunnelled to CU_1 and the other part of the traffic would be directly carried over Uu in CU_1.

With this type of configuration, the MT and UE context (e.g. PDCP) remain in the CU_1, which could potentially minimize reconfigurations towards UEs and other IAB nodes. When CU_1 can handle the additional load, the traffic could again be re-routed directly to CU_1.

In case, the load carried via CU_1 keeps growing, it could be possible to reconfigure as suggested in Figure 2, so that the load would, then, be moved gradually towards Donor 2.

The discussion on which type of inter-CU migration scenario to consider depends on RAN3 decisions. However, their choice has obvious consequences in the RAN2.

RAN2 to study inter-donor migration mechanisms the connection of top-level node to the source donor is maintained.

RAN2 to study inter-donor migration mechanisms where the IAB nodes and UEs that need to be migrated are gradually migrated to the target CU, in order to avoid a signalling storm.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Unlike mobility, inter-CU migration (i.e. topology adaptation) is not time-critical.

Observation 2	Topology adaptation will mainly be used for load balancing, which means only a fraction of the traffic will be carried by another/new donor-node.

Observation 3	Consequently, the connection to the first CU will typically be maintained even after connected to another CU.

Observation 4	Inter-CU migration solutions should be transparent as far as possible and should not incur massive simultaneous reconfigurations.

Observation 5	A “classic handover” on a top-level node and its descendant nodes and UEs means that 100% of the traffic carried over that node is moved to another donor.

Observation 6	It will be extremely challenging for a network to execute, in parallel, the handover and reconfigurations of many migrating IAB nodes and served UEs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should consider load balancing of some of the traffic served by an IAB node as the primary use case for the Rel-17 inter-CU migration

Proposal 2	Inter-CU migration solutions should avoid massive reconfigurations and minimize the signalling load.

Proposal 3	RAN2 to de-prioritize the “classic” approach for inter-donor migration, where the migrating IAB nodes and UEs are completely handed over to the target concurrently, and the connections to the source are torn down.

Proposal 4	RAN2 to study inter-donor migration mechanisms the connection of top-level node to the source donor is maintained.

Proposal 5	RAN2 to study inter-donor migration mechanisms where the IAB nodes and UEs that need to be migrated are gradually migrated to the target CU, in order to avoid a signalling storm.