3GPP TSG-RAN2 Meeting #111	R2-2008346

# E-meeting, August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1777 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for V2X UE capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-07-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on updated RAN1/4 features included in the latest received RAN1/4 LS (i.e., R2-2008441, R2-2008442, R2-2008443, R2-2008459), and the following agreement from RAN2#111-E Agreements on UE capabilities: 1: Keep the agreed RAN2 capability on multiple configured grants and also keep 15-2/5-2 component-1 as maximum value. 2: Do not capture the “Note: the UE supports up max(B, C) as the total number of sidelink HARQ processes across both Mode 1 and Mode 2”. 3: For PC5 RLC RTT will adopt the table raised by Huawei in R2-2004402. N=5 is set as working assumption. Unless big problem is raised next meeting, WA will be agreed. 4: A parameter to indicate whether UE supports PC5 BC when NR Uu BC is configured as NR-DC is not supported in this release. 5a: Merge the sidelink BC lists in UE-NR-Capability, i.e., pure LTE sidelink BC list, pure NR sidelink BC list and mixed LTE-NR sidelink BC list, into one. 5b: Merge the sidelink BC lists in UE-EUTRA-Capability, i.e., pure NR sidelink BC list and mixed LTE-NR sidelink BC list, into one. 6a: Capture in 38.306 that “A fallback band combination resulting from the reported sidelink band combination shall be supported by the UE.” 6b: Capture in 38.306 that “For each band combination reported in the list, a combination, which consists of a fallback band combination resulting from the associated Uu band combination and a fallback band combination resulting from the associated sidelink band combination, shall be supported by the UE.” 7: For the component with note “Component X is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2”, to separately capture the component X explicitly in ASN.1 within the FG. 8: For 15-18/5-14 and 15-19/5-15, no capability report to network. 9: Report the supported channel bandwidth for each sidelink band. To add V2X UE capability for LTE-Uu controlling NR-PC5 scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Merge the sidelink BC lists in UE-EUTRA-Capability, i.e., pure NR sidelink BC list and mixed LTE-NR sidelink BC list, into one. - Add the optional capability without signalingfor short-term time-scale TDM for in-device coexistence Impact analyses Impacted functionality UE capability report signalling Inter-operablity There is no inter-operability issue between the NW and the UE, as the change only concerns capability aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2#111-E agreement on V2X UE capability for LTE-Uu controlling NR-PC5 scenario is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.21, 6.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 4349 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

4.3.21.32	v2x-SupportedBandCombinationListNR-r16

This field indicates the band combination(s) on which the UE supports NR sidelink communication-only, as defined in TS 23.287 [42] and specified in TS 38.331 [35], or simultaneously with V2X sidelink communication, as defined in TS 23.285 [29] and specified in TS 36.331 [5].

Next Change

6.13	Sidelink features

6.13.1	Sidelink Relay UE operation

It is optional for UE to support sidelink relay UE operation as specified in TS 36.331 [5].

6.13.2	Sidelink Remote UE operation

It is optional for UE to support sidelink remote UE operation as specified in TS 36.331 [5].

6.13.3	Sidelink discovery gap

It is optional for UE to support sidelink discovery gaps as specified in TS 36.331 [5].

6.13.4	Enhanced sidelink resource selection

It is optional for limited TX capability UE to support enhanced sidelink resource selection with carrier aggregation as specified in clause 5.14.1.1 of TS 36.321 [4].

6.13.X	Short-term time-scale TDM for in-device coexistence

It is optional for UE to support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception. This feature is only applicable if the UE supports at least one of sl-Reception-r16, sl-TransmissionMode1-r16 and sl-TransmissionMode2-r16 as specified in TS 38.331 [35], and if UE supports LTE V2X sidelink in the band combination.

End of Change