3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2008674

# Elbonia, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0294 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of CAPC for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CAPC description would benefit from having the following aspects fixed: - Existing text may be difficult to understand and is not compatible with the rest of the specification; - The meaning of “traffic class” is not clear; - Stage 3 details on the cases where LBT is performed are given with a reference to RAN1 specification; - MSG3/MSGA handling is not aligned with RAN1 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are incorporated: - Statements are reordered to improve legibility; - Regular QoS terminology is used (Qos characteristics, QoS flow…); - Only the reference to RAN1 specifications is kept; - MSG3/MSGA handling is aligned with RAN1 specifications. Impact analysis Impacted functionality: CAPC Inter-operability: - If the network is implemented according to the CR and the UE is not, RACH completion may be delayed due to LBT failure. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible discrepancies across implementation leading to wrong CAPC selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | [MCC]: Clauses affected added in rev1. |  |  |  |  |  |  |  |  |

### 5.6.2	Channel Access Priority Classes

The Channel Access Priority Classes (CAPC) of radio bearers and MAC CEs are either fixed or configurable:

-	Fixed to the lowest priority for the padding BSR and recommended bit rate MAC CEs;

-	Fixed to the highest priority for SRB0, SRB1, SRB3 and other MAC CEs;

-	Configured by the gNB for SRB2 and DRB.

When choosing the CAPC of a DRB, the gNB takes into account the 5QIs of all the QoS flows multiplexed in that DRB while considering fairness between different traffic types and transmissions. Table 5.6.2-1 below shows which CAPC should be used for which standardized 5QIs i.e. which CAPC to use for a given QoS flow.

NOTE: 	A QoS flow corresponding to a non-standardized 5QI (i.e. operator specific 5QI) should use the CAPC of the standardized 5QI which best matches the QoS characteristics of the non-standardized 5QI.

# Table 5.6.2-1: Mapping between Channel Access Priority Classes and 5QI

| CAPC | 5QI |  |
| --- | --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |  |
| 2 | 2, 7, 71 |  |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |  |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

When performing Type 1 LBT for the transmission of an uplink TB (see TS 37.213 [37], clause 4.2.1.1) and when the CAPC is not indicated in the DCI, the UE shall select the CAPC as follows:

-	If only MAC CE(s) are included in the TB, the highest priority CAPC of those MAC CE(s) is used; or

-	If CCCH SDU(s) are included in the TB, the highest priority CAPC is used; or

-	If DCCH SDU(s) are included in the TB, the highest priority CAPC of the DCCH(s) is used; or

-	The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used otherwise.