3GPP TSG-RAN WG2 Meeting#111-e	Draft_R2-2008192

Online, 17th - 28th August 2020

Agenda Item:	8.12.2.2

Source:	Huawei

Title:	[Draft] Summary of offline 110 - Identification and access restriction

Document for:	Discussion and Decision

# Introduction

This document is for the following offline discussion on identification and access restriction for REDCAP UEs:

[AT111e][110][REDCAP] Identification and access restriction (Huawei)

Scope: Discuss the proposals in R2-2007345, R2-2006661, R2-2006786 and R2-2007493. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): 				Monday 2020-08-24 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2008192): 	Tuesday 2020-08-25 02:00 UTC

The following contributions are summarised in this document:

R2-2006661	Coexistence between legacy UEs and RedCap UEs		Samsung

R2-2006786	Discussion on RedCap UE’s identification and access control	OPPO

R2-2007345	Identification and access restriction of REDCAP UE		Huawei, HiSilicon

R2-2007493	On UE identification and access restrictions			MediaTek Inc.

# Discussion

According to the proposals in above contributions, the following issues are summarised:

- Camping criteria for REDCAP UE

- When/How to identify REDCAP UE

- UAC for REDCAP UE

- How to ensure REDCAP UEs for intended use cases

## Camping criteria

Regarding whether the REDCAP UE is allowed to camp on a cell, corresponding proposals in above contributions are listed as follows:

| Tdoc number | Company name | Proposals |
| --- | --- | --- |
| R2-2006661 | Samsung | Proposal 1: RAN2 to confirm that a gNB that supports RedCap UEs also supports legacy UEs simultaneously. Proposal 2: The legacy MIB signalling is re-used to accommodate legacy UEs. Proposal 3: The legacy UE determines whether it can access the cell based on the legacy values of controlResourceSetZero. Proposal 4: If a RedCap UE does not support the bandwidth from the controlResourceSetZero of a cell, it considers the cell as barred. Proposal 5: The field intraFreqReselection is reused to determine whether a RedCap UE performs cell selection/reselection to intra-frequency cells if the cell is barred. |
| R2-2006786 | OPPO | Proposal 1 A separate cellBarred indication can be added in MIB/SIB1 for RedCap UEs, to differentiate from cellBarred indication for normal UEs. |
| R2-2007345 | Huawei, HiSilicon | Proposal 1: Allow a REDCAP UE to camp on a cell with larger initial DL/UL BWP than supported by REDCAP UE to avoid negative impact on legacy UEs. Proposal 4: Consider to indicate whether REDCAP UEs are allowed to camp on in MIB/SIB1 to avoid REDCAP UE camping in a legacy network and allow network to bar all REDCAP UE. |
| R2-2007493 | MediaTek Inc. | Proposal 1: A RedCap UE only camps on a cell that indicates support of RedCap operation Proposal 2: Support of RedCap operation in a cell is broadcasted by the network Proposal 3: RAN2 to discuss further if RedCap support is indicated per cell or per frequency by the network. |

Above proposals are summarised as 3 camping criterions and the use of intraFreqReselection if the UE cannot camp on.

Criterion 1: The bandwidth of CORESET#0

This criterion corresponds to proposals 1 to 4 in R2-2006661.

According to the following RAN1 agreements, for FR2, it is possible that the maximum bandwidth of REDCAP UE is smaller than the bandwidth of CORESET#0 indicated by MIB.

- For FR1, study at least 20MHz maximum UE bandwidth at least for initial access - Other bandwidths FFS - For FR2, study 50MHz and 100 MHz maximum UE bandwidth at least for initial access - Other bandwidths FFS

In this case, the UE should consider the cell as barred and do not camp on the cell.

Question 1. If the maximum bandwidth of REDCAP UE is smaller than the bandwidth of CORESET#0 indicated by MIB, do you agree that the UE should consider the cell as barred and do not camp on the cell?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | That’s how it works in legacy |

Criterion: REDCAP UEs are allowed to access the cell

This criterion corresponds to proposal 1 in R2-2006786, proposal 4 in R2-2007345 and proposals 1, 2 in R2-2007493.

The existing NR cell works based on the assumptions that the 100M minimum bandwidth and (2 or 4) RX antennas are mandatory for the UE. For REDCAP UEs, above requirements for legacy NR devices will be relaxed. Thus, on one hand, a REDCAP UE should not camp on a legacy NR cell which does not support REDCAP operation. On another hand, it should be possible for the network to bar the access of REDCAP UE.

Based on above, an indication is needed in system information to indicate whether a REDCAP UE is allowed to camp on the cell.

Question 2. Do you agree that an indication is needed in system information to indicate whether a REDCAP UE is allowed to camp on the cell?

| Company name | Yes/No? | Comments |
| --- | --- | --- |
| Qualcomm | Yes | gNB may want to dynamically control whether RedCap UEs can access it or not, e.g. base on its loading etc. So we think such an indication in system information is necessary. |

The following options for the indication is mentioned in the contributions:

- MIB

- SIB1

Thus, if the answer to Question 2 is Yes, please indicate which option do you prefer to introduce the indication:

Question 2a. If the answer to Question 2 is Yes, which system information should be used to indicate whether a REDCAP UE is allowed to camp on? MIB/SIB1/other?

| Company name | MIB/SIB1/other? | Comments |
| --- | --- | --- |
| Qualcomm | SIB1 | SIB1 is where access barring/control information is signalled and hence the right place for indicating whether access by RedCap UE is allowed or not. |

Criterion 3: The bandwidth of initial UL/DL BWP configured by SIB1

This Criterion corresponds to proposal 1 in R2-2007345.

Initial UL/DL BWP can be configured by SIB1, which may have larger bandwidth compared with the maximum bandwidth supported by REDCAP UEs. Whether a REDCAP UE can camp on the cell in this case needs to be discussed.

If a REDCAP UE is not allowed to camp on the cell with larger initial UL/DL BWP than supported by the UE, there will be restriction on the network configuration to support REDCAP UE, i.e. in case REDCAP UEs are supported in the cell, the network needs to guarantee that the configured initial UL/DL BWP is smaller or equal to the bandwidth supported by REDCAP UEs.

If a REDCAP UE is allowed to camp on in this case, the REDCAP UE needs to be identified at early stage as the gNB needs to schedule Msg3/Msg5 transmission properly.

Question 3. Do you agree with above analysis and whether a REDCAP UE should be allowed to camp on a cell with larger initial DL/UL BWP than supported by the UE?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes/no | As long as a RedCap UE can support the coreset #0 in the initial DL BWP, it should be allowed to camp on it. Network implementation can ensure all PDSCH during paging or initial access is transmitted within the same frequency locations. But we are not sure how UL may work out if a cell has wider initial UL BWP than the one supported by UE. |

The use of intraFreqReselection

The use of intraFreqReselection is discussed by proposal 5 in R2-2006661 and proposal 4 in R2-2007493.

In NR, if the UE considers the cell is barred and cannot camp on the cell, the field intraFreqReselection in MIB can be used to indicate whether frequency is barred.

For a REDCAP UE, in case the UE considers the cell is barred and cannot camp on the cell due to any of above criteria, whether the current intraFreqReselection applies to REDCAP UE needs to be discussed.

Question 4. In case a REDCAP UE considers the cell is barred and cannot camp on:

- Option 1. The UE checks legacy intraFreqReselection to determine whether the frequency is barred

- Option 2. A separate flag in system information is introduced for REDCAP UE to determine whether the frequency is barred for REDCAP UEs

| Company name | Option? | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | We do not see strong use case that would require the use of a separate flag in system information for the same purpose but just for RedCap UE. |

## When/How to identify REDCAP UE

Proposals in above contributions related to when and how to identify REDCAP UE are listed as following:

| Tdoc number | Company name | Proposals |
| --- | --- | --- |
| R2-2006786 | OPPO | Proposal 4 RAN2 wait for RAN1’s input before considering the need of early RedCap UE’s identification, e.g. in Msg1 or Msg3. |
| R2-2007345 | Huawei, HiSilicon | Proposal 2: REDCAP UE could be identified by Msg1/Msg A or by different initial UL BWP. |
| R2-2007493 | MediaTek Inc. | Proposal 6: A RedCap UE that is registered to a network is identified by the network at msg5. Proposal 7: The UE can indicate that it is a RedCap UE as part of msg5. |

In above proposals, the following options were mentioned:

- Option 1: Separate initial UL/DL BWP for REDCAP UE

- Option 2: Msg1/A

- Option 3: Msg3

- Option 4: Msg5

Whether a REDCAP UE needs to be identified by the gNB at early stage depends on:

- Whether the UE is allowed to camp on a cell with larger initial UL/DL BWP than supported by the UE, see Question 3.

- Whether special handling is needed for scheduling of REDCAP UE during RACH procedure, e.g., scheduling of RAR or Msg4.

Question 5. Which option do you prefer for the gNB to identify RECCAP UE?

| Company name | Option? | Comments |
| --- | --- | --- |
| Qualcomm | Option 2 | RedCap UE has reduced coverage due to its reduced capabilities. Among the four messages in a RACH procedure, PUSCH Tx in msg3 hence is the bottleneck and repetition is very likely to be introduced for it to help recover the reduced coverage. But for network to decide whether to schedule repetition for msg3 for a UE, it has to be able to identify RedCap UE since msg1/A. |

## UAC for REDCAP UE

In order to achieve load balancing, UAC mechanism for REDCAP UEs is proposed in above contributions as follows:

| Tdoc number | Company name | Proposals |
| --- | --- | --- |
| R2-2006786 | OPPO | Proposal 2 Existing UAC framework can be reused for RedCap UEs. Proposal 3 After concluding on the number of RedCap UE types, RAN2 ask CT1 to define access identity(ies) for RedCap UEs. |
| R2-2007345 | Huawei, HiSilicon | Proposal 5: Study whether to enhance UAC mechanism for REDCAP UEs. |
| R2-2007493 | MediaTek Inc. | Proposal 4: The UAC mechanism is re-used to control the access of RedCap devices to the network. Proposal 5: Send an LS to SA1 to determine if changes are needed to the UAC mechanism to support RedCap access control. |

The following enhancements are mentioned in above proposals

- Option 1: Introduce a set of additional UAC configuration including UAC parameters of all access categories and access identities for REDCAP UEs

- Option 2: Define new Access Identity for REDCAP UEs (need SA1 work)

- Option 3: Define new Access Categories for REDCAP UEs (need SA1 work)

Question 5. Do you agree to use UAC mechanism for REDCAP UEs?

| Company name | Yes/No? | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |

Question 5a. If the answer to Question 5 is Yes, do you think enhancements to the current UAC mechanism listed above are needed?

| Company name | Option? | Comments |
| --- | --- | --- |
| Qualcomm | Option 2 | We agree with Proposal 2 & 3 in R2-2006786. We don’t see any strong use case that would require introduction of new access categories. RedCap-specific access causes can be supported through operator defined access categories. |

## Ensure REDCAP UEs for intended use cases

One objective of REDCAP SID is ensuring that REDCAP UEs are only used for intended use cases:

- Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1].

The following proposal is related to how to ensure that REDCAP UEs are only used for intended use cases:

| Tdoc number | Company name | Proposals |
| --- | --- | --- |
| R2-2007345 | Huawei, HiSilicon | Proposal 3: It is up to CN to ensure the device type is used for the intended use case. |

In general, use case is related to traffic information which is transparent to RAN. RAN schedules the UEs only based on QoS parameters, i.e. 5QI (data rate, latency, packet error rate, GBR or Non-GBR, etc.) provided by CN. Thus, it is difficult for RAN to identify the use case and there is proposal to let the CN to ensure that REDCAP UEs are only used for intended use cases.

Question 6. Do you agree to let CN to ensure that REDCAP UEs are only used for intended use cases?

| Company name | Yes/No? | Comments |
| --- | --- | --- |
| Qualcomm | Yes | This topic is also discussed in Offline-109. We probably should not duplicate the discussions. |

# Summary

TBD

# Conclusion

This offline discussion focused on proposals for REDCAP:

TBD

# Contact delegates

| Delegate | Company name | Email |
| --- | --- | --- |
| Linhai He | Qualcomm | linhaihe@qti.qualcomm.com |