3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2006800

Online, August 17th - 28th, 2020

Agenda Item:	8.6.2

Source:	Panasonic

Title:	Handling of small data transmission in RRC_INACTIVE

Document for:	Discussion, Decision

RAN#86 meeting approved the new WI on NR small data transmission (WID: RP-193168), with the focus on enabling the RRC_INACTIVE UE to transmit UL small data without changing the UE state.

- The main objects of WID includes:

  - UL small data transmissions for RACH-based schemes (i.e. 2-step and 4-step RACH):

    - General procedure to enable UP data transmission for small data packets from INACTIVE state (e.g. using MSGA or MSG3) [RAN2]

    - Enable flexible payload sizes larger than the Rel-16 CCCH message size that is possible currently for INACTIVE state for MSGA and MSG3 to support UP data transmission in UL (actual payload size can be up to network configuration) [RAN2]

  - Context fetch and data forwarding (with and without anchor relocation) in INACTIVE state for RACH-based solutions [RAN2, RAN3]

In this document, we discuss the UE criteria to switch from RRC_INACTIVE to RRC_CONNECTED for an efficient data transmission.

# Discussion

It is usually assumed that gNB decides to let the UE continue UL data transmission in RRC_INACTIVE or RRC_CONNECTED depend on UE buffer status report transmitted together with small data. UE may not include BSR when provided grant is large enough to carry all UL data. As a result, gNB may not switch a UE from RRC_INACTIVE state to RRC_CONNECTED state as gNB may assume not containing BSR imply no more data to be sent by UE.

Observation 1: For UL data transmission, the gNB may decide whether to keep UE in RRC_INACTIVE or RRC_CONNECTED based on UE buffer status report.

Based on the observation 1, a key problem is that the gNB may not put UE into RRC_CONNECTED if UE has frequent small UL data transmission at short interval (i.e. if MSGA or MSG3 grant is sufficient to carry all UL data). Therefore, UE would send data transmission in RRC_INACTIVE by performing successive RACH procedures. This would defeat the purpose of data transmission in RRC_INACTIVE since it increases power consumption as well as signaling overhead. As a result, for frequent amount of small UL data, it would be good to rely on the RRC_CONNECTED state for data transmission. In order to prevent frequent transmission of small data in RRC_INACTIVE, one approach could be UE should allow to trigger its preference regarding UE’s state (i.e. whether to perform small data transmission in RRC_INACTIVE or RRC_CONNECTED) to the network which can be determined by UE based on the traffic activity/characteristic.

Observation 2: The UE with frequent small UL data moved to RRC_INACTIVE results in a high power consumption and an increased signaling overhead.

Proposal 1: RRC_INACTIVE UE can send its preference of RRC CONNECTED or RRC_INACTIVE state depending on UE expects more small data to send in the near future based or not.

# Conclusion

In this document, we discuss the UE criteria to switch from RRC_INACTIVE to RRC_CONNECTED for an efficient data transmission. Additionally, we ask RAN2 to discuss the following observations and proposals:

Observation 1: For UL data transmission, the gNB may decide whether to keep UE in RRC_INACTIVE or RRC_CONNECTED based on UE buffer status report.

Observation 2: The UE with frequent small UL data moved to RRC_INACTIVE results in a high power consumption and an increased signaling overhead.

Proposal 1: RRC_INACTIVE UE can send its preference of RRC CONNECTED or RRC_INACTIVE state depending on UE expects more small data to send in the near future or not.

# References

[1] RP-193168: Work Item on NR small data transmissions in INACTIVE state