3GPP TSG-RAN2 #111e	R2-2007061

# Electronic meeting, August 17 – August 28, 2020

Agenda item:		6.1.3 (TEI16)

Source:	LG Electronics Inc.

Title: 	Stopping ongoing Random Access procedure

Document for:	Discussion and Decision

1.	Introduction

The UE is allowed to stop ongoing RA procedure in certain circumstances. This is optional UE behavior. There was an attempt to simplify the text on this optional UE behavior from RAN2#109bis-e meeting (R2-2002931).

During the discussion, most companies agreed that the current text is too complicated for optional UE behavior, and it would be nice to have some simplification. However, companies want to wait for the final text result from various WIs, e.g. 2-step RA, NR-U, and eMIMO.

Now the v16.1.0 is published, and it is good time to discuss the simplification of the text on the optional UE behavior.

2.	Discussion

The problematic part of the specification v16.1.0 is captured below.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BSR and BFR which has no valid PUCCH resources configured, which was initiated by MAC entity prior to the MAC PDU assembly. The ongoing Random Access procedure due to a pending SR for BSR may be stopped when the MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly, or when the UL grant(s) can accommodate all pending data available for transmission. The ongoing Random Access procedure due to a pending SR for BFR of an SCell may be stopped when the MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload and this PDU contains an BFR MAC CE or Truncated BFR MAC CE which includes beam failure recovery information of that SCell. Upon deactivation of SCell (as specified in clause 5.9) configured with beam failure detection the ongoing Random Access procedure due to a pending SR for BFR may be stopped if all triggered BFRs for SCells are cancelled. The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for consistent LBT failure, which has no valid PUCCH resources configured, if: - all the SCells that triggered consistent LBT failure are deactivated (see clause 5.9); or - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes an LBT failure MAC CE that indicates consistent LBT failure for all the SCells that triggered consistent LBT failure.

As can be seen, the text becomes huge in order to incorporate optional RA stopping for 2-step RA, BFR, and LBT. Most of all, the text is too much complex, and it is not easy to understand. Therefore, we first try to restructure the text in a bulletized form to be easily understood by readers. It is shown in TP1 below.

TP1: Bulletized form The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BSR, which has no valid PUCCH resources configured, if: - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and - this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly; or - the UL grant(s) can accommodate all pending data available for transmission. The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BFR of an SCell, which has no valid PUCCH resources configured, if: - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU contains a BFR MAC CE or a Truncated BFR MAC CE which includes beam failure recovery information of that SCell; or - SCell configured with beam failure detection is deactivated (as specified in clause 5.9) and all triggered BFRs for SCells are cancelled. The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for consistent LBT failure, which has no valid PUCCH resources configured, if: - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes an LBT failure MAC CE that indicates consistent LBT failure for all the SCells that triggered consistent LBT failure; or - all the SCells that triggered consistent LBT failure are deactivated (as specified in clause 5.9).

Though the TP1 could be easily understood, we worry about future update. As the NR grows up, more features will be introduced, and another triggers for RA will also be introduced. If the text is left as it is, each time a new RA trigger is introduced, this part of the MAC specification will also need to be updated. We think this kind of effort is not productive and worthless, and propose to simplify the text to avoid such effort.

During the e-mail discussions in past meetings, companies had opinions that the BSR related text which was there from Rel-15 should be kept, while the text related to Rel-16 features could be simplified. We think it is a reasonable approach, and propose to go this way. As example of the TP is shown in TP2 below.

TP2: Bulletized form with simplification The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BSR, which has no valid PUCCH resources configured, if: - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and - this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly; or - the UL grant(s) can accommodate all pending data available for transmission. The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for other reasons (e.g. BFR of an SCell, consistent LBT failure), which has no valid PUCCH resources configured, if the ongoing Random Access procedure is no more needed due to e.g. cancelling the pending SR that triggered the Random Access procedure.

3.	Proposals

It is not worth to develop more and more conditions for optional RA stopping. It is asked to companies to think about how to remove such redundancies from the MAC specification. Two TPs are provided in this document to help the discussion. It is proposed for RAN2 to adopt one of the TPs.

Proposal: RAN2 discuss how to simplify the text on optional RA stopping, and adopt either TP1 or TP2.