3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-20xxxxx

# Elbonia, 17 – 28 August 2020																		R2-2004907

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1515 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of FR2 MPE P-MPR reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Maximum Permissible Exposure (MPE) limit set by FCC aims at restricting the UE Tx power averaged over a defined period of time for limiting RF exposure on human body for FR2. UE may be required to apply a large P-MPR value to comply with those requirements. Unless network is aware of the applied P-MPR, radio link failure or connection releases due to significant and unpredictable UE P-MPR may occur. Therefore, UE reporting of the P-MPR used to meet the FR2 MPE limits is required to prevent this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FR2 MPE P-MPR reporting configuration is introduced: - FR2 MPE P-MPR reporting configuration within MAC-CellGroupConfig, applicable to all FR2 serving cells within the same cell group |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of FR2 MPE P-MPR reporting is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.300 CR0210R2, TS38.321 CR0707R2, TS38.306 CRXXXX (capability), TS38.331 CRXXX (capability) |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The UE capability signalling for the FR2 MPE P-MPR reporting are covered in a separate CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 6.3.2	Radio resource control information elements

<UNNECESSARY PARTS OMITTED>

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                                BOOLEAN                                                     OPTIONAL,   -- Need M

dataInactivityTimer                     SetupRelease { DataInactivityTimer }                        OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,    -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,    -- Need M

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,    -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,    -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup }                       OPTIONAL     -- Need M

]],

[[

fr2-MPE-P-MPR-Config-r16            SetupRelease { FR2-MPE-P-MPR-Config-r16 }                       OPTIONAL     -- Need M

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

fr2-MPE-P-MPR-Config Used to configure FR2 MPE P-MPR reporting as specified in TS 38.321 [3].

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre- emptive-BSR.

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config Used to configure DRX as specified in TS 38.321 [3].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3].

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

skipUplinkTxDynamic If set to true, the UE skips UL transmissions as described in TS 38.321 [3].

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |

<UNNECESSARY PARTS OMITTED>

#### –	FR2-MPE-P-MPR-Config

The IE FR2-MPE-P-MPR-Config is used to configure parameters for FR2 MPE P-MPR reporting.

P-MPR-Config information element

-- ASN1START

-- TAG-FR2-MPE-P-MPR-CONFIG-START

FR2-MPE-P-MPR-Config-r16 ::=     SEQUENCE {

p-MPR-Threshold-r16                     P-MPR-Range-r16,

p-MPR-ProhibitTimer-r16                 ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000},

...

}

-- TAG-FR2-MPE-P-MPR-CONFIG-STOP

-- ASN1STOP

P-MPR-Config field descriptions

p-MPR-ProhibitTimer Value in number of subframes for P-MPR reporting for FR2 MPE as specified in TS 38.321 [3]. Value sf0 corresponds to 0 subframe, value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes, and so on.

p-MPR-Threshold Value in dB for P-MPR reporting for FR2 MPE as specified in TS 38.321 [3]. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

<UNNECESSARY PARTS OMITTED>

#### –	P-MPR-Range

The IE P-MPR-Range specifies the value range used in for FR2 MPE P-MPR reporting in MAC. The integer value for P-MPR reporting is according to Table X.Y.Z-W in TS 38.133 [14].

P-MPR-Range information element

-- ASN1START

-- TAG-P-MPR-RANGE-START

P-MPR-Range-r16 ::= INTEGER (0..15)

-- TAG-P-MPR-RANGE-STOP

-- ASN1STOP