3GPP TSG-RAN WG2 Meeting RAN2#111 electronic	R2-2007472

Online, August 17th - 28th, 2020

Agenda item:		9.1.2

Source:		Lenovo, Motorola Mobility

Title:		       Neighbor cell measurements triggering before RLF

Document for:		Discussion and Decision

- Introduction

Based on WI RP-201306 as in [1], the following content is proposed in the WI objective:

- Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

In this contribution, it mainly focuses on the neighbour cell measurement triggering condition before RLF, and the signalling for neighbour cell measurement.

- Discussion

For legacy NB-IOT UE, considering UE power saving, there is no measurement configuration and measurement report procedure for UE in connected mode, the UE could trigger the RRC connection re-establishment procedure if it moves to another cell. However, in some scenario, the cell reselection based on RLF and re-establishment procedure may be not time efficient, because the cell selection will be triggered at the time of expired T310 as in figure.1.

Figure.1 The procedure of cell selection in RLF and Re-establishment procedure

In order to reduce the time taken to RRC reestablishment to another cell procedure, for UE in connected mode, the following options could be further discussed to trigger the neighbour cell measurement before the RLF procedure.

  - Option1, the neighbour cell measurement could be trigger when the serving cell channel quality is lower than a threshold. The definition that the serving cell channel quality is lower than a threshold could be various, it could be as follows:

    - The definition based on s-Measure,

    - The definition based on the formulation including the offset and Hys used in measurement configuration.

  - Option2，the neighbour cell measurement could be trigger based on the RLM procedure. For example, after n number of consecutive "out-of-sync" indications for PCell is detected.

Proposal1: The trigger condition for neighbor cell measurement could be down selected based on the below two options:

- Option1, the neighbour cell measurement could be trigger when the serving cell channel quality is lower than a threshold.

- Option2, the neighbour cell measurement could be trigger based on the RLM procdure. For example, after n number of consecutive "out-of-sync" indications for PCell is detected.

After triggering the neighbour cell measurement, since no measurement configuration could be transmitted to connected NB-IOT UE, the neighbour cell measurement could follow the parameter such as the frequency priority information configured in SIB for cell reselection. Then UE will choose the target cell with high channel quality and frequency priority to send RRC reestablishment request, it is UE implementation which cell is selected as the target cell.

Proposal2: The neighbour cell measurement could follow the parameter such as the frequency priority information configured in SIB for cell reselection, it is UE implementation which cell is selected as the target cell.

In legacy specification, the target cell needs to retrieve the UE context from source cell after receiving the re-establishment request from UE. In order to reduce the latency and ensure service continuity, some assistant information can be sent to eNB, then eNB could send its UE context to several neighbour eNB. The assistant information including the candidate neighbour cell information could be reported to eNB.

Proposal3: The assistant information including the candidate neighbour cell information could be reported to help eNB deliver the UE context to several neighbor eNB.

- Conclusion

In this contribution, the issues on neighbour cell measurement triggering condition before RLF, and the signalling for neighbour cell measurement are discussed, we have the following proposals:

Proposal1: The trigger condition for neighbor cell measurement could be down selected based on the below two options:

- Option1, the neighbour cell measurement could be trigger when the serving cell channel quality is lower than a threshold.

- Option2, the neighbour cell measurement could be trigger based on the RLM procdure. For example, after n number of consecutive "out-of-sync" indications for PCell is detected.

Proposal2: The neighbour cell measurement could follow the parameter such as the frequency priority information configured in SIB for cell reselection, it is UE implementation which cell is selected as the target cell.

Proposal3: The assistant information including the candidate neighbour cell information could be reported to help eNB deliver the UE context to several neighbor eNB.

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, June 29 - July 3, 2020.