3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007753

Online, August 17th – 28th, 2020

Agenda Item:	6.10.3 RRC corrections

Source:	Huawei, HiSilicon

Title:	Discussion on user consent

Document for: Discussion and Decision

Introduction

In the last RAN2#110-e meeting, RAN2 has agreed the network will not configure the UE to report location information in RLF /CEF/SCG failure report if network doesn’t have the user consent of this UE.

=> From RAN2 understanding network will not configure UE to report location information for SON/MDT purpose if network doesn’t get the user consent from this UE. Inform SA5 our understanding and ask SA5 to finish the work accordingly. => User consent should be also applied to RLF report cases (including SCG failure case). Inform SA5 this agreement. => Draft reply LS to SA5 and cc RAN3 to show the whole picture of SON/MDT and request SA5 to complete the corresponding work in time.

In the last meeting, RAN2 has extended the meaning of obtainCommonLocation in TS 38.331

1> if the received otherConfig includes the obtainCommonLocation: 2> include available detailed location information for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

In this contribution, we will discuss more details impacts on the specifications.

Discussion

RLF report and CEF report

In TS 38.331 [1], for RLF report and CEF report, the network can control whether the UE should collect and send the location info to the network, i.e. Common location, WLAN measurements, BT measurements, and Sensor measurements. Detailed text is showed as below:

1> if the received otherConfig includes the obtainCommonLocation: 2> include available detailed location information for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation; NOTE 1: The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation. 1> if the received otherConfig includes the BT-NameList: 2> if BT-NameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation; 1> if the received otherConfig includes the WLAN-NameList: 2> if WLAN-NameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation; NOTE 2: The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation. 1> if the received otherConfig includes the Sensor-NameList: 2> if Sensor-NameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

Observation 1: For NR RLF report and CEF report, the network can control the UE whether to report location info or not.

For LTE specification [1], currenlty it is missing location info for RLF report and CEF report. Detailed text is showed as below. In order to fix the issue, NR definition can be considered and the corresponding changes are in red.

1> if the received otherConfig includes the obtainLocation: 2> attempt to have detailed location information available for any subsequent measurement report or any subsequent RLF report, CEF report; NOTE 1: The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation. 1> if the received otherConfig includes the bt-NameListConfig: 2> if bt-NameListConfig is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report or any subsequent RLF report, CEF report; 1> if the received otherConfig includes the wlan-NameListConfig: 2> if wlan-NameListConfig is set to setup, attempt to have WLAN measurement results available for subsequent measurement report or any subsequent RLF report, CEF report; NOTE 2: The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.

Proposal 1: For LTE RLF report and CEF report, it is proposed to apply location info configuration, i.e. obtainLocation, bt-nameListConfig, wlan-NameListConfig.

SCG failure report

For SCG failure report, it is more complicated than RLF report and CEF report because it involves more than one nodes/RATs. So we are to discuss location info configuration for SCG failure report in different scenarios.

    - NR-DC scenario

For NR-DC scenario, inside the SCGFailureInformation message, the field locationInfo indicates location info. As mentioned in section 2.1, the current NR spec has already covered location info configuration for RLF report, CEF report and SCGFailureInformation cases.

FailureReportSCG ::= SEQUENCE { failureType ENUMERATED { t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-ReconfigFailure, srb3-IntegrityFailure, other-r16, spare1}, measResultFreqList MeasResultFreqList OPTIONAL, measResultSCG-Failure OCTET STRING (CONTAINING MeasResultSCG-Failure) OPTIONAL, ..., [[ locationInfo-r16 LocationInfo-r16 OPTIONAL, failureType-v1610 ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16, t312-Expiry-r16, bh-RLF-r16, spare4, spare3, spare2, spare1} OPTIONAL ]] }

Observation 2: For NR-DC scenario, the current NR spec has already covered location info configuration for SCGFailureInformation cases.

    - NE-DC scenario

For NE-DC scenario, SCG failure information is related to the SCGFailureInformationEUTRA message. The details are showed as below.

In general, two places can include location info, measResultSCG-FailureMRDC (highlighted in yellow) and locaitonInfo-r16 (highlighted in green). For measResultSCG-FailureMRDC, it refers to LTE 36.331 and currenlty there is no definition for location info for this field. For locationInfo, as mentioned in section 2.1, the procedural text in TS 38.331 is missing location info configuration for the SCGFailureInformationEUTRA message.

SCGFailureInformationEUTRA message -- ASN1START -- TAG-SCGFAILUREINFORMATIONEUTRA-START SCGFailureInformationEUTRA ::= SEQUENCE { criticalExtensions CHOICE { scgFailureInformationEUTRA SCGFailureInformationEUTRA-IEs, criticalExtensionsFuture SEQUENCE {} } } SCGFailureInformationEUTRA-IEs ::= SEQUENCE { failureReportSCG-EUTRA FailureReportSCG-EUTRA OPTIONAL, nonCriticalExtension SCGFailureInformationEUTRA-v1590-IEs OPTIONAL } SCGFailureInformationEUTRA-v1590-IEs ::= SEQUENCE { lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } FailureReportSCG-EUTRA ::= SEQUENCE { failureType ENUMERATED { t313-Expiry, randomAccessProblem, rlc-MaxNumRetx, scg-ChangeFailure, spare4, spare3, spare2, spare1}, measResultFreqListMRDC MeasResultFreqListFailMRDC OPTIONAL, measResultSCG-FailureMRDC OCTET STRING OPTIONAL, ..., [[ locationInfo-r16 LocationInfo-r16 OPTIONAL ]] } MeasResultFreqListFailMRDC ::= SEQUENCE (SIZE (1.. maxFreq)) OF MeasResult2EUTRA -- TAG-SCGFAILUREINFORMATIONEUTRA-STOP -- ASN1STOP

SCGFailureInformationEUTRA field descriptions

measResultFreqListMRDC The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by measConfig.

measResultSCG-FailureMRDC Includes the E-UTRA MeasResultSCG-FailureMRDC IE as specified in TS 36.331 [10]. The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by the E-UTRA RRCConnectionReconfiguration message.

In LTE, the details of MeasResultSCG-FailureMRDC are showed as below:

- – MeasResultSCG-FailureMRDC The IE MeasResultSCG-FailureMRDC is used to provide measurement information concerning E-UTRA measurements upon SCG failure detected by a UE configured with NE-DC. MeasResultSCG-FailureMRDC information element -- ASN1START MeasResultSCG-FailureMRDC-r15 ::= SEQUENCE { measResultFreqListEUTRA-r15 MeasResultList3EUTRA-r15, ..., [[ locationInfo-r16 LocationInfo-r10 OPTIONAL, logMeasResultListBT-r16 LogMeasResultListBT-r15 OPTIONAL, logMeasResultListWLAN-r16 LogMeasResultListWLAN-r15 OPTIONAL ]] } MeasResultList3EUTRA-r15 ::= SEQUENCE (SIZE (1..maxFreq)) OF MeasResult3EUTRA-r15 MeasResult3EUTRA-r15 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueEUTRA-r9, measResultServingCell-r15 MeasResultEUTRA OPTIONAL, measResultNeighCellList-r15 MeasResultListEUTRA OPTIONAL, ... } -- ASN1STOP

Proposal 2: In LTE 36.331, apply location info configuration (i.e. location, WLAN measurement, Bluetooth measurement) to measResultSCG-FailureMRDC.

Proposal 3: In LTE 38.331, apply location info configuration (i.e. locationInfo, WLAN measurement, Bluetooth measurement, Sensor measurement) to SCGFailureInformationEUTRA.

    - EN-DC scenario

For EN-DC scenario, SCG failure information is related to the SCGFailureInformationNR message. The details are showed as below.

In general, two places can include location info, measResultSCG (highlighted in yellow) and locaitonInfo-r16 (highlighted in green). For measResultSCG, it refers to NR 38.331 and currenlty there is no definition for location info for this field. For locationInfo, the procedural text in TS 36.331 is missing location info configuration for the SCGFailureInformationNR message.

- – SCGFailureInformationNR The SCGFailureInformationNR message is used to provide information regarding NR SCG failures detected by the UE. Signalling radio bearer: SRB1 RLC-SAP: AM Logical channel: DCCH Direction: UE to E-UTRAN SCGFailureInformationNR message -- ASN1START SCGFailureInformationNR-r15 ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE { scgFailureInformationNR-r15 SCGFailureInformationNR-r15-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } SCGFailureInformationNR-r15-IEs ::= SEQUENCE { failureReportSCG-NR-r15 FailureReportSCG-NR-r15 OPTIONAL, nonCriticalExtension SCGFailureInformationNR-v1590-IEs OPTIONAL } SCGFailureInformationNR-v1590-IEs ::= SEQUENCE { lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } FailureReportSCG-NR-r15 ::= SEQUENCE { failureType-r15 ENUMERATED { t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-reconfigFailure, srb3-IntegrityFailure, other-r16}, measResultFreqListNR-r15 MeasResultFreqListFailNR-r15 OPTIONAL, measResultSCG-r15 OCTET STRING OPTIONAL, ..., [[ locationInfo-r16 LocationInfo-r10 OPTIONAL, logMeasResultListBT-r16 LogMeasResultListBT-r15 OPTIONAL, logMeasResultListWLAN-r16 LogMeasResultListWLAN-r15 OPTIONAL, failureType-v1610 ENUMERATED {t312-Expiry, scg-lbtFailure, beamFailureRecoveryFailure, spare1} OPTIONAL ]] } MeasResultFreqListFailNR-r15 ::= SEQUENCE (SIZE (1..maxFreqNR-r15)) OF MeasResultFreqFailNR-r15 MeasResultFreqFailNR-r15 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, measResultCellList-r15 MeasResultCellListNR-r15 OPTIONAL, ... } -- ASN1STOP

SCGFailureInformationNR field descriptions

failureType Indicates the cause of the SCG failure.

measResultFreqListNR The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

In NR, the details of MeasResultSCG-Failure are showed as below:

- – MeasResultSCG-Failure The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in EN-DC. MeasResultSCG-Failure information element -- ASN1START -- TAG-MEASRESULTSCG-FAILURE-START MeasResultSCG-Failure ::= SEQUENCE { measResultPerMOList MeasResultList2NR, ..., [[ locationInfo-r16 LocationInfo-r16 OPTIONAL ]] } MeasResultList2NR ::= SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR -- TAG-MEASRESULTSCG-FAILURE-STOP -- ASN1STOP

Proposal 4: In LTE 36.331, apply location info configuration (i.e. location, WLAN measurement, Bluetooth measurement) to SCGFailureInformationNR.

Proposal 5: In LTE 38.331, apply location info configuration (i.e. locationInfo, WLAN measurement, Bluetooth measurement, Sensor measurement) to MeasResultSCG-Failure.

In previous meetings, RAN2&3 introduced the user consent (i.e. Management Based MDT Allowed indication and optionally the Management Based MDT PLMN List) for the MDT. From RAN2 point of view, the motivation of including the location information in RLF/CEF/SCG failure reporting is from the feature MDT. Even if the RLF/CEF/SCG failure reporting are separate from MDT, the network can also use the MDT user consent info to decide whether the UE can report the location information in these failure reporting, and this will minimize the impacts on specifications.

TS 38.413 of RAN3: If the Management Based MDT PLMN List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, use it to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11].

TS 32.422 of RAN3: 1 Scope The mechanisms for Trace, MDT and RLF reporting activation/deactivation are detailed in clause 4; clause 5 details the various Trace, MDT and RLF reporting control and configuration parameters and the triggering events that can be set in a network. Trace, MDT and RLF reporting concepts and requirements are covered in 3GPP TS 32.421 [2] while Trace and MDT data definition and management is covered in 3GPP TS 32.423 [3]. ….. ## 5.9a Job type (CM) The Job type is a conditional mandatory parameter. The condition is either MDT or RLF or RCEF data collection functionality is supported. It defines if a single trace job, a combined MDT and trace job or RLF report collection job or RCEF report collection job is activated. This parameter also defines the MDT mode. The Job type parameter is an enumerated type with the following values: - Immediate MDT only (0); - Logged MDT only (1); - Trace only (2); - Immediate MDT and Trace (3); - RLF reports only (4) ; - RCEF reports only (5). - Logged MBSFN MDT (6). NOTE 1: The Job type "RLF reports only" and "RCEF reports only" are applicable only in management based trace activation in E-UTRAN. NOTE 2: Logged MBSFN MDT is applicable only for activation in E-UTRAN.

Proposal 6: The RLF/CEF/SCG failure reporting and MDT use the same user consent.

Conclusion

Based on the above technical discussion, we have the following proposals:

Proposal 1: For LTE RLF report and CEF report, it is proposed to apply location info configuration, i.e. obtainLocation, bt-nameListConfig, wlan-NameListConfig.

Proposal 2: In LTE 36.331, apply location info configuration (i.e. location, WLAN measurement, Bluetooth measurement) to measResultSCG-FailureMRDC.

Proposal 3: In LTE 38.331, apply location info configuration (i.e. locationInfo, WLAN measurement, Bluetooth measurement, Sensor measurement) to SCGFailureInformationEUTRA.

Proposal 4: In LTE 36.331, apply location info configuration (i.e. location, WLAN measurement, Bluetooth measurement) to SCGFailureInformationNR.

Proposal 5: In LTE 38.331, apply location info configuration (i.e. locationInfo, WLAN measurement, Bluetooth measurement, Sensor measurement) to MeasResultSCG-Failure.

Proposal 6: The RLF/CEF/SCG failure reporting and MDT use the same user consent.

For proposal 1~5, the corresponding changes have been shown in section 5, and there are no ASN1 changes.

Reference

[1] TS 38.331 v16.1.0

[2] TS 36.331 v16.1.1

Changes on 38.331 for user consent

#### 5.3.5.9	Other configuration

The UE shall:

----------------------------------skip the unchanged part-------------------------------

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report, CEF report, SCGFailureInformation, SCGFailureInformationEUTRA and MeasResultSCG-Failure;

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the BT-NameList:

2>	if BT-NameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report, CEF report, SCGFailureInformation, SCGFailureInformationEUTRA and MeasResultSCG-Failure ;

1>	if the received otherConfig includes the WLAN-NameList:

2>	if WLAN-NameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report, CEF report, SCGFailureInformation, SCGFailureInformationEUTRA and MeasResultSCG-Failure ;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.

1>	if the received otherConfig includes the Sensor-NameList:

2>	if Sensor-NameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report, CEF report, SCGFailureInformation, SCGFailureInformationEUTRA and MeasResultSCG-Failure ;

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	if sl-AssistanceConfigNR is set to true:

3>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

Changes on 36.331 for user consent

#### 5.3.10.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the reportProximityConfig:

2>	if proximityIndicationEUTRA is set to enabled:

3>	consider itself to be configured to provide proximity indications for E-UTRA frequencies in accordance with 5.3.14;

2>	else:

3>	consider itself not to be configured to provide proximity indications for E-UTRA frequencies;

2>	if proximityIndicationUTRA is set to enabled:

3>	consider itself to be configured to provide proximity indications for UTRA frequencies in accordance with 5.3.14;

2>	else:

3>	consider itself not to be configured to provide proximity indications for UTRA frequencies;

1>	if the received otherConfig includes the obtainLocation:

2>	attempt to have detailed location information available for any subsequent measurement report or any subsequent RLF report, CEF report, SCGFailureInformationNR and MeasResultSCG-FailureMRDC;

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the bt-NameListConfig:

2>	if bt-NameListConfig is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report or any subsequent RLF report, CEF report, SCGFailureInformationNR and MeasResultSCG-FailureMRDC;

1>	if the received otherConfig includes the wlan-NameListConfig:

2>	if wlan-NameListConfig is set to setup, attempt to have WLAN measurement results available for subsequent measurement report or any subsequent RLF report, CEF report, SCGFailureInformationNR and MeasResultSCG-FailureMRDC;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.