3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007871

E-Meeting, 17th Aug - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1939 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to 38.331 on SL operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-8-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. In the section 5.8.7 Sidelink communication reception, UE is configured to ‘monitor sidelink control information and the corresponding data’ in the RX pools when related conditions are satisfied, and the description text is inherited from LTE specification. However, in NR sidelink communication, the UE may also monitor PSFCH for HARQ feedback. Therefore, the text would be misleading and implythat UE doesn’t monitor PSFCH. 2. During timer T400 running period for sidelink RRC reconfiguration procedure, if there is detection of sidelink RLF for a PC5-RRC connection based on indication from lower layers (e.g., HARQ DTX in MAC), the PC5-RRC connection is considered as released for the destination. However, T400 may continue running and expire, there would be detection of sidelink RLF again for the PC5-RRC connection based on T400 expiry. It’s an unexpected case at the UE because the PC5-RRC connection has already been considered as released before. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add ‘receive HARQ feedback (if any)’ as one of UE’s behaviour in RX pools in 5.8.7. 2. Add procdure for stopping T400 upon sidelink RLF detection based on indication from lower layers (i.e., MAC/RLC/PDCP) in 5.8.9.3. 3. Add stop condition for T400 upon sidelink RLF detection based on indication from lower layers (i.e., MAC/RLC/PDCP) in 7.1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. UE will not be configured to receive HARQ feedback (if any) on RX pools. 2. Unexpected case may occur that a sidelink UE may perform PC5 RRC connection release towards a destination which doesn’t exsit. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.7, 5.8.9.3, 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------[Start of Change] ---------------------------

5.8.7	Sidelink communication reception

A UE capable of NR sidelink communication that is configured by upper layers to receive NR sidelink communication shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in RRCReconfiguration message or sl-FreqInfoList included in SIB12:

3>	if the UE is configured with sl-RxPool included in RRCReconfiguration message with reconfigwithSync (i.e. handover):

4>	configure lower layers to monitor sidelink control information and the corresponding data, and receive HARQ feedback (if any) using the pool of resources indicated by sl-RxPool;

3>	else if the cell chosen for NR sidelink communication transmission provides SIB12:

4>	configure lower layers to monitor sidelink control information and the corresponding data, and receive HARQ feedback (if any) using the pool of resources indicated by sl-RxPool in SIB12;

2>	else:

3>	configure lower layers to monitor sidelink control information and the corresponding data, and receive HARQ feedback (if any) using the pool of resources that were preconfigured by sl-RxPool in SL-PreconfigurationNR, as defined in sub-clause 9.3;

--------------------------[Start of Next Change] --------------------------

5.8.9.3	Sidelink radio link failure related actions

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry; or

1>	upon indication from sidelink MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3:

2>	consider sidelink radio link failure to be detected for this destination;

2>	if T400 is running:

3>	stop timer T400;

2>	release the DRBs of this destination, in according to sub-clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to sub-clause 5.8.9.1a.3;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3 or sub-clause 5.10.X in TS 36.331 [10];

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

--------------------------[Start of Next Change] --------------------------

7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T400 | Upon transmission of RRCReconfigurationSidelink | Upon reception of RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink or upon sidelink RLF detection based on indication from lower layers (i.e., MAC/RLC/PDCP) | Perform the sidelink RRC reconfiguration failure procedure as specified in 5.8.9.1.4 |

---------------------------[End of change] ---------------------------------