3GPP TSG-RAN WG2 Meeting #111-e													R2-2008365

Electronic meeting, August 17th - 28th, 2020

Agenda Item:	6.14.2

Source: 	CMCC

Title:  	Issue on SCell Slot Slit for Unaligned CA

Document for:	Discussion and Decision

Introduction

During recent RAN1 meetings, most of the remaining issues on unaligned CA have been identified and addressed. Among the remaining issue, one issue related to SCell slot slit for unaligned CA will impact on RAN2:

	Issue 1: Interpretation of slots indicated by the bitmap in measurementSlots in TS38.331 for unaligned CA case

Hence, in this contribution, we elaborate the issue and provide the solution to address the issue.

Discussion

As illustrated in [1] [2], the issue comes from that when unaligned CA is applied, there will be slots partially overlapped with the SMTC window. For example, for Pcell=60kHz, SCell=15kHz, N = -1 (ca_slot_offset = -1), it is not clear whether slot 0 or slot 1 shown below (in red rectangle) should be the first slot in measurementSlots for the SCell. And in the below figure, SF means subframe, MO means measurement occasion; SMTC window is defined by PCell timing.

On the other hand, in TS 38.331 [3], the definition of “first slot in the SMTC window” is highlighted in yellow, then for unaligned CA case, it is not clear whether a partially overlapped slot with SMTC window should be treated as “a slot in the SMTC window”. Then to avoid ambiguity, some companies propose only complete slots inside the SMTC window are indicated by the bitmap in measurementSlots.

Regarding RSSI measurement, RSSI is a measured value not related to cell dimension, but rather an aggregated measured value from all cells in a specific frequency layer. Then, in the definition of SS reference signal received quality (SS-RSRQ) in TS38.215 [4], it is noted that:

-	For intra-frequency measurements, NR Carrier RSSI is measured with timing reference corresponding to the serving cell in the frequency layer

This means a reference RSSI measurement timing based on one serving cell in this frequency layer needs to be determined. If there is only one SCell (no PCell) in this frequency layer, and the SCell has a slot offset compared to PCell, then this issue arises.

Additionally, with the definition of “periodicityAndOffset”in TS38.331 for SMTC configuration, the offset is subframe-based granularity, however, in the case of unaligned CA or slot-aligned CA, the offset between Pcell/PScell and Scellis is slot-based granularity, which will produce the case that [2] exemplified, however, even in non-CA case, such example may present between two TDD bands when their frame boundaries are not aligned, furtherly, it may present in asynchronized FDD system for inter-frequency measurement, therefore, RAN2 is required to handle this issue in a unified way.

Observation 1: when unaligned CA is applied, there will be slots partially overlapped with the SMTC window.

Observation 2: Regarding the definition of “first slot in the SMTC window” in TS 38.331, for unaligned CA case, it is not clear whether a partially overlapped slot with SMTC window should be treated as “a slot in the SMTC window”.

Observation 3: Regarding RSSI measurement, a reference RSSI measurement timing based on one serving cell in this frequency layer needs to be determined. If there is only one SCell (no PCell) in this frequency layer, and the SCell has a slot offset compared to PCell, then this issue arises.

Therefore, we have the following proposal:

Proposal: it is proposed that for the IE measurementSlots defined in 38.331, only complete slots inside the SMTC window are indicated by the bitmap in measurementSlots in case of both intra- frequency measurement and inter-frequency measurement, and add the corresponding clarification for the slot bitmap interpretation of measurementSlots in TS38.331.

Conclusion

Observation 1: when unaligned CA is applied, there will be slots partially overlapped with the SMTC window.

Observation 2: Regarding the definition of “first slot in the SMTC window” in TS 38.331, for unaligned CA case, it is not clear whether a partially overlapped slot with SMTC window should be treated as “a slot in the SMTC window”.

Observation 3: Regarding RSSI measurement, a reference RSSI measurement timing based on one serving cell in this frequency layer needs to be determined. If there is only one SCell (no PCell) in this frequency layer, and the SCell has a slot offset compared to PCell, then this issue arises.

Proposal: it is proposed that for the IE measurementSlots defined in 38.331, only complete slots inside the SMTC window are indicated by the bitmap in measurementSlots in case of both intra- frequency measurement and inter-frequency measurement, and add the corresponding clarification for the slot bitmap interpretation of measurementSlots in TS38.331.

Reference

- R1-20xxxx FL summary on support of unaligned frame boundary for R16 NR inter-band, CMCC

- R1-2005626, Remaining issues on Rel-16 carrier aggregation, MediaTek Inc..

- 3GPP TS 38.331 V16.0.0, ”NR; Radio Resource Control (RRC) protocol specification”

- 3GPP TS 38.215 V16.2.0, ”NR; Physical layer measurements”