3GPP TSG-RAN2#111-e	R2-2008466

# Electronic, 17-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0184 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections (Rapporteur) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Stage-1 requirements for the “list of forbidden TAs” is given in TS 22.011 for NR. This should be added to the related text in this specification. Some IE names in the description of Pcompensation in 5.2.3.2 are not in italics. The subscript for Qrxlevmin is not correct in two places in 5.2.3.2. There is a typo in 5.2.4.9.1 where “relaxed monitoring” is used instead of “relaxed measurement”. The reference to TS 22.261 in “suitable cell” definition is not necessary. For “reserved cell” definition, “tracking area” should be used instead of “registration area”. The name “Forbidden Tracking Areas” is missing “for Roaming”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The reference to TS 22.011 is added for “list of forbiddgen TAs”. - Changed the typeface of “additionalPmax and “NR-NS-PmaxList” to italics in 5.2.3.2. - Corrected subscript for Qrxlevmin is not correct in two places in 5.2.3.2. - Replaced “relaxed monitoring” with “relaxed measurement” in 5.2.4.9.1. - Removed reference to TS 22.261 in “suitable cell” definition. - In “reserved cell” definition, changed “registration area” to “tracking area”. - Added “for Roaming” after “Forbidden Tracking Areas”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will contain editorial errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5,5.2.3.2, 5.2.4.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR Overall Description; Stage 2".

[3]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) - Protocol Specification".

[4]	3GPP TS 38.213: "NR; Physical layer procedures for control ".

[5]	Void

[6]	3GPP TS 36.331: "E-UTRA; Radio Resource Control (RRC) - Protocol Specification".

[7]	3GPP TS 36.304: "E-UTRA; User Equipment (UE) procedures in RRC_IDLE state ".

[8]	3GPP TS 38.133: "NR; Requirements for Support of Radio Resource Management".

[9]	3GPP TS 23.122: "NAS functions related to Mobile Station (MS) in RRC_IDLE state".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 38.215: "NR; Physical layer measurements".

[12]	3GPP TS 22.261: "Service requirements for the 5G system".

[13]	3GPP TS 24.890: "5G System – Phase 1; CT WG1 Aspects".

[14]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[15]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[16]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[17]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[xx]	3GPP TS 22.011: "Service accessibility".

Next Change

## 4.5	Cell Categories

The cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network:

-	The cell is not barred, see clause 5.3.1;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2.

suitable cell:

A cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2.

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" (TS 22.011 [xx]), which belongs to a PLMN that fulfils the first bullet above.

barred cell:

A cell is barred if it is so indicated in the system information, as specified in TS 38.331 [3].

reserved cell:

A cell is reserved if it is so indicated in system information, as specified in TS 38.331 [3].

Following exception to these definitions are applicable for UEs:

-	if a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for the duration of the emergency call.

-	camped on a cell that belongs to a tracking area that is forbidden for regional provision of service; a cell that belongs to a tracking area that is forbidden for regional provision service (TS 23.122 [9], TS 24.501 [14]) is suitable but provides only limited service.

Next Change

#### 5.2.3.2	Cell Selection Criterion

The cell selection criterion S is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm). If the UE supports SUL frequency for this cell, Qrxlevmin is obtained from q-RxLevMinSUL, if present, in SIB1, SIB2 and SIB4, additionally, if QrxlevminoffsetcellSUL is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell; else Qrxlevmin is obtained from q-RxLevMin in SIB1, SIB2 and SIB4, additionally, if Qrxlevminoffsetcell is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell. |
| Qqualmin | Minimum required quality level in the cell (dB). Additionally, if Qqualminoffsetcell is signalled for the concerned cell, this cell specific offset is added to achieve the required minimum quality level in the concerned cell. |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9]. |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9]. |
| Pcompensation | For FR1, if the UE supports the additionalPmax in the NR-NS-PmaxList, if present, in SIB1, SIB2 and SIB4: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) For FR2, Pcompensation is set to 0. |
| PEMAX1, PEMAX2 | Maximum TX power level of a UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 38.101 [15]. If UE supports SUL frequency for this cell, PEMAX1 and PEMAX2 are obtained from the p-Max for SUL in SIB1 and NR-NS-PmaxList for SUL respectively in SIB1, SIB2 and SIB4 as specified in TS 38.331 [3], else PEMAX1 and PEMAX2 are obtained from the p-Max and NR-NS-PmaxList respectively in SIB1, SIB2 and SIB4 for normal UL as specified in TS 38.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 38.101-1 [15]. |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN (TS 23.122 [9]). During this periodic search for higher priority PLMN, the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

Next Change

##### 5.2.4.9.1	Relaxed measurement criterion for UE with low mobility

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

End of changes