3GPP TSG-RAN WG3 #109-e	R3-205783

17-28 August 2020

Online

Agenda item:	17.2

Source: 	CMCC, ZTE, Huawei, Nokia, Nokia Shanghai Bell, NEC, Samsung, CATT

Title: 	TP for TR38.382 on service continuity for slicing

Document for:	Discussion and Approval

Introduction

This contribution provides scenario and issue description to support service continuity for slicing in TR 38.832.

Text proposal for TR 38.832

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

6	Study necessity and mechanisms to support service continuity

6.1	Scenario and issue description

Editor Note: capture the description of scenario and issue.

Editor’s Note: The proposed scenarios listed in this clause remain to be evaluated and updated.

The following two scenarios are considered to support service continuity.

Scenario 1: Slice resource shortage in case of Intra-RA mobility and Inter-RA mobility

Figure  6.1-1: Service interruption due to slice resource shortage

As shown by Figure  6.1-1, the UE’s ongoing slice(s) is/are supported by both the source and the target NG-RAN node. At the time of handover, the target node fails to accept the UE with at least one of the ongoing S-NSSAIs due to e.g. high slice-related load at the target node. Under such circumstance, the service(s) for failed ongoing slice(s) is/are interrupted for the UE.

Editor’s Note:  The study shall analyse the implications of slice remapping in these conditions, e.g. whether or not the remapping of a slice to the re-mapped S-NSSAI, may create an issue of overload in the re-mapped S-NSSAI.

Scenario2: Non-supported slice in case of Inter-RA mobility

Figure  6.1-2: Service interruption due to slice not supported

As shown by Figure 6.1-2, the UE is moving towards an area that does not support at least one of UE’s ongoing slices. The target node fails to accept the UE with at least one of the ongoing S-NSSAIs. Under such circumstance, the service(s) for failed ongoing slice(s) is/are interrupted for the UE.

Editor’s Note: It needs to be analyzed whether, for a well defined SLA and a correctly defined Registration Area in which the slice needs to be available, the slice services should be available also outside of the RA.

6.2	Solutions

Editor Note: Capture the solutions for the scenario and issue.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>