3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2008803

# Online, Nov 2nd - 13th, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.305 | CR | 0093 | rev | <> | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor corrections on TS36.305 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The descriptions of step 2 in clause 7.3.1 is inconsistent with the procedure shown in the corresponding figure 7.3.1-1; Becuase the current descriptions of step 2 in clause 7.3.1 mentions the interaction with serving eNB. However step 2 is the LPP Transactions(s) in figure 7.3.1-1. So some misunderstanings may be introduced that E-SMLC can obtain location related information from the UE and/or from the serving e-NB in LPP transactions as shown in figure 7.3.1-1 - The descriptions of step 3 in clause 7.3.2 is inconsistent with the procedure shown in the corresponding figure 7.3.2-1. The same issue happens in step 3 in clause 7.3.2 and figure 7.3.2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the interaction with serving eNodeB from the description of step2 in clause 7.3.1. - Delete the interaction with serving eNodeB from the description of step3 in clause 7.3.2. Impact analysis Impacted architecture options: LTE Impacted functionality: Positioning Procedures Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability problem - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The descriptions of step2 in clause 7.3.1 and step3 in clause 7.3.2 is inconsistent with the procedure shown in figure 7.3.1-1 and 7.3.2-1 and may introduce some misunderstandings of LPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 7.3.1	NI-LR and MT-LR Service Support

Figure 7.3.1-1 shows the sequence of operations for an NI-LR or MT-LR location service, starting at the point where the MME initiates the service in the E-SMLC.

Figure 7.3.1-1: UE Positioning Operations to support an MT-LR or NI-LR

1.	The MME sends a location request to the E-SMLC for a target UE and may include associated QoS.

2.	If the E-SMLC needs location related information from the UE , the E-SMLC instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate one or more LPP procedures after the first LPP message is received from the E-SMLC (e.g., to request assistance data from the E-SMLC).

3.	If the E-SMLC needs location related information for the UE from the eNode B, the E-SMLC instigates one or more LPPa procedures. Step 3 is not necessarily serialised with step 2; if the E-SMLC and eNode B have the information to determine what procedures need to take place for the location service, step 3 could precede or overlap with step 2.

4.	The E-SMLC returns a location response to the MME with any location estimate obtained as a result of steps 2 and 3.

NEXT CHANGE

### 7.3.2	MO-LR Service Support

Figure 7.3.2-1 shows the sequence of operations for an MO-LR service, starting at the point where an LCS Client in the UE or the user has requested some location service (e.g., retrieval of the UE's location or transfer of the UE's location to a third party).

Figure 7.3.2-1: UE Positioning Operations to support an MO-LR

1.	The UE sends a NAS level MO-LR request to the MME. The MO-LR request may carry an LPP PDU to instigate one or more LPP procedures to transfer capabilities, request assistance data, request location information and/or transfer location information (e.g. location measurements).

2.	The MME sends a location request to the E-SMLC and includes any LPP PDU received in step 1.

3.	If the E-SMLC needs location related information from the UE  or if an immediate response is needed to any LPP procedure instigated by the UE in step 1 (e.g., a request for assistance data), the E-SMLC instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate further LPP procedures after the first LPP message is received from the E-SMLC (e.g., to request assistance data or to request further assistance data).

4.	If the E-SMLC needs location related information for the UE from the eNode B, the E-SMLC instigates one or more LPPa procedures. Step 4 may also precede step 3 or occur in parallel with it.

5.	The E-SMLC returns a location response to the MME with any location estimate obtained as a result of steps 3 and 4, and/or with a final LPP message (e.g., that could provide a location estimate to the UE if requested by the UE in step 1).

6.	If the UE requested location transfer to a third party the MME transfers the location received from the E-SMLC in step 5 to the third party as defined in TS 23.271 [2].

7.	The MME sends a NAS level MO-LR response to the UE, carrying any final LPP PDU that was received in step 5.

END OF CHANGE