3GPP TSG-RAN WG2 Meeting #112-e                                                      R2-2008859

Online, November 2nd - 13th, 2020

Source:	CATT

Title:	Co-existence of NR-U and IIOT in R16

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

During GTW online RAN2#111-e session discussing R17 Uplink enhancements for URLLC in unlicensed controlled environments, some companies mentioned that IIOT and NR-U protocols can already be configured (and coexist) together in R16, thus allowing supporting both feature sets simultaneously. Specifically, it was clarified during this online discussion that the key point to check is whether “IIoT autonomous transmission and NR-U CG retransmission timer can be configured together?”[1] We analyze in this contribution the UE behavior under such configuration and draw conclusions on whether this is indeed possible and if it makes any sense.

Discussion

  - Configuration

In R16, cg-RetransmissionTimer-r16 is mandatorily configured in unlicensed spectrum. So the configuration also supporting IIOT would be:

- lch-BasedPrioritization is configured in MAC-CellGroupConfig

- autonomousTx, harq-ProcID-Offset and cg-RetransmissionTimer-r16 are configured in ConfiguredGrantConfig

With above configuration, the following MAC statement always applies when determining the HARQ ID of a given configured grant opportunity (CGO) [2]:

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation select an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. The UE shall prioritize retransmissions before initial transmissions.

And as a result, CG PUSCH transmissions are always with CG-UCI.

  - Use cases analysis

In order to understand the UE behavior when both IIOT and NR-U are configured in R16, we identify the below usecases:

Usecase A: LBT failure on a prioritized initial transmission

Usecase B: Handling of deprioritized grants:

Here, the only case to check is when the UL grant is first prioritized and then deprioritized, otherwise, no MAC PDU is generated and there is no problem to solve. R16 cases where such PUSCH preemption is supported in PHY are:

- CG-CG with different PHY priorities, the latter can pre-empt the former which is partially transmitted.

- PUSCH transmission stopped by CI indication/SFI indication

- PUSCH preempted by PUCCH of higher priority

In above cases, LBT is always successful otherwise the prioritized PUSCH would not be transmitted in first place so there would not be a need to de-prioritize/preempt it.

There are two subcases:

- B1: gNB decodes neither CG-UCI nor the TB and does not send back DFI=ACK (most likely case)

- B2: gNB decodes CG-UCI and TB of preempted PUSCH and sends back DFI=ACK

Usecase C: LBT failure on retransmissions

We can check if there is any difference with usecase A.

Usecase D: Special case of cg-RetransmissionTimer = configuredGrantTimer

Use case A: LBT failure on a prioritized initial transmission

Figure 1: Handling of LBT failure on a prioritized initial transmission

As shown in Figure 1, upon LBT failure, NR-U autonomous retransmission is triggered since the HARQ process is pending, while IIOT autonomous transmission is not triggered on the next available CGO since the previous CGO for this HARQ process was prioritized.

Unlike for LBT failure on a retransmission (see Section 2.2.4), since configuredGrantTimer is not started on the first CGO, there is no limitation on the number of consecutive LBT failures handled by NR-U autonomous retransmissions.

Observation 1: The pending PDU following an LBT failure on a prioritized initial transmission is handled by NR-U’s autonomous retransmission.

Use case B1: deprioritized grant with reception failure at gNB

Note: Usecases B1/2 and C assume a typical configuration where cg-RetransmissionTimer < configuredGrantTimer while the special case of cg-RetransmissionTimer = configuredGrantTimer is addressed in usecase D.

Figure 2: Handling of deprioritized grant with reception failure at gNB

As shown in Figure 2, NR-U autonomous retransmission is triggered first when cg-RetransmissionTimer expires since no DFI-ACK was received. And in the same CGO, no IIOT autonomous transmission is triggered since the configuredGrantTimer is still running. As can be further observed, the number of consecutive NR-U autonomous retransmissions in case of consecutive transmission failures is limited by the duration of the configuredGrantTimer which is not restarted by the retransmission(s). Such limitation could have been recovered by the IIOT’s autonomous transmission on the first available CGO for the same HARQ process following configuredGrantTimer expiry. However, this is not the case because the condition “if the previous configured uplink grant, in the BWP, for this HARQ process was not prioritized” for triggering an IIOT autonomous transmission is not fulfilled.

Observation 2: The pending PDU of a deprioritized configured grant is handled by NR-U’s autonomous retransmission.

Observation 3: In case of retransmission failure, further retransmissions are also handled by NR-U’s autonomous retransmissions until configuredGrantTimer expires.

Observation 4: In case of consecutive retransmission failures, (IIOT) autonomous transmission doesn’t help in continuing more attempts in transmitting the (still) pending PDU when (NR-U) autonomous retransmission reaches its limit (due to configuredGrantTimer expiry).

Use case B2: deprioritized grant successfully received by gNB

In such case, gNB decodes CG-UCI and TB of preempted PUSCH and sends back DFI=ACK, as shown in Figure 3. As a consequence, DFI-ACK stops both timers and no NR-U autonomous retransmission is triggered. Similarly, IIOT autonomous transmission should not be triggered because the obtained MAC PDU should be considered as “transmitted”.

Figure 3: Handling of deprioritized grant with successful reception at gNB

However, this is not the case currently and the IIOT autonomous transmission will indeed happen because all conditions are met for triggering it (Section 5.4.2.1):

3> else if this uplink grant is a configured grant configured with autonomousTx; and 3> if the previous configured uplink grant, in the BWP, for this HARQ process was not prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process; and 3> if the uplink grant size matches with size of the obtained MAC PDU; and 3> if a transmission of the obtained MAC PDU has not been performed: 4> consider the MAC PDU has been obtained.

In IIOT, the yellow highlighted condition was added to prevent from an autonomous transmission to be triggered if gNB scheduled a dynamic retransmission, as agreed in (RAN2#108):

=> UE shall not perform autonomous transmission of the PDU if network has scheduled a retransmission grant for the PDU. FFS whether we specify some time restriction.

And the yellow highlighted condition obviously means that a complete transmission was performed, otherwise no autonomous transmission would be triggered following a partial transmission of a deprioritized (and preempted) CGO. Hence, in this usecase, with current text, MAC will abnormally trigger an autonomous transmission of a correctly received (and acknowledged) PDU.

Observation 5: Deprioritized grants successfully received by gNB are not further addressed by NR-U retransmissions but are abnormally transmitted twice by IIOT autonomous transmission.

Use case C: LBT failure on retransmissions

In this usecase, the LBT for the first transmission was successful but either the CG was deprioritized, as in Usecase B1, or the transmission failed. In any case, no DFI=ACK is received, which triggers an autonomous retransmission, for which LBT fails. As shown in Figure 4, this is the regular NR-U case and LBT failures are addressed by NR-U autonomous retransmissions similar to Usecase A. But different from Usecase A, configuredGrantTimer is running and NR-U autonomous retransmissions can only be triggered up to configuredGrantTimer expiry. And here again, similar to Usecase B1, IIOT autonomous transmission cannot recover in case NR-U autonomous retransmissions reach the configuredGrantTimer limit because the condition “if the previous configured uplink grant, in the BWP, for this HARQ process was not prioritized” is not fulfilled.

Figure 4: Handling of LBT failure on retransmissions

Observation 6: The pending PDU following an LBT failure on a retransmission is handled by NR-U’s autonomous retransmissions up to configuredGrantTimer expiry.

Observation 7: In case of consecutive LBT failures, (IIOT) autonomous transmission doesn’t help in continuing more attempts in transmitting the (still) pending PDU when (NR-U) autonomous retransmission reaches its limit (due to configuredGrantTimer expiry).

Use case D: Special case of cg-RetransmissionTimer = configuredGrantTimer

This special configuration was allowed in RAN2#111-e [3] to support “immediate new transmission on CG” [1] for traffic types where retransmission is considered useless [4]. As a result, NR-U autonomous retransmission can only be triggered by LBT failure in this configuration.

Figure 5: Handling of deprioritized grant with reception failure at gNB when cg-RetransmissionTimer = configuredGrantTimer

Now the UE behavior on such configuration for a deprioritized CG undetected by gNB is that IIOT autonomous transmission is triggered on the first available CGO after CGT expiry and restarts the timers accordingly (see Figure 5). In other words, IIOT autonomous transmission “replaces” the NR-U autonomous retransmission in this configuration. However, if the NR-U protocol for this configured grant configuration was configured to disallow any autonomous retransmission, it is questionable whether the pending PDUs due to deprioritized grants should be recovered or abandoned as well. The latter approach would be consistent with the “no-retransmission” principle assumed for transmission failures of this configuration. Moreover, it is also questionable whether such traffic type would ever be deprioritized.

Observation 8: (IIOT) autonomous transmissions “replace” the (NR-U) autonomous retransmissions on deprioritized CGs when NR-U is configured for traffic types configured to support “immediate new transmission on CG”. But that is not the expected behavior for such traffic type.

Conclusion of usecase analysis

From observations #1-8, it is clear that, when both cg-RetransmissionTimer and autonomousTx are configured:

- Retransmissions due to either LBT failure or failed transmission or deprioritized CGO are handled by NR-U autonomous retransmissions, not IIOT autonomous transmissions

- IIOT autonomous transmission does not help recovering NR-U limitations on the number of consecutive autonomous retransmissions

- (IIOT) autonomous transmissions lead to abnormal behaviors in some usecases.

Proposal #1: RAN2 confirms that, in R16, NR-U autonomous retransmission feature can already address pending PDUs of deprioritized configured grants, and configuring IIOT’s autonomousTx on top is both useless and can lead to abnormal behavior in some configurations.

  - Further ambiguous behaviors

Prioritization of retransmissions

As mentioned during the GTW online session, “in NR-U retx are always prioritized over initial tx so we need to check if this causing any issue” [1]. And there is indeed a contradictory behavior when both cg-RetransmissionTimer and lch-BasedPrioritization are configured, captured in the same Section 5.4.1:

- NR-U: retransmissions are always prioritized over new transmissions:

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation select an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. The UE shall prioritize retransmissions before initial transmissions.

- IIOT: retransmissions are prioritized based on LCH priorities, same as new transmissions:

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels with data available that are multiplexed or can be multiplexed in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2.

Observation 9: R16 MAC specification currently captures inconsistent behaviors regarding the prioritization of retransmission grants when both cg-RetransmissionTimer and lch-BasedPrioritization are configured.

HARQ sharing across CG configurations

- NR-U: autonomous retransmissions can be performed on a different CG configuration than the initial transmission (Section 5.4.2.2):

Retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH, or on the same resource and with the same MCS as was used for last made transmission attempt within a bundle, or on stored configured uplink grant resources and stored MCS when cg-RetransmissionTimer is configured. Retransmissions with the same HARQ process may be performed on any configured grant configuration if the configured grant configurations have the same TBS.

- IIOT: autonomous transmissions assume no HARQ process sharing across CG configurations, per the RAN2#109-e agreements and associated MAC specification:

- UE autonomous transmission uses the same HARQ process and the same CG configuration. No change to the current running CR. - A HARQ process cannot be shared between different CGs.

NOTE 5: If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

This ambiguity could be solved if it is clarified that HARQ sharing across CG configurations is supported for NR-U’s autonomous retransmissions but not for IIOT’s autonomous transmissions. We propose a R16 CR in [5] clarifying that the above highlighted text only applies when cg-RetransmissionTimer is configured.

Observation 10: It could be clarified in R16 MAC specification that HARQ sharing across CG configurations is supported for NR-U’s autonomous retransmissions only.

Conclusion on ambiguous behaviors

From observations #5, 8, 9, it appears that the current R16 MAC specification reflects contradictory and ambiguous behaviors regarding CG autonomous (re)transmissions preventing from safely configuring cg-RetransmissionTimer and lch-basedPrioritization/autonomousTx altogether.

Proposal #2: RAN2 confirms that R16 MAC specification requires changes and clarifications to remove any contradiction and ambiguity on UE behavior when both cg-RetransmissionTimer and lch-basedPrioritization are configured.

Conclusion

This contribution discussed in details the UE behavior when both IIOT and NR-U are configured altogether in R16, specifically cg-RetransmissionTimer, lch-basedPrioritization and autonomousTx. The resulting observations and proposals are as follows:

Observation 1: The pending PDU following an LBT failure is handled by NR-U’s autonomous retransmission.

Observation 2: The pending PDU of a deprioritized configured grant is handled by NR-U’s autonomous retransmission.

Observation 3: In case of retransmission failure, further retransmissions are also handled by NR-U’s autonomous retransmissions until configuredGrantTimer expires.

Observation 4: In case of consecutive retransmission failures, (IIOT) autonomous transmission doesn’t help in continuing more attempts in transmitting the (still) pending PDU when (NR-U) autonomous retransmission reaches its limit (due to configuredGrantTimer expiry).

Observation 5: Deprioritized grants successfully received by gNB are not further addressed by NR-U retransmissions but could be transmitted twice by IIOT autonomous transmission.

Observation 6: The pending PDU following an LBT failure on a retransmission is handled by NR-U’s autonomous retransmissions up to configuredGrantTimer expiry.

Observation 7: In case of consecutive LBT failures, (IIOT) autonomous transmission doesn’t help in continuing more attempts in transmitting the (still) pending PDU when (NR-U) autonomous retransmission reaches its limit (due to configuredGrantTimer expiry).

Observation 8: (IIOT) autonomous transmissions “replace” the (NR-U) autonomous retransmissions on deprioritized CGs when NR-U is configured for traffic types configured to support “immediate new transmission on CG”. But that is not the expected behavior for such traffic type.

Proposal #1: RAN2 confirms that, in R16, NR-U autonomous retransmission feature can already address pending PDUs of deprioritized configured grants, and configuring IIOT’s autonomousTx on top is both useless and can lead to unexpected behavior in some configurations.

Observation 9: R16 MAC specification currently captures inconsistent behaviors regarding the prioritization of retransmission grants when both cg-RetransmissionTimer and lch-BasedPrioritization are configured.

Observation 10: It could be clarified in R16 MAC specification that HARQ sharing across CG configurations is supported for NR-U’s autonomous retransmissions only.

Proposal #2: RAN2 confirms that R16 MAC specification requires changes and clarifications to remove any contradiction and ambiguity on UE behavior when both cg-RetransmissionTimer and lch-basedPrioritization are configured.

Reference

- R2-2008124	 “Session minutes for NR-U, Power Savings, NTN and 2-step RACH”; Session chair (InterDigital)

- TS 38.321 v16.2.1 “NR; Medium Access Control (MAC) protocol specification (Release 16)”; 3GPP

- R2-2008629 38331 CR#1976 rev2 “Miscellaneous corrections for NR-U”, Qualcomm Inc.

- R2-2007884 “Support of IIoT on Unlicensed spectrum”; LG Electronics

- R2-2008858 “Corrections on autonomous retransmissions”; CATT