3GPP TSG-RAN WG2 #112-e	TDoc R2-2008882

Electronic, November 2nd – 13th, 2020

Agenda Item:	8.5.4

Source:	Ericsson

Title:	RAN enhancements based on new QoS related parameters

Document for:	Discussion, Decision

# Introduction

This paper addresses the below objective in the WID

- RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

We first present two necessary clarifications for these parameters and then discuss how RAN can utilize these parameters.

# Background

The Central network controller (CNC) in TSN computes and schedules the end-to-end transmission for each TSN stream and then transfers the computed schedule to each TSN bridge (e.g. a 5G network acting as a TSN bridge). As part of the schedule computation, the CNC provides each TSN bridge with information that allows for forwarding packets (ethernet or IP) to the next appropriate TSN bridge according to an acceptable time schedule using an appropriate reception time interval.

Issue 1: The concept of TSN stream aggregation allows a 5G network (e.g. from a TSN AF) to identify and aggregate a set of TSN streams received within a given time interval (i.e. the time from the start of a TSN gate open until gate close) that have the same traffic class and have a compatible periodicity (e.g., can be used for transmission over the radio interface using a common MAC PDU). The aggregation process results in the 5G network identifying TSCAI information (e.g. Burst Arrival time and Periodicity, see Table 1 below) and QoS flow characteristics that are representative of the aggregated set of TSN streams. gNB uses this representative information to allocate resources suitable for transmitting the set of aggregated ethernet frames.

Issue 2: TS 22.104 defines Survival Time (ST) as “the time that an application consuming a communication service may continue without an anticipated message.”. However, even with this definition of ST there is still a need for RAN2 to identify the possible options for when the ST period begins. In addition, SA2 has discussed two interpretations of ST and it seems that one of them is sufficient for RAN2. This can be further addressed upon reception of the corresponding LS from SA2.

# Discussion

## 3.1	New TSCAI Attribute: Burst End Time

In RAN2#105, RAN2 sent a LS to SA2 (R2-1902354) indicating that the knowledge of a TSN traffic pattern is useful for the gNB to allow it to more efficiently schedule traffic either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. SA2 has correspondingly specified TSCAI in TS 23.501.  In this paper we discuss the benefits, from a RAN perspective, to be realized by supplementing burst arrival time (BAT) with a new TSCAI parameter referred to as Burst End Time (BET). Reception of BET allows a gNB to determine the latest point in time where UE or gNB can receive a packet for inclusion in the next instance of a radio resource (uplink or downlink respectively) used to support transmission of packets associated with a given QoS flow. Note that, this issue was discussed previously (see question 12 in [4]) but not concluded yet. In this section, we have updates based on the comments provided in that email discussion.

One key TSCAI attribute of value is the Burst Arrival time (BAT, see Table 1).

Table 1 - TSCAI information (see section 5.27.2 in 23.501 V16.6.0)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two bursts. |
| Burst Arrival time | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). |

A gNB uses Burst Arrival time to determine when UE/gNB will first receive payload (e.g. an uplink/downlink ethernet frame) for inclusion in the next instance of the radio resource used in support of the corresponding TSN stream. In the remaining discussion below, we focus only on the case of downlink traffic and it can be noted that the same applies for the uplink traffic arrival at UE.

BAT therefore serves as the earliest point in time at which the gNB can begin accumulating packets for inclusion in a MAC PDU to be sent using the next instance of the corresponding DRB resource. It will be beneficial for TSCAI to be supplemented with a Burst End Time (BET) parameter indicating the time when the gNB can expect to receive the last packet to be transmitted using the next instance of the corresponding DRB resource (i.e. BET = the right side of the blue rectangle in Figure 2).  This is in particular useful in the case of TSN stream aggregation in which there are multiple packet arrivals within the burst.

Figure 1 MAC PDU Assembly and Transmission Time

Burst Period

BAT

…

One or more MAC PDU transmissions

Time Available for MAC PDU assembly and transmission

BET

- The key benefit of BET is that it provides the gNB with enhanced flexibility regarding where it can schedule the DRB resources needed for supporting a given QoS flow while satisfying the PDB requirement. Without BET a gNB will not know the point in time when the last packet will arrive which forces it to identify a latest point in time (i.e. a “drop dead time”) for accepting packets to be aggregated into the next MAC PDU to be sent, e.g., using a periodic DL SPS configuration or in a dynamic resource allocation used for the corresponding QoS flow. Assuming a “drop dead time” for BET can substantially reduce the flexibility regarding where a gNB can schedule the corresponding DRB resources and thereby reduce overall efficiency in managing radio resources.

- The value of BET can be determined e.g. using the width of the TSN gate open cycle during which the UPF/NW-TT receives the set of packets subject to aggregation and therefore enhancing TSCAI to include BET seems to be a feasible option for the CN.

  - Note that the meaning of the time interval between BAT and BET is different from that of “burst spread” as discussed in SA2, which is defined as the “variation of burst arrival time for DL traffic resulting from jitter on N6, if applicable)” - see “Key Issue #3A: Exposure of deterministic QoS” in [1].  As such the “burst spread” can be seen as a value that extends the time interval between BAT and BET. One example is that the burst spread applies equivalently to the BET and so that BET = BAT + gate-open-duration + burst spread in SA2.

- For the case where TSN stream aggregation is not used, the meaning of BET is still valid as it indicates the point in time when e.g. the only packet of the burst will have arrived at the gNB, thereby allowing for a potentially large width of the TSN gate open cycle and burst spread on N6 interface.

Therefore, in this paper, we propose introducing BET as new TSCAI parameter that allows for more flexible use of available radio resources by the gNB. A corresponding draft LS to SA2 can be found in the Annex A below.

Proposal 1: 	It is beneficial for the RAN to obtain a new Burst End Time parameter as part of TSCAI that indicates the time when it can expect to receive the last packet within a burst.

## 3.2	Clarification on How to Measure Survival Time

TS 22.104 [3] defines Survival Time (ST) as “the time that an application consuming a communication service may continue without an anticipated message.” See Figure C.3-1 in Annex C.3 of the TS 22.104 for an intuitive illustration of the survival time. SA2 has decided that the survival time is a part of the TSC AI [1]. On the exact survial time metric, there are two alternative definitions under discussion in SA2:

- as a maximum time in units of “time” where each unit corresponds to the data burst periodicity defined in TSCAI in Rel-16; or

- as a maximum number of consecutive data burst transmission failures, where a data burst corresponds to a single application message.

Further considering in Section 3.1 that the packets/frames can arrive at any time between BAT and BET, there is a need to clarify how the survival time is measured and at which time the anticipated message is declared as “not received”. Assuming that there is one application-layer message per burst, potentially segmented into multiple packets, and operation for survival time is related to burst as suggested by SA2 by indicating survival time in TSCAI, the main question is whether to consider survival time starting related to beginning or end of the burst, and what is the relation to the PDB of the QoS flow.

SA2 has defined 5G Access Network Packet Delay Budget (5G-AN PDB) for each packet within a QoS flow, and typically it is counted from the time when the packet is received into the PDCP layer of UE or gNB. A packet is considered as lost once its 5G-AN PDB after reception for transmission expires.

Due to the variation of the arrival time of the packets in two different bursts, the time when packet is considered as lost is not consistent in these two bursts. This may lead to that the survival time is wrongly calculated. One example is illustrated as below. It leaves very short time for the packet delivery to meet the survival time, which is even before its PDB. The issue is that the packet in the second burst arrives close to the burst end time while the packet in the first burst arrives close to the burst start time.

Figure 2 Incorrect Survival Time Calculation

To address the above unclarity for RAN, one interpretation is that the survival time is counted from the PDB after the burst end time, see below figure 2. In this way, it accounts for the possible latest arrival time in the subsequent bursts and thus the survival time can make sense for RAN.

Figure 3 Correct Survival Time Calculation

In this approach, the survival time ends after the PDB of the second packet. However, for the delay critical GBR resource type, packets delayed more than the PDB can be discarded or delivered depending on local decision [2]. For a QoS flow provided with survival time, a smart implementation should still deliver the packet up-to the survival time even if after the PDB.

With the above discussion, it is clean to define that a message transmission is declared as failed after the burst end time plus 5G-AN PDB in RAN and survival time starts at the burst end time plus the 5G-AN PDB. Note that a message (with a very large size) can be segmented into multiple packets/frames sent to RAN, see Annex B. Therefore, we propose that

Proposal 2: 	If not all segments of a message is delivered before the time equal to burst end time (BET) + 5G-AN PDB, the message transmission is declared as failed and the survival time starts (if not already).

If all segments of a message are successfully delivered before the time equal to this message’s burst end time (BET) + 5G-AN PDB, then it can be understood that the application is “survived” and the survival time is reset and not appliable anymore till the next time it is triggered by a message delivery failure.

Considering the Proposal 1 and Proposal 2, we propose to send a LS to SA2 to clarify RAN2 understanding.  See a draft in Annex A

Proposal 3:	RAN2 to send SA2 an LS stating

“It is beneficial for the RAN to obtain a new Burst End Time parameter as part of TSCAI that indicates the time when it can expect to receive the last packet within a burst”; and

“If not all segments of a message is delivered before the time equal to burst end time (BET) + 5G-AN PDB, the message transmission is declared as failed and the survival time starts (if not already).”

## 3.3 How RAN uses survival time

Once the survival time (ST) period starts, a RAN implementation may schedule the radio resource more robustly to make sure any subsequent messages can be delivered successfully before the survival time. If the message is successfully delivered, the robust resource allocation is ceased. If a message is not successfully delivered prior to the end of ST period, then a gNB determines that a ST violation has occurred and subsequent gNB action is implementation specific. This is illustrated in the below figure.

Figure 4 How to use survival time at RAN

A robust resource allocation is only needed if previous messages are not successfully delivered while in all other cases, a normal resource allocation is needed. Note that the message failure rate is already a very rare event. For example, standard 5QI value of Delay Critical GBR QoS flows (from 82 to 86) in TS 23.501 [2] has a packet error rate target of 10^-4 or 10^-5. The mechanism of survival time can be one way to ensure an even higher communication service availability target value (for example, 1-10^-9).

Observation 1: 	The start of survival time is a rare event.

One key question is how to trigger the resource allocation shift.  For periodic traffic, gNB is aware of the packet arrival through TSCAI at either UE or gNB and then gNB can observe that the packet is not delivered within the delay budget. After this, gNB can schedule the subsequent packet with high reliability to meet the survival time.

For aperiodic traffic, in the TR of the SA2 study [1], it is clearly mentioned in Section 8.4 that the survival time is conveyed together with TSCAI periodicity parameter (the time between periodic TSC bursts), see below:

## 8.4 Key Issue #5: Use of Survival Time for Deterministic Applications in 5GS - Survival Time is specified by the AF in units of "time" with respect to burst periodicity or as the maximum number of consecutive message transmission failures (i.e. whose loss can be tolerated). It is conveyed together with TSCAI Periodicity parameter (the time between periodic TSC bursts) and burst size (e.g. MDBV).

In other words, the survival time metric in TSC AI does not apply for aperiodic traffic. Thus,

Observation 2: 	gNB knows when the expected message is not received and can schedule the subsequent message with high reliability to meet survival time.

There have been some proposals for UE to indicate the start of the survival time. However, as discussed above, in the case of periodic traffic, there is no such need since gNB knows when the expected message is not received.  For UL traffic, it has been proposed in Rel-16 for UE to autonomously activate PDCP duplication, with conditions configured by the network and related with the survival time. This approach requires to have reserved UL resources all the time for the LCHs with PDCP duplication configured but not activated yet. However, as mentioned in observation 1, the survival time is a rare event, using ONLY for one out of 10^4 to 10^5 transmission is not spectrum efficient. One may argue to use intra-UE feature in this case, but this is an overkill in which there is a clear better alternative of gNB scheduling.

Based on the above discussions, we propose that

Proposal 4: 	How to use survival time is up-to network implementation.

# Conclusion

The following observations and proposals are made:

Proposal 1: 	It is beneficial for the RAN to obtain a new Burst End Time parameter as part of TSCAI that indicates the time when it can expect to receive the last packet within a burst.

Proposal 2: 	If not all segments of a message is delivered before the time equal to burst end time (BET) + 5G-AN PDB, the message transmission is declared as failed and the survival time starts (if not already).

Proposal 3:	RAN2 to send SA2 an LS stating that

“It is beneficial for the RAN to obtain a new Burst End Time parameter as part of TSCAI that indicates the time when it can expect to receive the last packet within a burst”; and

“If not all segments of a message is delivered before the time equal to burst end time (BET) + 5G-AN PDB, the message transmission is declared as failed and the survival time starts (if not already).”

Observation 1: 	The start of survival time is a rare event.

Observation 2: 	gNB knows when the expected message is not received and can schedule the subsequent message with high reliability to meet survival time.

Proposal 4: 	How to use survival time is up-to network implementation.

# References

3GPP TR 23.700-20 V1.0.0, Study on enhanced support of Industrial Internet of Things (IIoT) in the 5G System (5GS), Release 17

TS 23.501, System architecture for the 5G System (5GS), Stage 2

TS 22.104, Service requirements for cyber-physical control applications in vertical domains

R2-2004149, Summary of offline discussion-026- Scheduling Enhancements, CMCC, RAN2#109bis-e

# Annex A – Draft LS to SA2 on TSC-AI parameter clarification

Title:	Draft LS on TSC-AI parameter clarification

Release:	Rel-17

Work Item:	NR_IIOT_URLLC_enh-Core

Source:	[To be replaced with RAN2]

To:	SA2

Contact Person:

Name:	Zhenhua Zou

E-mail Address:	Zhenhua.Zou@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 has agreed that it is beneficial for the RAN to obtain a new Burst End Time parameter as part of TSCAI that indicates the time when it can expect to receive the last packet within a burst.

The new Burst End Time (BET) parameter indicates the time when the gNB can expect to receive the last packet within the burst for the DL traffic and when the UE can expect to receive the last packet within the burst for the UL traffic. This parameter is useful both in case of TSN-stream aggregation and in case of no TSN-stream aggregation. The key benefit of BET is that it provides the gNB with enhanced flexibility regarding where it can schedule radio resources needed for supporting a given QoS flow while satisfying the PDB requirement. It is RAN2’s understanding that the value of BET can be determined by, for example, the width of the TSN gate open cycle and the "burst spread: variation of burst arrival time for DL traffic resulting from jitter on N6” discussed in SA2.

RAN2 has also discussed when the survival time is supposed to start and agrees that if not all segments of a message is delivered before the time equal to burst end time (BET) + 5G-AN PDB, the message transmission is declared as failed and the survival time starts (if not already).  If all segments of a message are successfully delivered before the time equal to this message’s burst end time (BET) + 5G-AN PDB, then the survival time is not appliable anymore till the next time it is triggered by a message delivery failure.

2. Actions:

To SA2 group

ACTION: 	1. RAN2 respectfully asks SA2 to introduce the “Burst End Time” as a new TSCAI parameter to be sent to gNB.

2. When providing survival time, burst arrival time, burst end time as part of the TSC AI parameters to meet the application requirement, RAN2 respectfully asks SA2 to consider the RAN2 agreements on when the survival time starts.

3. Date of Next TSG-RAN2 Meetings:

3GPPRAN2#113-e		Jan 25th to Feb 5th, 2021	Online

3GPPRAN2#113-bis-e		Apr 12th to Apr 20th, 2021	Online

3GPPRAN2#114-e		May 19th to May 27th, 2021	Online

# Annex B – Large message size use case in TS 22.104

(Excerpt of part of) Table 5.2-1: Periodic deterministic communication service performance requirements

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value (note 1) | Communication service reliability: mean time between failures | End-to-end latency: maximum (note 2) (note 12a) | Service bit rate: user experienced data rate (note 12a) | Message size [byte] (note 12a) | Transfer interval: target value (note 12a) | Survival time (note 12a) | UE speed (note 13) | # of UEs | Service area (note 3) | Remarks |
| > 99,9999 % | ~ 1 year | < transfer interval value | – | 15 k to 250 k | 10 ms to 100 ms (note 7) | transfer interval value | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 | Mobile robots – video-operated remote control (A.2.2.3) |

NOTE 12a:	It applies to both UL and DL unless stated otherwise.

The message size is 15 k to 250 k bytes and cannot be fit into one Ethernet frames or IP packet