3GPP TSG-RAN WG2 Meeting #112e	R2-2008945

Electronic, 02nd – 13th November 2020

Agenda Item:	8.1.2.3

Source:    TCL Communication Ltd.

Title:          Reliability and latency handling during NR multicast mobility

Document for:	Discussion and Decision

# Introduction

In RAN Meeting #88-e a new working item was approved WID [1]  targeting a RAN support of Multicast/ Broadcast Services (MBS) in R17.One of the key objective of this WID is to study and

- Specify the support for basic mobility with service continuity [RAN2, RAN3].

During RAN2 meeting #111e meeting some agreements have been achieved on Rel-17 NR multicast mobility related issues. These agreements include:

- Requirements for lossless mobility are TBD. Assume for now that RAN2 will anyway discuss service continuity functionality for low or no data loss.

- R2 assumes that for Rel-17 NR multicast Mobility in Connected mode, handover (including variants) is the baseline, TBD exactly which variants.

Based on the above agreements, we can observe that the most focus regarding service continuity functionality NR multicast mobility is given to the reliability aspect. However, given the fact that some MBS use cases targeted in NR mobility (e.g., V2X and some industrial applications such as motion sensors and actuators) which may also involve MBS reception during inter-RAN mobility which requires very low service interruption delay on the top of their reliability requirements. Therefore, its necessary to discuss both the reliability and latency aspects of NR multicast mobility in RAN2 for efficient MBS service continuity functionality. In this contribution, we discuss this issue in more details and propose some enhancements in RAN2 that could help addressing the issue.

# Discussion

New radio (NR) MBS is targeting variety of applications and use cases such as public safety and mission critical, V2X applications, transparent IPv4/IPv6 multicast delivery, IPTV, software delivery over wireless, group communications and IoT applications. Most of these use cases involve MBS service reception during inter-RAN mobility, e.g. public safety, V2X applications and so on. Most of the use cases that may involve MBS service reception during inter-RAN mobility require high reliability as shown in Table .1[2]. For example, V2X, applications require 90% to 99.9999% level of reliability, while public safety and some industrial applications require a reliability level up to 99.9999%. On the other hand,  some MBS use-cases such as V2X and some industrial applications  (e.g., motion related sensor such as industrial actuators) which may also involve MBS reception during inter-node (RAN) mobility require very low service interruption time due to their very strict delay requirement (i.e. 5ms to 0.5ms, see also Table .1). While most of the proposals discussed during the last 3GPP RAN2 meeting suggest to adopt /reuse the legacy NR unicast handover with some lossless feature to support NR multicast mobility, it is quite obvious that adopting a handover type with only lossless feature may not be able to fulfil the QoS requirements for some MBS use-cases targeted in R17.The upcoming section of this contribution presents some enhancements in RAN2 to address this problem.

Table 1. Requirements for different MBS use cases

| MBS use cases | Latency | Reliability |
| --- | --- | --- |
| V2X | 5-100ms variable | 90% to 99.9999% |
| Live Video | 150ms | 99.9% |
| IOT, Software update | Latency Tolerant | Higher reliability is beneficial |
| Industry applications | 0.5ms | 99.9999% |

# Enhancements

In this section, we present RAN enhancements to support NR multicast mobility as observations and proposals as follows:

Observation 1:    In the framework of NR multicast mobility, the current focus in RAN2 is to reuse the legacy NR unicast handover with lossless features to support a reliable MBS mobility service continuity. However, given the fact that some MBS use cases (e.g., V2X and some industrial applications such as motion sensors and actuators) require very low service interruption delay on the top of their reliability requirements, a handover with additional feature on the top of lossless feature (e.g., lossless and low service interruption delay features [3]) may also need to be supported for efficient MBS mobility with service continuity of some NR MBS uses-cases (See Table 2). Therefore, we kindly ask:

- Proposal 1: RAN2 discuss the support of handover types with lossless feature and/or with lossless and low-latency features for NR multicast mobility.

Table 2. Handover type based on MBS use cases

| MBS use cases | Latency | Reliability | Handover Type |
| --- | --- | --- | --- |
| V2X | 5-100ms variable | 90% to 99.9999% | Low-latency and Lossless |
| Live Video | 150ms | 99.9% | Lossless |
| IOT, Software update | Latency Tolerant | Higher reliability is beneficial | Lossless |
| Industry applications | 0.5ms | 99.9999% | Low-latency and Lossless |

Observation 1:    In an MBS scenario where an MBS capable UE moving from one NG-RAN to another (Fig.1). If the UE is receiving an MBS service that requires high or moderate level of reliability but it is a latency tolerant service (e.g., live video and software update), it would be more beneficial (in term of handover complexity reduction) if the RAN node has a capability to determine the handover features to be configured for the given UE based on the knowledge of UE service interest, i.e., configuring only a reliable handover for the given UE instead of configuring a handover type to guarantee both low-latency and reliability features.

Control

Data

UE1

Target RAN

UE1

Core Network

i.e. identities

Data

Control

Uu

Uu

Target RAN

Control

Data

Figure 1:  UE inter-RAN mobility from one NG-RAN to another

Therefore, we kindly ask:

- Proposal 2:  RAN2 agrees that the selection of handover type or the handover features to be configured for a UE to be based on UE service interest or the QoS requirements information of interested service.

# Conclusion

In this contribution, we discussed the issue of handling both the reliability and latency during NR multicast mobility and propose some RAN enhancements proposals to address it. These proposals are summarized as follows:

- Proposal 1: RAN2 discuss the support of handover type with lossless feature and/or with lossless and low-latency features for NR multicast mobility.

- Proposal 2: RAN2 agree that the selection of handover type or the handover features to be configured for a UE to be based on UE service interest or the QoS requirements information of interested service.

# References

- RP-201308, “NR Multicast and Broadcast Services”, Huawei, HiSilicon, RAN#88e, E-meeting, June 29 - July 3, 2020

- R1-2005250,”Mechanisms to improve reliability for RRC_CONNECTED UEs”, Huawei, HiSilicon, RAN#102,E-meeting , August 17th - 28th August, 2020

- R2-2009320, “Discussion on the support of reliable and low-latency NR multicast mobility”, TCL Communication, RAN WG2 Meeting #112e, Electronic, 02nd – 13th November 2020