3GPP TSG-RAN WG2 Meeting #112-e                                                     R2-2008957

Electronic, 02 November – 13 November 2020

Source:	CATT

Title:	Discussion on Paging with Service Indication

Agenda Item:	8.3.4

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, a new WI “Support for Multi-SIM devices in Rel-17‎” was approved [1]. One of the objectives is to specify the mechanism for an incoming page to ‎indicate to the UE whether the service is VoLTE/VoNR. During the email discussion#917 [2], based on the overhead analysis, most of companies agree it is feasible to have paging cause on Uu for EPS and 5GS from RAN2 point of view.

The following issues identified by the email discussion#917 which can be further discussed.

- Issue 1: Whether will the increasing of paging cause impact the real deployment about paging volume and coverage‎?

- Issue 2: What is the granularity of paging cause expected to be supported in RAN nodes?

In this contribution, we will discuss the above issues and give our preference.

Discussion

Paging with Service Indication

This objective is associated with Key Issue#1 in TR 23.761[3]. There are several solutions to address this Key Issue#1 as following.

- Solution #1: Paging Cause in the Uu Paging message

  - UE receives paging message with the Paging Cause information.

- Solution #9: Handling of MT IMS voice service with ‎different Paging Cause

  - In addition of Solution#1, UE provides a preferred list and/or a non-preferred list of MSISDN or tel/sip URI to TAS via Ut interface.

- Solution #10: Network-based paging filtering

  - Network corresponding to USIM-1 sends paging for USIM-1 only if Paging Filtering Rules in the network allows paging the UE.

- Solution #11: Operator-defined upper bound timer for ‎paging response

  - This solution allows the operator to have a deterministic behaviour from the UEs by providing them with an upper bound timer for paging response.

For Solution#1 and Solution#9, the main impact is to add paging cause in the paging message. Thus Solution#1 and Solution#9 are simpler solutions. For Solution#10 and Solution#11, there should be some impact on RAN and the method is more complicated than adding paging cause in the paging message.

Observation 1: The solution to add paging cause in the paging message is simpler.‎

However, this objective is mainly impact Core Network. During the email discussion, all the companies agree that paging cause should be specified in RAN2 after SA2 progress.‎ We also suggest to wait SA2 conclusion before discussion this issue in RAN2, which is in line with WID scope.

Proposal 1: ‎Suggest RAN2 to discuss the paging cause after SA2 progress‎.

Impact on the real deployment about paging volume and coverage‎

During the email discussion#917 [2], based on the rapporteur’s analysis and companies inputs, there is an observation that if the paging cause (e.g., 3 bits per UE) is added, the paging message size is generally increased by ~6% for E-UTRA ‎and ~8% for NR.‎

Currently, there are 32 paging records are available. If the paging cause (e.g., 3 bits per UE) is added, we can assume 2 or 3 paging records can be reduced, so that at least 29 paging records are available with paging cause. Thus, we think the paging volume isn’t much reduced by the paging cause bits. ‎

Proposal 2: ‎The impact on paging volume is acceptable when the paging cause (e.g., 3 bits per UE) is added‎.

Regarding to paging coverage, it should be in RAN1 scope. Thus, we suggest to send LS to RAN1 to ask whether there is any impact on paging coverage if paging message size is generally increased by ~6% for E-UTRA ‎and ~8% for NR.‎

Proposal 3: Suggest to send LS to RAN1 to ask whether there is any impact on paging coverage if paging message ‎size is generally increased by ~6% for E-UTRA ‎and ~8% for NR.‎

Granularity of paging cause

During the email discussion#917 [2], the granularity of paging cause has been discussed. There are several options as following:

- Option A: Per PLMN;

- Option B: Per TA;

- Option C: Per Ran Node;

- Option D: Per Cell;

- Option E: Other.

Most companies agree the paging cause should be at least per PLMN. However, we think the granularity of paging cause depends on what kinds of paging cause specified by SA2. Thus, it’s necessary to also discuss this issue in SA2. RAN2 can discuss this issue after SA2 progress on paging cause.

Proposal 4: ‎Suggest RAN2 to discuss the granularity of paging cause after SA2 progress‎.

Conclusion

According to the above discussion, the observations and proposals for the issues the mechanism for an incoming page to ‎indicate to the UE are as follows:

Observation 1: The solution to add paging cause in the paging message is simpler.‎

Proposal 1: ‎Suggest RAN2 to discuss the paging cause after SA2 progress‎.

Proposal 2: ‎The impact on paging volume is acceptable when the paging cause (e.g., 3 bits per UE) is added‎.

Proposal 3: Suggest to send LS to RAN1 to ask whether there is any impact on paging coverage if paging message ‎size is generally increased by ~6% for E-UTRA ‎and ~8% for NR.‎

Proposal 4: ‎Suggest RAN2 to discuss the granularity of paging cause after SA2 progress‎.

Reference

- RP-193250‎‎, New WID: Support for Multi-SIM devices in Rel-17‎.

- R2-2009325,‎ ‎[Post111-e][917][Multi-SIM] Multi-Sim, vivo.‎

- TR 23.761, V1.0.0‎, Study on system enablers for devices having multiple ‎Universal Subscriber Identity Modules (USIM).