3GPP TSG-RAN WG2 Meeting #112e	R2-2009115

Online, 2nd – 13th November 2020 	Revision of R2-2007492

Agenda Item:	8.12.2.1

Source:	MediaTek Inc.

Title:	On the definition of a RedCap device type

Document for:	Discussion and decision

# 1 Introduction

The SID on reduced capability (RedCap) devices [1] includes the following objective for study:

Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases.

Email discussion Post111-e#913 [2] triggered a discussion on the definition of a RedCap UE. In this contribution, we expand in some detail, those aspects that warrant further discussion.

# 2 Discussion

The UE capability framework in NR is significantly different from LTE. In LTE, UE categories were used to associate a set of features with an identifier. These identifiers were signalled to the network to indicate the minimum set of capabilities that a UE supported. In NR however, the UE’s support of features are individually signalled to the network. This framework is a sensible approach for NR, in the context of being one technology that can be deployed in several scenarios (eMBB, URLLC and so on). NR needs to be flexible to allow for operation in deployments with differing requirements. However, along with this flexibility in the framework comes the drawback that it is ill-suited to define a device type as indicated in the SI objective.

Observation 1: The NR UE capability framework is very flexible resulting in it being ill-suited to define a device type.

## 2.1. The importance of limiting the set of RedCap device types

With the work on RedCap, 3GPP is attempting to allow NR to address market segments where extremely high data rates are not important. These market segments include wearables such as smart watches, factory sensors such as temperature monitors, and other connected devices such as cameras. The data requirements of these devices lie somewhere between eMBB devices (e.g. smartphones) and mMTC devices (e.g. power meters), and the natural expectation is that RedCap device cost will correspondingly lie somewhere between the two.

While smartphones are extremely capable computing devices, they are made affordable by leveraging economies of scale: global smartphone sales are now over 1.5 billion devices annually [3]. If we were to re-use the existing flexible NR framework for RedCap, we could define bespoke NR UEs tailored to reduce component costs for each market segment that it addresses. However, the downside with such an approach is that we would end up fragmenting the RedCap market, resulting in a loss of economies of scale, which in turn negates all component cost optimisations. This is the reason why it is important to limit the set of RedCap device types – ideally to just one.

Observation 2: For truly meaningful cost reductions with RedCap, market fragmentation must be avoided to preserve economies of scale.

Observation 3: Market fragmentation is avoided by limiting the set of RedCap device types.

This aspect has been acknowledged by 3GPP, and the following was agreed in R2#111e [4], i.e. that device types will only be introduced if there is a need identified by RAN1 to differentiate them at initial access.

The number of device types should be minimised, to reduce market fragmentation, and introduced only where essential to control UE accesses and differentiate them from legacy R15/R16 and non-Redcap R17 UEs, (e.g. number of Tx/Rx antennas, maximum supportable BW, etc.). The exact composition of the set of L1 capabilities of the device type can be discussed by RAN1

## 2.2 Definition of a RedCap device type

The other SI objectives aim to reduce the RedCap device cost by reducing its component costs. These include reducing the UE’s operating bandwidth, number of antennas supported and so on. As an example, the bandwidth that a UE is mandated to support in Rel-15 for band n41 is 100MHz. This is the baseline required of all NR devices in Rel-15 and being a minimum capability expected of all NR devices, it is not signalled as part of UE capabilities. For a Redcap UE, the bandwidth requirement is expected to be reduced down to 20MHz. This introduces a new (lower) baseline requirement for NR devices in Rel-17. The set of features in RedCap that reduce mandatory minimum capabilities of a legacy NR device can be grouped together to define a RedCap UE device type.

Such a device type definition would indicate the new mandatory minimum capability set expected of an NR device. Having a baseline device type helps avoid market fragmentation if it is defined such that it can work for all market segments it is intended for. Additionally, having a single lower minimum capability set helps ease network deployment as the new common baseline can be used for planning network coverage and capacity.

At R2#111e, it was identified that there is a need for an indication in system information to control whether RedCap devices can camp on a cell or not. Email discussion Post111-e#914 [5] also discusses the need for early identification of a RedCap UE to the network. Having a device type definition (associated with a new mandatory minimum capability set) as discussed above can be helpful in simplifying the signalling design needed to support a network indication as well as for early device type identification.

Proposal 1: The set of RedCap features that reduce mandatory minimum capabilities of a legacy NR device are grouped together to define a RedCap UE device type.

# 3 Conclusion

In this contribution we observe that:

Observation 1: The NR UE capability framework is very flexible resulting in it being ill-suited to define a device type.

Observation 2: For truly meaningful cost reductions with RedCap, market fragmentation must be avoided to preserve economies of scale.

Observation 3: Market fragmentation is avoided by limiting the set of RedCap device types.

Based on the observations above, we propose:

Proposal 1: The set of RedCap features that reduce mandatory minimum capabilities of a legacy NR device are grouped together to define a RedCap UE device type.

# 4 References

- RP-201386 - Revised SID on Study on support of reduced capability NR devices (Ericsson)

- Report of [Post111-e][913][REDCAP] Definition and constraining of reduced capabilities (Intel)

- https://www.statista.com/statistics/263437/global-smartphone-sales-to-end-users-since-2007/

- R2-2008701 - RAN2#111-e Meeting Report

- Summary of email discussion 914 on UE identification and access restrictions (Huawei)