3GPP TSG-RAN WG2 Meeting #112 electronic 	R2-2009128

Online, November 2 - 13, 2020

Agenda Item:	8.1.2.2

Source: 	MediaTek Inc.

Title:  	NR MBS Radio Bearer Structure

Document for:	Discussion and decision

# Introduction

An email discussion was taken to discuss the L2 architecture of NR MBS Radio Bearers. This document discusses the detailed structure of the MBS Radio Bearer (MRB).

# Terminology

Based on the related discussion at SA2 for 5G MBS, there are two delivery methods for the transmission of MBS packet flows over radio:

- Point-to-Multipoint (PTM) delivery method: a RAN node delivers a single copy of MBS data packets over radio to a set of UEs.

- Point-to-Point (PTP) delivery method: a RAN node delivers separate copies of MBS data packet over radio to individual UE.

The RAN node (i.e. gNB) may use PTM, PTP or a combination of PTP/PTM mode to deliver the MBS data of a particular MBS service to the interested UEs within a cell. The support of the simultaneous PTP/PTM delivery method is to cater for the diverse handling for the UEs for the MBS service reception e.g. radio resource utilization scheme, different QoS requirements.

Proposal 1:  The simultaneous PTP/PTM delivery method is supported for NR MBS data transmission.

In addition, taking account of the requirement to support reliable multicast transmission, a companion PTP delivery leg may be used to perform UE specific retransmissions.  Then from the RAN node perspective, there are four delivery modes available to deliver MBS data over the air interface:

- PTM only transmission

- PTP only transmission

- PTM and PTP (with PTP for initial transmission)

- PTM and PTP (with PTP only for retransmission)

It should be noted that when the RAN node configures both PTM leg and PTP leg for MBS data delivery for a particular MBS session, some UEs may only receive the PTM leg, some UEs may only receive PTP leg, and the other UEs may receive both PTM and PTP legs.

Proposal 2:  RAN2 take the following four delivery modes to support the transmission of NR MBS data:

- PTM only transmission

- PTP only transmission

- PTM and PTP (with PTP for initial transmission)

- PTM and PTP (with PTP only for retransmission)

# Options for MRB structure

According to the discussion at the last RAN2 meeting, both HARQ layer feedback and L2 feedback were discussed to ensure reliable MBS data transmission. However the actual L2 feedback may be at RLC layer or PDCP layer depending on the exact protocol stack for MRB structure. This section describes two options for MRB structure, RLC based MRB structure and PDCP based MRB structure.

## RLC based MRB structure

In Figure 1, RLC layer based protocol stack for MRB transmission is illustrated. In the network side, PTM transmission mode is used for downlink MBS transmission and its RLC transmission mode is in UM mode. In network side, there is only one compact RLC entity established per MRB (serving all of the UEs) for reliable MBS transmission. The PDCP entity at network side delivers the PDCP packets to the compact RLC entity following legacy procedure.

The compact RLC entity is tasked to perform both PTM transmission and PTP transmission. A new Transmission Mode (or cast type) is introduced for this compact RLC entity i.e. PTM mode and/or PTP mode (or multicast type and/or unicast type) on top of the concept of RLC TM/UM/AM mode. The compact RLC entity runs in PTM mode (or multicast type) for initial PTM transmission and/or PTM Retransmission. Any transmission in PTP mode (or unicast type) is for PTP Retransmission. Initial PTM transmission or PTM Retransmission of data is multicast/broadcast to multiple UEs using G-RNTI via a specific logical channel (i.e. LCH). The LCH for PTM transmission can be MTCH as defined by legacy system. PTP retransmission if needed is unicast to the UE using C-RNTI via a unicast LCH (i.e. DTCH). As such, one compact RLC entity corresponds to multiple logical channels.

Figure 1: RLC based MRB structure

As depicted in Figure 1, at UE side, there is single/combined protocol stack established for the reception of both PTM transmission and PTP transmission for a particular MRB. It means the UE establishes a single DRB with a single RLC entity (in RLC AM mode) and a single PDCP entity for the MRB. UE monitors two independent LCHs (one for PTM data and the other for PTP data) via different RNTIs. UE assembles the data packets from two independent LCHs at RLC assuming the SN is aligned. UE performs packet reordering, packet redundant detection and packet discard at RLC layer. In uplink, UE provides the uplink feedback (i.e. RLC status report) using C-RNTI to the network.

For the MRB protocol stack as depicted in Figure 1, one transmission window is maintained at gNB for each UE based on its specific retransmission buffer within the compact RLC entity. Each UE specific retransmission buffer within the compact RLC entity (at network side) buffers the RLC packets when they are received from RLC SN allocation function. RLC entity of UE (at UE side) provides RLC status report to network. Each UE specific retransmission buffer within the compact RLC entity (at network side) removes the RLC packets when positive packet status report is received.

Proposal 3:  RAN2 agree that the RLC layer based MRB structure are featured by:

- An associated RLC leg is established for UE specific PTP retransmission

- A compact RLC entity is established supporting both PTM and PTP to serve all UEs

- The compact RLC entity corresponds to multiple logical channels at DL

- UE establishes a single RLC entity to receive both PTM and PTP

## PDCP based MRB structure

Figure 2: PDCP based MRB structure

In Figure 2, PDCP layer based compact protocol stack for MBS RB transmission is illustrated. The RLC feedback function block and retransmission function block is moved to PDCP layer, as compared to Figure 1 (RLC layer based approach). The RLC layer runs in UM mode for PTM transmission at network side. In network side, there is only a compact PDCP entity per MBS RB (shared by all UEs) for reliable MBS transmission.

In network side, there are a PTM RLC entity and one or more UE specific RLC entity (with one per each UE). The SDAP entity delivers the packets to the compact PDCP entity following legacy procedure. The compact PDCP entity is tasked for both PTM transmission and PTP transmission. A new Transmission mode is introduced for the compact PDCP entity (PTM mode or PTP mode). The compact PDCP entity runs in PTM mode for initial PTM transmission (and potential PTM Retransmission). Any transmission in PTP mode is for PTP Retransmission. Initial PTM transmission and/or PTM Retransmission of data is multicast/broadcast to multiple UEs using G-RNTI via specific LCH (i.e. MTCH). PTP retransmissions if needed is unicast to the UE using C-RNTI via a unicast LCH (i.e. DTCH).

At UE side, there is single/combined protocol stack (i.e. a single DRB) established for the reception of the PTM transmission and PTP transmission. UE monitors two independent LCHs (one for PTM data and the other for PTP data) via different RNTIs. UE establishes one RLC entity for each LCH at RLC layer: one for PTP, and the other for PTM. The RLC entity for PTM reception runs in RLC UM mode. The RLC entity for PTP reception runs in RLC AM mode. UE assembles the data packets received from two independent RLC entities at PDCP assuming the SN is aligned, as the SN is supposed to be allocated by a single PDCP SN allocation function block at network side. In uplink, UE provides the uplink feedback (i.e. PDCP status report) using C-RNTI.

The compact PDCP entity at gNB maintains independent retransmission buffer for each UE. One transmission window is maintained at network side per each UE via UE specific retransmission buffer within the compact PDCP entity. Each UE specific retransmission buffer at network side buffers the PDCP packets when they are received from PDCP SN allocation function block. The PDCP entity of UE (at UE side) provides PDCP status report to network. Each UE specific retransmission buffer (at network side) removes the PDCP packets when positive packet status report is received.

Proposal 4:  RAN2 agree that the PDCP layer based MRB structure are featured by:

- An associated PDCP leg is established for UE specific PTP retransmission

- A compact PDCP entity is established supporting both PTM and PTP to serve all UEs

- One PTM RLC entity and UE specific RLC entity are established

- PDCP layer based feedback and retransmission is supported

## Evaluation

RLC based MRB structure allows the network to establish a combined RLC leg for PTP/PTM transmission to the UE, which presents efficient resource utilization since the PTP leg runs with PTM in a coordinated manner taking the advantage of the RLC feedback. For example, only missing packets is transmitted by PTP leg.  So then the highest RLC combination gain can be expected and achieved.  This MRB can achieve unicast-alike reliable performance. However it requires to standardize a new RLC model to support the coordinated PTP/PTM transmission mode.

PDCP based MRB structure allows the network to establish a combined PDCP leg for PTP/PTM transmission to the UE, which presents efficient resource utilization since the PTP leg runs with PTM in a coordinated manner taking the advantage of the PDCP feedback. PDCP based MRB structure works as same as RLC based MRB structure but the transmission reliability is ensured at PDCP layer. This MRB can also achieve unicast-alike reliable performance. However it requires to standardize a new PDCP model to support the coordinated PTP/PTM transmission mode.

Proposal 5:  RAN2 selects RLC layer or PDCP layer based MRB structure dependent on the preferred standard impact at RLC layer or PDCP layer.

# Conclusion

The following proposals are made:

Proposal 1:  The simultaneous PTP/PTM delivery method is supported for NR MBS data transmission.

Proposal 2:  RAN2 take the following four delivery modes to support the transmission of NR MBS data:

- PTM only transmission

- PTP only transmission

- PTM and PTP (with PTP for initial transmission)

- PTM and PTP (with PTP only for retransmission)

Proposal 3:  RAN2 agree that the RLC layer based MRB structure are featured by:

- An associated RLC leg is established for UE specific PTP retransmission

- A compact RLC entity is established supporting both PTM and PTP to serve all UEs

- The compact RLC entity corresponds to multiple logical channels at DL

- UE establishes a single RLC entity to receive both PTM and PTP

Proposal 4:  RAN2 agree that the PDCP layer based MRB structure are featured by:

- An associated PDCP leg is established for UE specific PTP retransmission

- A compact PDCP entity is established supporting both PTM and PTP to serve all UEs

- One PTM RLC entity and UE specific RLC entity are established

- PDCP layer based feedback and retransmission is supported

Proposal 5:  RAN2 selects RLC layer or PDCP layer based MRB structure dependent on the preferred standard impact at RLC layer or PDCP layer.

# References

[1]	3GPP TS36.331

[2]	3GPP TS36.300

[3]	RP-193248 New Work Item on NR support of Multicast and Broadcast Services