3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2009159

Elbonia, 02 – 13 November 2020

Agenda item:	5.4.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification to usage of MN and SN configuration restrictions

WID/SID:	NR_NewRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

During the RAN2#111e post-meeting email discussion [Post-111e][251], it seemed that the Rel-15 principles of using the MN/SN configuration restructions were not clear. In this contribution, we try to clarify how the ConfigRestrictMCG and ConfigRestrictModReqSCG are currently specified and should be used.

# 2	MN/SN configuration for MR-DC

## 2.1	MN/SN resource coordination for MR-DC

The MN/SN resource coordination in MR-DC is done via the inter-node messages, and the procedures using those can be either MN- or SN-intiated. These are:

- MN-initiated procedures: SN addition request, SN modification request, SN release request

- SN-initiated procedures: SN modification required, SN release required, SN Counter Check Request

There are also some other auxiliary processes, but these are the ones that do utilize the INM defined in NR RRC.

Observation 1: Both MN and SN can initiate procedures involving exchange of RRC INM.

For MN-intiated processes First MN determines the relevant parts of its own configuration and processes the SN configuration restruictions based on overall UE capabilities. These are then communicated to SN via inter-node message, and SN will accept or reject the MN configuration. When accepting the configuration, SN may also inform MN of its chosen configuration to assist in MN RRM functions, and MN may also still adapt its own chosen configuration within the UE capability limits (e.g. to align DRX or RRM measurement configurations). However, SN cannot at this time start a "negotiation" process wherein it indicates to MN that is "conditionally accepts" the request if MN would just "modify" its parameters: Only "accept" or "reject" are allowed, and any response based on those must comply with the MN configuration restrictions.

Observation 2: The Rel-15 MR-DC principle is that SN (or MN) must always either accept or reject the MN (or SN) request. No negotiation is supported.

To do this, the INM used are the CG-ConfigInfo (MN-->SN) and CG-Config (SN-->MN), which can contain all the information necessary for any of the procedures. However, this also means that not all fields in the INM are used in every procedure: For example, the general principle of MR-DC in Rel-15 is that MN is in control of the overall MR-DC connectivity, and can "overrule" SN decisions, but if MN does so, SN can always decide to request release of SN resources and MN cannot reject that. This means that even if SN requests MN to modify the used configuration, it is always possible for MN to reject the request. For the SN addition case, MN will indicate the restrictions and SN must comply with those if it wishes to accept the procedure.

Observation 3: The Rel-15 MR-DC principle is that SN must comply with MN-indicated configurations restrictions in all MN-initiated procedures.

## 2.2	MN/SN coordination inter-node messages

The basic usage of the MN/SN configuration restrictions is based on the Rel-15 principles, as discussed below:

- Both MN and SN can provide a lot of information that is not a "direct request" but the other node can take into account (e.g. RRM configuration, DRX, PHR, etc.) to ensure UE capabilities are not exceeded.

- Both MN and SN can request changes to the current configuration - MN via explicit assignment of configuration restrictions and SN via request to change the configuration restrictions.

- For every "requestedField" in ConfigRestrictModReqSCG, a corresponding field is provided via ConfigRestrictInfoSCG.

- For every MN-requested configuration restriction in ConfigRestrictInfoSCG, the corresponding SN-decided value is provided in the main CG-Config IE (i.e. NOT inside ConfigRestrictModReqSCG)

Observation 4: For every MN-requested configuration restriction in ConfigRestrictInfoSCG, the corresponding SN-decided value is provided in the main CG-Config IE (i.e. NOT inside ConfigRestrictModReqSCG)

Observation 5: For every SN-requested configuration restriction change in ConfigRestrictModReqSCG, the corresponding SN-decided value is provided within the ConfigRestrictInfoSCG IE.

However, the current field descriptions are not very clear on these roles even if the signalling seems to have been done according to these. Hence, as was seen in previous meeting email discussion [251], this can cause confusion on how SN indicates the values it uses. Since this is a Rel-15 principle we think it should be captured in Rel-15 RRC to ensure the same confusion doesn't apply anymore.

Proposal 1: Capture in Rel-15 RRC that ConfigRestrictInfoSCG may be used in both MN- and SN-initiated procedures, but ConfigRestrictModReqSCG can only be used in SN-initiated procedures.

The TP based on this can be found in Annex A (based on Rel-16 RRC specification).

Proposal 2: Discuss whether the principle of the TP in Annex A is agreeable. If it is, agree to corresponding CRs (from Rel-15 onwards).

# 3	Conclusion

Observation 1: Both MN and SN can initiate procedures involving exchange of RRC INM.

Observation 2: The Rel-15 MR-DC principle is that SN (or MN) must always either accept or reject the MN (or SN) request. No negotiation is supported.

Observation 3: The Rel-15 MR-DC principle is that SN must comply with MN-indicated configurations restrictions in all MN-initiated procedures.

Observation 4: For every MN-requested configuration restriction in ConfigRestrictInfoSCG, the corresponding SN-decided value is provided in the main CG-Config IE (i.e. NOT inside ConfigRestrictModReqSCG)

Observation 5: For every SN-requested configuration restriction change in ConfigRestrictModReqSCG, the corresponding SN-decided value is provided within the ConfigRestrictInfoSCG IE.

Proposal 1: Capture in Rel-15 RRC that ConfigRestrictInfoSCG may be used in both MN- and SN-initiated procedures, but ConfigRestrictModReqSCG can only be used in SN-initiated procedures.

Proposal 2: Discuss whether the principle of the TP in Annex A is agreeable. If it is, agree to corresponding CRs (from Rel-15 onwards).

The RRC CRs for Rel-15 and Rel-16 are also illustrated in R2-2009160 (Rel-15) and R2-2009161(Rel-16).

# Annex A: TP to RRC inter-node messages (based on Rel-16 RRC)

This TP illustrates the changes proposed to clarify the usage of the MN/SN configuration restriction fields.

CG-Config field descriptions

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can be used to request configuring an NR band combination whose use MN has previously forbidden. SN only includes this field in SN-initiated procedures.

CG-ConfigInfo field descriptions

configRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction. This field may be included in both MN- and SN-initiated procedures.

# Annex B: MN/SN coordination information (as of Rel-16)

To delve a bit deeper into the details of the INM signalling, Tables 2.1-1, 2.1-2, 2.1-3 and 2.1-4 detail the currently (as of Rel-16, RRC version 16.2.0) defined information fields and their usage purposes. Fpor the purpose of explanation, the tables also use a color coding wherein green = MN request (SN must accept or reject), cyan = SN request (MN accepts or rejects), grey = MN/SN information to SN/MN, yellow = MN response to SN request and pink = SN response to MN request. Note that these are sometimes ambiguous as "information" may also be considered as "response" to the configuration restrictions, but the most important point is the information that is shared and how it is shared (i.e. within which IEs).

Table A-1: MN information to SN within CG-Config-Info

| Field name | Purpose |
| --- | --- |
| ue-CapabilityInfo | UE capability information |
| candidateCellInfoListMN, candidateCellInfoListMN-EUTRA | MN candidate cells to allow SN to determine the used band combination |
| candidateCellInfoListSN, candidateCellInfoListSN-EUTRA | SN candidate cells, used for handover with SN |
| measResultCellListSFTD-NR, measResultCellListSFTD-EUTRA, sftdFrequencyList-NR, sftdFrequencyList-EUTRA | SFTD measurement cells and frequencies for SN |
| scgFailureInfo, scgFailureInfoEUTRA, scgFailureInfo-r16, scgFailureInfoEUTRA-r16 | SCG failure information based on UE reporting |
| configRestrictInfo | MN decision or response to configuration restrictions for SN, see Table 2.1-3 |
| drx-InfoMCG, drx-ConfigMCG, drx-InfoMCG2, alignedDRX-Indication | MN DRX information to allow SN to align its DRX configuration with MN |
| measConfigMN | MN RRM measurement configuration |
| sourceConfigSCG, sourceConfigSCG-EUTRA | Current SN configuration - used in handover |
| scg-RB-Config | Current SN bearer configuration - used in handover |
| mcg-RB-Config | Current MN bearer configuration - used in SN addition and in handover |
| mrdc-AssistanceInfo | IDC assistance information |
| ph-InfoMCG | PHR information on MN cells |
| measResultReportCGI, measResultReportCGI-EUTRA | ANR information from UE as per SN request |
| fr-InfoListMCG | Information on which serving cell is using which FR (for e.g. measurement gap purposes) |
| servFrequenciesMN-NR | MN serving cell exact (center) frequencies |
| sidelinkUEInformationNR-r16, sidelinkUEInformationEUTRA-r16 | Sidelink assistance informion |
| ueAssistanceInformationSourceSCG-r16 | UE assistance information received over Uu |

Table A-2: SN information to MN within CG-Config

| Field name | Purpose |
| --- | --- |
| scg-CellGroupConfig, scg-CellGroupConfigEUTRA | Basic SCG configuration |
| scg-RB-Config | Indication: SCG Bearer configuration |
| configRestrictModReq | Request: SN asks to modify MN restrictions - see table 2.1-4 |
| drx-InfoSCG, drx-InfoSCG2, drx-ConfigSCG | SN DRX configuration (information for EN-DC, exact configuration for NR-DC) |
| candidateCellInfoListSN, candidateCellInfoListSN-EUTRA | List of candidate cells for SN, only used in HO |
| measConfigSN | SN measurement configuration |
| selectedBandCombination | Band combination selected by SN |
| fr-InfoListSC | FRX information from SCG serving cells |
| candidateServingFreqListNR, candidateServingFreqListEUTRA | Frequencies of candidate cells (for IDC purpose) |
| pSCellFrequency, pSCellFrequencyEUTRA | Exact ARFCN for selected PSCell |
| reportCGI-RequestNR:: requestedCellInfo, reportCGI-RequestEUTRA:: requestedCellInfoEUTRA | ANR information |
| ph-InfoSCG | PHR information for MCG MAC |
| needForGaps | NE-DC only: Indicates whether SN requests MN to configure gaps |
| scellFrequenciesSN-NR, scellFrequenciesSN-EUTRA | Exact SCell frequencies (different fields for (NG)EN-DC/NR-DC and NE-DC) |
| ueAssistanceInformationSCG-r16 | UE assistance information reported to SN via NR RRC |

Table A-3: MN configuration restrictions to SN within CG-ConfigInfo::ConfigRestrictInfoSCG

| Field name | Purpose |
| --- | --- |
| allowedBC-ListMRDC | Allowed band combination for SN addition |
| powerCoordination-FR1, p-maxNR-FR1-MCG-r16, powerCoordination-FR2-r16 | Power coordination parameters for FR1/FR2 |
| servCellIndexRangeSCG | Allowed indexes for SN cells |
| maxMeasFreqsSCG | Max number of frequencies SN is allowed to configure for measurements |
| selectedBandEntriesMNList | MN-selected bands for determining allowed MR-DC band combination |
| pdcch-BlindDetectionSCG | Max number of allowed SCG PDCCH blind detection candidates |
| maxNumberROHC-ContextSessionsSN, maxNumberEHC-ContextsSN-r16 | Max number of allowed RoHC/EHC contexts for SN |
| maxIntraFreqMeasIdentitiesSCG, maxInterFreqMeasIdentitiesSCG | Max number of intra-/inter-frequency measIDs SN can configure for the UE |
| nrdc-PC-mode-FR1-r16, nrdc-PC-mode-FR2-r16 | UL PC mode for NR-DC |
| maxMeasSRS-ResourceSCG-r16, maxMeasCLI-ResourceSCG-r16 | Max number of CLI/SRS resources used in SCG |
| allowedReducedConfigForOverheating-r16 | Allowed configuration reduction for NR-DC in case of overheating |
| maxToffset-r16 | Maximum allowed value for T-offset when dynamic PC is used in NR-DC |

Table A-4: SN request to MN for modification of configuration restrictions via CG-Config::ConfigRestrictModReqSCG

| Field name | Purpose |
| --- | --- |
| requestedBC-MRDC | Requested band combination for MR-DC (i.e. SN would like to use one or more bands in the requested band combination) |
| requestedP-MaxFR1, requestedP-MaxFR2-r16 | Requested P-Max value for FR1/FR2 |
| requestedPDCCH-BlindDetectionSCG | Requested value for SCG PDCCH blind detection candidates |
| requestedMaxInterFreqMeasIdSCG-r16, requestedMaxIntraFreqMeasIdSCG-r16 | Requested value for maximum number of allowed measIDs for inter-/intra-frequency measurements |
| requestedToffset-r16 | Requested value for T-offset when dynamic UL PC is used in NR-DC |

Table A-5: MN request and SN response for MN-initiated procedures

| MN restriction request | SN response to the restriction (all in CG-Config) |
| --- | --- |
| allowedBC-ListMRDC | selectedBandCombination |
| powerCoordination-FR1, p-maxNR-FR1-MCG-r16, powerCoordination-FR2-r16 | N/A |
| servCellIndexRangeSCG | N/A |
| maxMeasFreqsSCG | measConfigSN |
| selectedBandEntriesMNList | candidateCellInfoListSN, candidateCellInfoListSN-EUTRA, selectedBandCombination, candidateServingFreqListNR, candidateServingFreqListEUTRA, pSCellFrequency, pSCellFrequencyEUTRA |
| pdcch-BlindDetectionSCG | N/A |
| maxNumberROHC-ContextSessionsSN, maxNumberEHC-ContextsSN-r16 | N/A |
| maxIntraFreqMeasIdentitiesSCG, maxInterFreqMeasIdentitiesSCG | N/A |
| nrdc-PC-mode-FR1-r16, nrdc-PC-mode-FR2-r16 | N/A |
| maxMeasSRS-ResourceSCG-r16, maxMeasCLI-ResourceSCG-r16 | N/A |
| allowedReducedConfigForOverheating-r16 | N/A |
| maxToffset-r16 | N/A (Currently missing!) |

Table A-5: SN request and MN response for SN-initiated procedures

| SN restriction change request via CG-Config::ConfigRestrictModReqSCG | MN response via CG-ConfigInfo::ConfigRestrictInfoSCG |
| --- | --- |
| requestedBC-MRDC | allowedBC-ListMRDC |
| requestedP-MaxFR1, requestedP-MaxFR2-r16 | powerCoordination-FR1, p-maxNR-FR1-MCG-r16, powerCoordination-FR2-r16 |
| requestedPDCCH-BlindDetectionSCG | pdcch-BlindDetectionSCG |
| requestedMaxInterFreqMeasIdSCG-r16, requestedMaxIntraFreqMeasIdSCG-r16 | maxIntraFreqMeasIdentitiesSCG, maxInterFreqMeasIdentitiesSCG |
| requestedToffset-r16 | maxToffset-r16 |