3GPP TSG RAN WG2 Meeting #112e                                                        	   R2-2009198

E-Meeting, 2nd – 13th November, 2020

Agenda Item:	8.8.2

Source: 	Intel Corporation

Title:	Consideration of slice based cell (re)selection

Document for: 	Discussion/Decision

# Introduction

In the last meeting, RAN2 discuss the problematic deployment scenarios and the use of existing mechanisms such as dedicated priority to solve the problematic scenarios.  The agreements are as follow:

Agreements:

- Scenarios for now to be studied by RAN2:

- Multiple and different slices can be supported on different frequencies

- Multiple and different slices can be supported in the same frequency layer in different regions.

2	For each scenario we study both IDLE and INACTIVE and determine whether there is need for a solution and possible solutions.  Connected mode will also be considered but with a lower priority.

3	RAN2 will study both cell selection and cell re-selection

Identify the problem with existing mechanisms with dedicated priority and study if some enhancements are needed

An email discussion is organised to further discuss and identify additional scenarios and also discuss whether the existing Rel-15 mechanisms can solve the issue.

In this contribution, we further provide our view in more details than the information we provide in the email discussion on the problematic scenarios and on how the existing Rel-15 mechanisms use two different UE registration areas but also discuss the case of 1 UE registration area.

In the subsequent section, the term ‘intended slice’ is used to define the slice that the UE is intending to access when making a connection to the network.  It can be one of the slices in the allowed NSSAI or a new slice that the UE wants to request for over NAS signalling.

# Discussion

## Additional scenario to consider in the study

In the last meeting, the following example scenario was identified [1] that are typical scenario where slice availability can be different for different frequency in different areas:

Scenario 1: Multiple and difference slices are on the same frequencies in different area

This is based on the general description of the scenario:

•	Multiple and different slices can be supported on different frequencies

•	Multiple and different slices can be supported on the same frequency in different regions.

We think that there are 2 different scenarios in the example scenario, based on the above general description of the scenario:

Scenario 1: Multiple and difference slices are on the same frequencies in different area

Scenario 2: Multiple and different slices are on different frequencies in the same area

Scenario 1 is the example scenario already in the TR where UE is moving from an area without slice 2 into an area with slice 2.

As for Scenario 2, this is the scenario where UE camps on F1 either due to cell selection/reselection criteria and the UE intended slice for registration is slice 2 which is available in F2 as shown below.  The problem is how to ensure that UE slice 2 is also an allowed slice and UE can get access to the slice 2 immediately after registration without having to go to Idle.

Scenario 2: Multiple and different slices are on different frequencies in the same area

Observation#1: Scenario 2 provides the scenario where the UE camps in F1 but requires slice from F2. The question is slice 2 can be allowed slice and whether the UE can fast access to slice 2 in F2 while it is camping on F1.

Proposal#1: Include Scenario 2 (multiple and different slices are on different frequencies in the same area) to the TR.

The text proposal is shown in Annex A.

## Issues to be solved for Scenario 1

In the email discussion, the main focus seems to be on the issue related to the use of dedicated frequency priority configuration if the 2 Areas uses the same dedicated frequency priority configuration. The solution seems to be in the direction that if UE knows the slice availability of each frequency and UE can autonomously manipulate the frequency priority configuration based on the slice availability information to lead the UE to cell reselect to the frequency of the intended slice (See Issues 1-4 in [2]).

However, we think this does not address one issues particularly on how the UE handles the data of the intended slice when it moves between Area 1 and Area 2, and also whether the allowed NSSAI needs to be updated and if so how. For example, when the UE is in Area 1 and the UE moves to Area 2 where the Slice 2 is no longer available and has data for intended Slice 2 - how the UE handles the data for slice 2 while in Area 2 has not been discussed.  Does the UE:

- buffer the data and try continuously to obtain the intended Slice 2 and discard the data when the discard timer runs out? Or

- Is it required to ‘release’ the Slice 2 such that the higher layers are aware that the slice is not available to not generate data for Slice 2?

Currently in SA2 and CT1, it is assumed all cells advertised the same slices in the same UE registration, this data handling issue is addressed by performing UE registration update.

Particularly, if the Slice 2 is ‘released’, the UE has to ‘re-establish’ the Slice 2 when it moves back to Area 1. Further, when the UE moves back to Area 1, how is the UE aware of Slice 2 availability to re-establish Slice 2?  If the Slice 2 is ‘released’, then it may not provide fast access when the UE moves back to Area 1 since the Slice 2 needs to be ‘re-established’

Proposal#2: Solution should not only address moving to the right frequency layer for the intended slice but should also ensure data handling at the UE is also addressed. The solution should resolve the following:

Problem 1: Frequency priority configuration for Area 1 and 2 needs to be different (Issue 1, 3 and 4 in the email discussion)

Problem 2: How the data for the slice is handled at the UE when UE moves between Area 1 and Area 2 (Issue 5)

## Solution to solve the issue for Scenario 1

As in the email discussion, the issues are mainly related to the use of dedicated frequency priority configuration.  In our view, how the data is handled at the UE should also be considered (that is, issues 1, 3, 4 and 5 in [2]) below:

Issue 1: The UE is unaware of the slices supported on different cells or frequencies, which prevents UE from (re)select to the cell or frequency supporting the intended slice.

Issue 3: Operator may require different frequency priority configurations for the specific slice in different areas, however the dedicated priority always overwrites the broadcast priorities if configured.

Issue 4: If the serving cell is unable to support the requested slices for the subsequent access of the UE, the serving cell may bring on handover or rejection of access request. That may increase control plane signalling overhead as well as long control plane latency for the UE to access the network.

Issue 5: If the intended slice is no longer available (e.g. UE moves from Area 1 to 2), the UE behaviour may be unspecified when it has data for the intended slice while Slice 2 is initiated and ongoing (PDU session is still active).

Current Rel-15 mechanisms address these issues with different registration areas for Area 1 and 2 and using dedicated priorities.  The slices that are not available in a registration area are released and will be re-established when UE moves back to an area where it is available.  This may not allow fast access to the slice in scenario 1.

Observation#2: Rel-15 solution uses separate TA for Area 1 and Area 2 to address Problem 1 and 2 of Scenario 1 but cannot support fast access as the slices may need to be re-established.

On the other hand, if Area 1 and Area 2 in Scenario 1 are in the same UE registration area:

For Problem 1 (issue 1, 3, 4), When the UE moves back to Area 1, UE may initially camp in cell 2 based on the dedicated priority provided in Area 2 and hence not be able to get fast access to slice 2 in cell 1.  But this is only for the first access after UE enters an area that the UE may not have the right frequency priority configuration.  For this case, the UE can still request the intended slice as in Rel-15 and the network can use Rel-15 mechanisms such as handover, redirection, CA and DC to provide the service for the intended slice for this first access.

After the first access in an area (Area 1 or 2), the UE can be provided with the right frequency priority configurations for that area as long as the slices are in UEs allowed list, and thus will camp on the right cell without additional delay and will not need additional signalling.

If this first access is also considered a problem, then a new mechanism may need to be introduced to allow for different frequency priority configuration for Area 1 and 2. For example the slice availability information per frequency needs to be broadcast in the SIB or dedicated RRC message (e.g. RRC Release) so that the UE can use it to manipulate the frequency priority configuration to move the UE to the appropriate frequency layer for the intended slice.

For Problem 2 (issue 5), when UE moves from Area 1 to Area 2 with active Slice 2, what does the AS do with the data?  If Slice 2 is ‘released’ in Area 2, UE may need to know whether it can re-establish Slice 2 when it moves back to Area 1.  To facilitate this, providing slice availability of the serving and neighbour frequency/cell may assist the UE in deciding whether to perform the establishment of the PDU session for Slice 2 again.

Observation#3: With a common registration area for scenario 1, for the first access after UE enters an Area, UE may not have the right frequency priority configuration.  After the first access in an area (Area 1 or 2), the UE can be provided with the right frequency priority configurations for that area.   If this first access is also considered an issue, then it may require providing the Slice availability information per frequency in SIB or in dedicated RRC message.

Observation#4: With the same registration area for Area 1 and 2, UE will need another mechanism to trigger the reestablishment of slice 2 such as the intended slice is available when it enters an area (e.g. via broadcasting the slice availability per frequency/cell).

Observation#5: If the Slice 2 is ‘released’ while in Area 2, then it may not provide fast access when the UE moves back to Area 1 since the Slice 2 needs to be ‘re-established’.

Based on the above observations, we make the following proposal:

Proposal#3: If the 2 Areas have to be in the same UE registration areas, RAN2 should study how the data handling is done in Access stratum for the intended slice when it is no longer available in the new cell (or whether the slice it will be ‘released’ by NAS); and if it is ‘released’, how can it be re-established again when the intended slice becomes available again.

## Issues to be solved for Scenario 2 and the corresponding solution

During cell selection, the UE will just find a suitable cell and camp on it. However, this cell may not contain the intended slice for registration (a new slice that the UE wants to request for over NAS signalling) that the UE wants to get a connection with.  A cell in another carrier in the overlapping area offers the intended slice.

For this scenario, the UE must be allowed to register and slice 2 be included in the allowed list.  Further, UE must be allowed to access slice 2 while in the same RRC connection.

This can be addressed by network, taking into consideration the available slices across all the carriers in the region.  That is, network should include slice 2 in the UEs allowed list even if it is not available in the cell that the UE is accessing it.  The service for the slice can be provided using CA/DC or HO to cell 2.

The UE can be subsequently be provided with dedicated priority taking into account all the allowed slices such that UE will camp on cell 1 for subsequent access.

Observation#6: In the case of registration of intended slice, the UE may not be on the frequency layer of the intended slice. Network has to consider all the slices available across the cells when providing the allowed slice list.  If the UE needs to initiate the intended slice in the same access after the registration, the network can always use Rel-15 mechanisms such as handover, CA and DC to provide the service for the intended slice.

Proposal#4: Network handling for registration and existing Rel-15 mechanism such as HO, CA and DC can address the issue for the initial access of the intended in the same access after the registration of the intended slice for scenario 2.

# Conclusion

It is requested that RAN2 discussed the following observations and proposals:

Observation#1: Scenario 2 provides the scenario where the UE camps in F1 but requires slice from F2. The question is slice 2 can be allowed slice and whether the UE can fast access to slice 2 in F2 while it is camping on F1.

Proposal#1: Include Scenario 2 (multiple and different slices are on different frequencies in the same area) to the TR.  The text proposal is shown in Annex A.

Proposal#2: Solution should not only address moving to the right frequency layer for the intended slice but should also ensure data handling at the UE is also addressed. The solution should resolve the following:

Problem 1: Frequency priority configuration for Area 1 and 2 needs to be different (Issue 1, 3 and 4 in the email discussion)

Problem 2: How the data for the slice is handled at the UE when UE moves between Area 1 and Area 2 (Issue 5)

Observation#2: Rel-15 solution uses separate TA for Area 1 and Area 2 to address Problem 1 and 2 of Scenario 1 but cannot support fast access as the slices may need to be re-established.

Observation#3: With a common registration area for scenario 1, for the first access after UE enters an Area, UE may not have the right frequency priority configuration.  After the first access in an area (Area 1 or 2), the UE can be provided with the right frequency priority configurations for that area.   If this first access is also considered an issue, then it may require providing the Slice availability information per frequency in SIB or in dedicated RRC message.

Observation#4: With the same registration area for Area 1 and 2, UE will need another mechanism to trigger the reestablishment of slice 2 such as the intended slice is available when it enters an area (e.g. via broadcasting the slice availability per frequency/cell).

Observation#5: If the Slice 2 is ‘released’ while in Area 2, then it may not provide fast access when the UE moves back to Area 1 since the Slice 2 needs to be ‘re-established’.

Proposal#3: If the 2 Areas have to be in the same UE registration areas, RAN2 should study how the data handling is done in Access stratum for the intended slice when it is no longer available in the new cell (or whether the slice it will be ‘released’ by NAS); and if it is ‘released’, how can it be re-established again when the intended slice becomes available again.

Observation#6: In the case of registration of intended slice, the UE may not be on the frequency layer of the intended slice. Network has to consider all the slices available across the cells when providing the allowed slice list.  If the UE needs to initiate the intended slice in the same access after the registration, the network can always use Rel-15 mechanisms such as handover, CA and DC to provide the service for the intended slice.

Proposal#4: Network handling for registration and existing Rel-15 mechanism such as HO, CA and DC can address the issue for the initial access of the intended in the same access after the registration of the intended slice for scenario 2.

# References

[1] TR38.832	Study on enhancement of Radio Access Network (RAN) slicing

[2] Email discussion#916		 RAN slicing study (CMCC)

# Annex A: Text proposal for a new scenario

### Scenario and issue description

Editor Note: capture the description of scenario and issue.

General description for the scenario:

•	Multiple and different slices can be supported on different frequencies

•	Multiple and different slices can be supported on the same frequency in different regions.

Editor Note: Additional scenarios can be discussed as part of the study.

Two scenarios are identified that fit to the general description:

Scenario 1: Multiple and difference slices are on the same frequencies in different area

Scenario 2: Multiple and different slices are on different frequencies in the same area

For each scenario we study both IDLE and INACTIVE and determine whether there is need for a solution and possible solutions. Connected mode will also be considered but with a lower priority.

We will investigate whether the R15 mechanism (e.g. dedicated priority mechanism) can solve the above issues and study if some enhancements are needed.

Editor Note: Both cell selection and cell re-selection will be studied.

Figure 5.1.1-1: Scenario 1: Multiple and difference slices are on the same frequencies in different area

Figure 5.1.1-2: Scenario 2: Multiple and different slices are on different frequencies in the same area

As shown in figure 1, slice1 (e.g. eMBB) is supported in both F1 and F2 everywhere, since the frequency resources are so valuable and the top requirement for all operators’ 5G network is to serve millions or billions of smart phone users. Slice2 (e.g. URLLC) is supported only in F2 in some area, e.g. factory or hospital.

Area 1 is deployed in the factory or hospital. In this area, F1 supports slice1 (e.g. eMBB), while F2  supports both slice 1 and slice 2 (e.g. eMBB and URLLC).

Area 2 is the public area. F1 and F2 all supporting slice1 (e.g. eMBB) for smart phone users, no slice2 (e.g. URLLC) is supported in area 2. And F2 is deployed as hotspot to provide wideband access.

eMBB and URLLC slices are used only as an example of various slices. The deployment of any slice on any frequency band is up to network implementation.