3GPP TSG-RAN WG2 #112e	R2-2009218

Electronic meeting, November 2nd – 13th 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0913 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MAC spec regarding SL-CSI reporting MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE may be triggered with multiple SL-CSI reporting events for different PC5-RRC connections. In this case, these events may be associated with different latency requirements. In other words, these events would be cancelled due to expiry of sl-CSI-ReportTimer at different time. For a SL-CSI event with shorter latency requirement, it would be beneficial to transmit the corresponding MAC CE earlier than other events if there is an available SL grant. Correspondingly, in the LCP procedure, UE needs to select the destination with the SL-CSI event with shortest remaining latency bound. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.22.1.4.1.2, add corresponding corrections in the note. If multiple Destinations have Sidelink CSI Reporting MAC CE, UE selects the Destination with shortest remaining latency bound among them. Remaining latency bound of a triggered CSI report is calculated as the latency bound of the SL CSI report minus the elapsed time of the sl-CSI-ReportTimer. Impact analysis Impacted functionality HARQ process operation Inter-operability: If UE implements the changes, while the network doesn’t, there is no inter-operability issue foreseen. If UE doesn’t implement the changes, while the network does, UE may not be able to transmit CSI reporting MAC CE with critial latency requirements in case there is no sufficient SL grant. If one UE implements the changes, while other UE doesn’t, there may be misaligned UE behaviors regarding SL-CSI reporting MAC CE handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case multiple SL-CSI report events are triggered for different PC5-RRC connections, UE actions on how to handle SL-CSI reporting MAC CEs are unclear. UE may not be able to transmit CSI reporting MAC CE with critial latency requirements in case there is no sufficient SL grant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

###### 5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination associated to one of unicast, groupcast and broadcast, having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

2>	SL data is available for transmission; and

2>	SBj > 0, in case there is any logical channel having SBj > 0; and

2>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

2>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE except Sidelink CSI Reporting MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE except Sidelink CSI Reporting MAC CE, which Destination is selected among them is up to UE implementation. If multiple Destinations have Sidelink CSI Reporting MAC CE, UE selects the Destination with shortest remaining latency bound among them. Remaining latency bound of a triggered CSI report is calculated as the latency bound of the SL CSI report minus the elapsed time of the sl-CSI-ReportTimer.

1>	select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination:

2>	SL data is available for transmission; and

2>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and.

2>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	if PSFCH is configured for the sidelink grant associated to the SCI:

3>	sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or

3>	sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions.

2>	else:

3>	sl-HARQ-FeedbackEnabled is set to disabled.

End of the first change