3GPP TSG-RAN WG2 #112e	R2-2009227

Electronic meeting, November 2nd – 13th 2020

Agenda Item:	6.4.3

Source:	Ericsson

Title:	Open issues on UL/SL prioritization

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#107-bis, the following agreements were made regarding prioritization between UL and SL:

Agreements on prioritization:

1: 	A separate LCH priority thresholds is configured for both NR-UL and NR-SL.

2:	For between SL-data and UL-data/SRB, the SL transmission is prioritized if the highest priority value of UL LCH(s) with available data is larger than the UL priority threshold and the highest priority value of SL LCH(s) with available data is lower than the SL priority threshold. Otherwise the UL transmission is prioritized.

3:	Prioritization between UL SR and SL data transmission could be based on priority of the UL LCH that triggered the UL SR and priority value(s) of SL LCH(s), similar as prioritization between NR UL data and NR SL data transmission.

In this paper, we discuss some remaining aspects related to UL/SL prioritization.

# 2	Discussion

- There are a few open issues related to UL/SL prioritization. We discuss several cases below:

Issue 1: PUSCH containing UL-SCH and SL HARQ report overlaps with PSCCH/PSSCH transmission

This is the case when SL HARQ report is contained in a PUSCH, which is multiplexed with an UL shared channel (UL-SCH), and this UL transmission collides in time domain with a SL transmission. In RAN1 it was agreed that the priority of PUCCH carrying SL HARQ report is the priority of the SL HARQ report, where the priority of SL HARQ report is the smallest priority value among PSSCH transmissions corresponding to SL HARQ report as described in clause 16.2.4.3.1 of TS 38.213. Naturally when PUSCH contains SL HARQ report the UL transmission should have one priority from the SL HARQ report. The UL transmission will also have another priority from the UL-SCH. The value of the highest priority of the logical channel(s) or the priority of the SL HARQ report in the MAC PDU should be used in determining whether SL or UL transmission should be prioritized.

When SL HARQ report is multiplexed with UL-SCH (i.e., UCI multiplexed on PUSCH), the UL transmission would be associated with two priority indices, i.e., one priority from the SL HARQ report, and another priority from UL-SCH.

In case PUSCH containing UL-SCH and SL HARQ report overlaps with SL transmission, the value of the highest priority of the logical channel(s) or the priority of the SL HARQ report in the PUSCH shall be used in the current UL/SL prioritization rule to determine whether UL or SL should be prioritized.

Issue 2: PUSCH containing UL-SCH and UCI other than SL HARQ report overlaps with PSCCH/PSSCH transmission

This is the case when UCI other than SL HARQ report is contained in a PUSCH, which is multiplexed with an UL-SCH, and this UL transmission collides in time domain with a SL transmission. In this case, the potential UCI is Uu HARQ report and Uu CSI report, which is not associated with a priority derived from data transmission. Therefore, the current UL/SL prioritization rule for PUSCH without UCI should be reused, i.e. priority from the UL-SCH is used in determining whether SL or UL transmission should be prioritized.

When PUSCH contains UL-SCH and UCI other than SL HARQ, the current UL/SL prioritization rule for PUSCH without UCI is reused.

We have also prepared corresponding CR [1].

# Conclusion

In the previous sections we made the following proposals:

Proposal 1	In case PUSCH containing UL-SCH and SL HARQ report overlaps with SL transmission, the value of the highest priority of the logical channel(s) or the priority of the SL HARQ report in the PUSCH shall be used in the current UL/SL prioritization rule to determine whether UL or SL should be prioritized.

Proposal 2	When PUSCH contains UL-SCH and UCI other than SL HARQ, the current UL/SL prioritization rule for PUSCH without UCI is reused.

# References

[1] R2-2009225_CR0919_38.321_Rel16_corrections to MAC spec regarding prioritization between UL Tx and SL Tx, Ericsson, RAN2#112-e.