3GPP TSG-RAN WG2 #112 electronic	R2-2009656

E-Meeting, 2nd – 13th November 2020

Source:		NEC

Agenda item:		8.6.3

Title:		Control plane issues for SDT

Document for:		Discussion

# 1. Introduction

During last RAN2 meeting, the following baseline agreements were made for RACH-based SDT and CG-based SDT [1]:

Agreements

1 	Small data transmission with RRC message is supported as baseline for RA-based and CG based schemes

2	RRC-less can be studied for limited use cases (e.g. same serving cell and/or for CG) with lower priority

3	Context fetch and data forwarding with anchor re-location and without anchor re-location will be considered.   FFS if there are problems with the scenario “without anchor relocation”.

4	From RAN2 perspective, stored “configuration” in the UE Context is used for the RLC bearer configuration for any SDT mechanism (RACH and CG).

5	The 2-step RACH or 4-step RACH should be applied to RACH based uplink small data transmission in RRC_INACTIVE

6	The uplink small data can be sent in MSGA of 2-step RACH or msg3 of 4-step RACH.

7	Small data transmission is configured by the network on a per DRB basis

8	Data volume threshold is used for the UE to decide whether to do SDT or not.   FFS how we calculate data volume.

FFS if an “additional SDT specific” RSRP threshold is further used to determine whether the UE should do SDT

9	UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported

10	When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant.  FFS on details and whether any indication to network is needed.

In this paper, we provide our further consideration on the design of small data transmission.

# 2. Discussion

During last meeting, we have agreed that SDT is configured by the network per DRB, and data volume threshold is used for the UE to decide whether to do SDT. The trigging scheme for SDT, however, is still not crystal clear. For example the DRBs support SDT shall be configured by dedicated signalling, but how is the threshold configured, by broadcast or by dedicated signalling? Do we allow different threshold for different traffic? The threshold is per UE or per DRB?

Firstly, we think a simple design of triggering condition is sufficient. The triggering of SDT should fulfil that the new data are all from DRBs configured with SDT, and the data volume of the new data is less than a threshold.

As RACH-based SDT can be initiated in new cell within RNA, data volume threshold can be broadcast by each cell. Currently, we don’t see obvious benefit of dedicated configured threshold, thus we propose that threshold should be broadcasted by system information only.

Proposal 1: The data volume threshold is configured by broadcast. FFS if there is any need to configure dedicatedly.

One FFS from last meeting is how we calculate data volume. The SDT triggering is decided by RRC layer, and by the time the data is transmitted, the data volume may be different from when SDT triggered, we think that an estimated MAC PDU size can be used. How RRC get it can be based on implementation

Proposal 2: The data volume is the estimated resulting MAC PDU size, how RRC get it is based on implementation.

Once the RRC decide to perform SDT transmission, the DRBs configured with SDT should be resumed, and the corresponding RLC and PDCP entities should be re-established. One argument point is if the DRBs not configured for SDT should also be resumed upon SDT initialization. If the DRBs not configured for SDT are also resumed, then there maybe new data arriving for these DRBs not configured with SDT during SDT. This will result in that the data from non-SDT DRBs transmitted by SDT way, which should be avoided.

Proposal 3: The DRB not configured with SDT should not be resumed upon SDT initialization.

In LTE EDT, the selected RACH resource and the data volume threshold are matched, and the data is transmitted within one time of transmission. If the packet size at the MAC layer is larger than the threshold, or if the dedicated RACH resource corresponding to the data volume threshold is not available, the EDT is cancelled. For SDT, do we still support the matching between data volume threshold and RACH resource? How does the UE select RACH resource from RACH configurations which provides flexible payload sizes?

As we have agreed that subsequent data transmission is supported, we don’t have to finish the SDT transmission in one-shot, thus there is no need to cancel the SDT if the resulting MAC PDU exceeds the threshold. And there is no strict matching between the data volume threshold and the TB size provided by the selected RACH resource. The MAC layer can try to select RACH resource that can carry the resulting MAC PDU. If such RACH resource does not exist, the RLC packet can be segmented, and UE indicate the remaining data by BSR to the network.

Proposal 4: The MAC layer does not take the data volume threshold into account.

As described above that the non-SDT DRBs should not be resumed. When UE performing SDT, including RACH-based SDT, CG-based SDT, or subsequent SDT, if there is new signalling or new UL data arriving from \DRBs not configured with SDT, the UE behaviour needs further discussion.

The following potential options can be considered

- Option 1. Initiate RRC Resume procedure for normal data transmission immediately

- Option 2. Finish the current SDT transmission first

- Option 3. Perform the actions upon going to RRC_IDLE

Option 2 will lead to delay for the new arriving data. Option 3 may results in packet loss for the on-going traffic, and will also results in longer latency. Option 1 needs further investigation on if there is any security issue, and how to support lossless for the on-going data need further discussion.

Proposal 5: RAN2 discuss how to handle the arriving of new signalling or new data from DRBs not configured with SDT.

# 3. Conclusion

In this contribution, we provide our further consideration on SDT, and have the following proposals:

Proposal 1: The data volume threshold is configured by broadcast. FFS if there is any need to configure dedicatedly.

Proposal 2: The data volume is the estimated resulting MAC PDU size, how RRC get it is based on implementation.

Proposal 3: The DRB not configured with SDT should not be resumed upon SDT initialization.

Proposal 4: The MAC layer does not take the data volume threshold into account.

Proposal 5: RAN2 discuss how to handle the arriving of new signalling or new data from DRBs not configured with SDT.

# Reference

[1] meeting report of RAN2 #111e