3GPP TSG-RAN WG2 Meeting #112 electronic							    	R2-2009672

E-Meeting, 2nd Nov – 13rd Nov, 2020

Source: 	Beijing Xiaomi Mobile Software

Title:  	Mobility related issues for the propagation delay compensation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

According to the discussion [1] in the RAN2#111-e meeting, RAN2 made the following agreements, and had an email discussion [2] on the error components and the potential solutions for the propagation delay compensation.

=> Discuss by email the delay components and understand the requirements with each component and agree on what needs to be addressed => Introduce propagation delay compensation for the improved synchronisation accuracy requirement in case of in UL Time Synchronization

According to the discussion in the RAN1#102-e meeting, RAN1 also discussed the potential solutions to compensate the propagation delay of the reference time. In this contribution, we provide our understandings on the mobility impacts on the PDC (Propagation Delay Compensation), which have not been discussed in both RAN1 and RAN2.

Discussion

Mobility related scenarios

According to the email discussion [2] for the propagation delay compensation in RAN2, RAN2 discussed the components of the error budget via the network, the UE and the Uu interface. The potential scenarios to be considered are listed as given in the Annex A (as provided in TS 22.104 Table 5.6.2-1). All companies think that Scenario 2 and 3 should be considered, and 10 out of the 17 companies think Scenario 1 should be considered as well. According to the section A.2.2.1 (as quoted in the Annex B) of the TS 22.104, the UE speed can be up-to 72 km/h for the motion control in Scenario 1.

Observation 1: In Scenario 1, the UE speed can be up-to 72 km/h.

In scenario 3, the service of the high data rate video streaming include the use case 2 and 4 of the Mobile robots with speed up-to 50 Km/h, according to the Table A.2.2.3-1 (as quoted in the Annex C) of the TS 22.104.

Observation 2: In Scenario 3, the UE speed can be up-to 50 km/h.

According to the agreements and the evaluations [4] in RAN1 (as quoted below), the UE speed is not considered in the evaluation of the errors. However the UE speed could cause more errors on the signal detection of both UL (e.g. preamble/SRS for UL timing estimation) and DL (e.g. SSB for DL synchronisation).

Agreements: - Take the following use cases as the representative use cases for further study on propagation delay compensation enhancements in Rel-17. User-specific clock synchronicity accuracy level; Number of devices in one Communication group for clock synchronisation; 5GS synchronicity budget requirement (note); Service area; Scenario. 2; Up to 300 UEs; ≤900 ns; ≤ 1000 m x 100 m; - Control-to-control communication for industrial controller. 4; Up to 100 UEs; <1 µs; < 20 km2; - Smart Grid: synchronicity between PMUs.

Thus we consider that the UE speed (i.e. up-to 75 km/h for scenario 1, and up-to 50 km/h for scenario 3) should be considered for the error budget evaluation.

Proposal 1: The UE mobility (i.e. up-to 75 km/h for scenario 1, and up-to 50 km/h for scenario 3) is considered for the evaluation of propagation delay compensation.

Mobility impacts on the solutions for PDC

According to the email discussion [2] of RAN2, we could have the following solutions for PDC:

- Option 1a: Leave this up to UE implementation and do not specify any enhancements. - Option 1b: Leave this up to UE implementation but specify finer granularity of TA command to assist the UE calculation. - Option 2a: Specify in the specifications propagation delay compensation based on TA command (no TA granularity enhancements). - Option 2b: Specify in the specifications propagation delay compensation based on TA command and enhance TA granularity. - Option 3: Perform pre-compensation on the network side (up to network implementation) and add the indication in the network to UE signalling that the time information was pre-compensated. - Option 4: reuse some aspects of the positioning framework timing difference measurements for propagation delay compensation

According to the agreements in RAN1, the potential solutions for PDC are listed as follows:

Agreements: The following options for propagation delay compensation are further studied in RAN1 - Option 1: TA-based propagation delay - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity). - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te) - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected) - Option 2: RTT based delay compensation: Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning.).

No matter which solution is selected for the PDC, we considered that the following two mobility-related issues should be considered:

- Issue 1: The timing of the propagation delay estimation should be sufficiently close to the timing when the reference time information is provided to the UE.

- Issue 2: The propagation delay compensation should only be required when the UE is located far from the transmission point of the gNB.

Figure 1: Issue 1 of the estimation of the propagation delay compensation

As illustrated in Figure 1 for Issue 1, as the UE could be moving around, if the timing of the propagation delay estimation of the UE is not close to the reception time of the referenceTimeInfo at the UE, the PDC itself could introduce extra error (i.e. “D2-D1”, D1 is the real propagation delay of the reference time and D2 is the estimated propagation delay) for the reference time.

Proposal 2: The timing of the propagation delay estimation should be sufficiently close to the timing when the reference time information is provided to the UE.

Figure 2: Issue 2 of the UE location of the propagation delay compensation

As illustrated in Figure 2 for Issue 2, the PDC is only required when the UE’s location is far away from the transmission point of the gNB, as the error of the propagation delay can be negligible when the UE is close to the transmission point of the gNB. Then the PDC would only be required for some UEs which is located at the edge of the cell (e.g. UE_2 in Figure 1), and for the UEs when its location is changed to the cell edge (e.g. the UE_1’s location in Figure 1 could be changed due to the UE_1’s mobility). If the UE/gNB is mandated to always perform PDC, the PDC would cause extra overhead on the UL signalling transmission (e.g. the signalling for estimating the TA) and the signalling used for providing the estimated propagation delay to the UE.

Proposal 3: The propagation delay compensation should only be required when the UE is located far from the transmission point of the gNB.

Conclusions

Based on the analysis given above, we have the following Observations and Proposals.

Observation 1: In Scenario 1, the UE speed can be up-to 72 km/h.

Observation 2: In Scenario 3, the UE speed can be up-to 50 km/h.

Proposal 1: The UE mobility (i.e. up-to 75 km/h for scenario 1, and up-to 50 km/h for scenario 3) is considered for the evaluation of propagation delay compensation.

Proposal 2: The timing of the propagation delay estimation should be sufficiently close to the timing when the reference time information is provided to the UE.

Proposal 3: The propagation delay compensation should only be required when the UE is located far from the transmission point of the gNB.

Reference

[1] RAN2#111-e meeting, Chair’s minutes.

[2] R2-200xxxx, Nokia, “E-mail discussion: [Post111-e][924][R17 URLLC/IIoT] Propagation delay for TSN (Nokia)”.

[3] 3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains; Stage 1”.

[4] R1-2007068	Summary#1 of email discussion [102-e-NR-IIOT_URLLC_enh-05]	Moderator (Huawei).

- Annex A

The following Table is extracted from the Table 5.6.2-1 of the TS 22.104.

Table 5.6.2-1: Clock synchronization service performance requirements for 5G System

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 1 | up to 300 UEs | ≤ 900 ns | ≤ 100 m x 100 m | - Motion control (A.2.2.1) - Control-to-control communication for industrial controller (A.2.2.2) |
| 2 | up to 300 UEs | ≤ 900 ns | ≤ 1,000 m x 100 m | - Control-to-control communication for industrial controller (A.2.2.2) |
| 3 | up to 10 UEs | < 10 µs | ≤ 2,500 m2 | - High data rate video streaming |
| 3a | up to 100 UEs | < 1 µs | ≤ 10 km2 | - AVPROD synchronisation and packet timing |
| 4 | up to 100 UEs | < 1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |
| 5 | up to 10 UEs | < 50 µs | 400 km | - Telesurgery (A.6.2) and telediagnosis (A.6.3) |
| NOTE: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system, as described in Clause 5.6.1. |  |  |  |  |

- Annex B

The following Table is extracted from the Table 5.6.2-1 of the TS 22.104.

Table A.2.2.1-1: Service performance requirements for motion control

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: lower bound | Transfer interval: upper bound | Survival time | UE speed | # of UEs | Service area (note) |
| 1 | 99.999 to 99.999 99 | ~ 10 years | < transfer interval value | – | 50 | 500 μs – 500 ns | 500 μs + 500 ns | 500 μs | ≤ 72 km/h | ≤ 20 | 50 m x 10 m x 10 m |
| 2 | 99.999 9 to 99.999 999 | ~ 10 years | < transfer interval value | – | 40 | 1 ms – 500 ns | 1 ms + 500 ns | 1 ms | ≤ 72 km/h | ≤ 50 | 50 m x 10 m x 10 m |
| 3 | 99.999 9 to 99.999 999 | ~ 10 years | < transfer interval value | – | 20 | 2 ms – 500 ns | 2 ms + 500 ns | 2 ms | ≤ 72 km/h | ≤ 100 | 50 m x 10 m x 10 m |
| NOTE: Length x width x height. |  |  |  |  |  |  |  |  |  |  |  |  |

- Annex C

The following Table is extracted from the Table A.2.2.3-1 of the TS 22.104.

Table A.2.2.3-1: Service performance requirements for mobile robots

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: lower bound | Transfer interval: target value (note) | Transfer interval: upper bound | Survival time | UE speed | # of UEs | Service area |
| 1 | > 99.999 9 | ~ 10 years | < target transfer interval value | – | 40 to 250 | – < 25 % of target transfer interval value | 1 ms to 50 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 |
| 2 | > 99.999 9 | ~ 1 year | < target transfer interval value | – | 15 k to 250 k | – < 25 % of target transfer interval value | 10 ms to 100 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 |
| 3 | > 99.999 9 | ~ 1 year | < target transfer interval value | – | 40 to 250 | – < 25 % of target transfer interval value | 40 ms to 500 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 |
| 4 | > 99.999 9 | ~ 1 week | 10 ms | > 10 Mbit/s | – | – |  | – | – | ≤ 50 km/h | ≤ 100 | ≤ 1 km2 |
| NOTE: The transfer interval is not so strictly periodic in these use cases. The transfer interval deviates around its target value within bounds. The mean of the transfer interval is close to the target value. |  |  |  |  |  |  |  |  |  |  |  |  |

Use case one

Periodic communication for the support of precise cooperative robotic motion control (transfer interval: 1 ms), machine control (transfer interval: 1 ms to 10 ms), co-operative driving (10 ms to 50 ms).

Use case two

Periodic communication for video-operated remote control.

Use case three

Periodic communication for standard mobile robot operation and traffic management.

Use case four

Real-time streaming data transmission (video data) from a mobile robot to the guidance control system.