3GPP TSG-RAN WG2 Meeting #112 electronic    	                              R2-2009685

# Online, November 2nd - 13th, 2020

Agenda Item:	8.13.2.3

Source: 	vivo

Title:         	Discussion on RACH report for SgNB

Document for: 	Discussion and Decision

- Introduction

RAN3 sent an LS to RAN2 [1] during the RAN3#109-e meeting, asking RAN2 to consider the UE RACH report for SgNBs and to provide feedback to RAN3. The overall description is shown below:

RAN3 discussed the use cases of RACH report for SgNBs, and observed that there was no means for the SgNB to retrieve from UE in MR-DC any information on RACH access procedure at SgNB, and thus there was no input for SON algorithm to adjust the RA related parameters in SgNBs.

In this contribution, we would like to discuss the signalling design for SgNB RACH report retrieval.

- Discussion

As shown in Figure 1, in Rel-16, UE indicates the availability of the existing reports at every transition to RRC connected mode, or by means of UEInformationReponse in case the UE has not transferred the total log in one RRC message. Upon the reception of such indications, the NW optionally requests the desired report(s) from UE via UEInformationRequest message with the inclusion of request indicators. Subsequently, UE delivers the requested report(s) via UEInformatonResponse message.

Figure 1: Report retrieval procedure in Rel-16.

The legacy report retrieval procedure is merely designed for MN and UE interaction because currently the recorded reports are all associated with MN and are irrelevant to SN. So there is no available signalling procedure for the SgNB to retrieve any information on RACH access procedure at SgNB from UE in MR-DC, as requested by RAN3.

There are primarily three steps for the retrieval procedure, where the first step can be omitted because the RACH report itself implies the successful operations on the access cell and, therefore, inevitably exists once the UE comes back to the CONNECTED state. The inability of the retrieval procedure for the SgNB RACH report arises from the subsequent two steps.

- The availability indicator for the SgNB RACH report is not required, hence no issue is identified for the availability indication step.

  - Issue for step (b)

The legacy UEInformationRequest message is directly sent to UE by MN, this message can only be transmitted via SRB1. Two options are feasible to enable the interaction between SN and UE, that is to either enhance UEInformationRequest message with SRB3 transmission, or reuse the current procedure but to embed UEInformationRequest message in EUTRA (in case MN is an eNB) or NR DLInformationTransferMRDC (in case MN is a gNB).

- Either to enhance UEInformationRequest message with SRB3 transmission, or to reuse the current procedure but to embed UEInformationResquest message in EUTRA (in case MN is an eNB) or NR DLInformationTransferMRDC (in case MN is a gNB) enables the interaction between SN and UE.

The former option can only be adopted when UE is configured with SRB3. In other words, SgNB cannot send the request message to UE, if SRB3 is not configured previously. By contrast, the latter option offers more flexibility to both UE and SgNB, by this means, SgNB is able to request the desired report (by including request indicator for RACH report) from UE anytime at its convenience.

- The latter option that UEInformationRequest message embedded in EUTRA/NR DLInformationTransferMRDC offers more flexibility to both UE and SgNB.

From our perspective, literally no restriction should be placed on this interaction, therefore we prefer the latter option:

The legacy UEInformationRequest message can be embedded in EUTRA/NR DLInformationTransferMRDC to enable the interaction between SN and UE.

  - Issue for step (c)

Once SgNB is in a position to send the request message, UE needs to respond with the corresponding RACH report via UEInformationReponse. Nonetheless, the legacy UEInformationReponse message is used to carry the MN RACH report solely, furthermore, UEInformationReponse is terminated at MN and unable to arrive at SgNB. Evidently, the legacy response message is not suitable for the transmission of SN RACH report.

- The SN RACH report cannot be sent to SN via the legacy UEInformationReponse message as the UEInformationReponse merely carries the MN RACH report and is terminated at MN.

Straight-forwardly, a new message, e.g., UEInformationReponseSCG, can be specified to carry the SN RACH report. As the name suggests, this message contains SN related report(s) only and can be sent to SN upon reception of the request.

Noticeably, SRB2 is used for the transfer of legacy UEInformationResponse with the presence of logged measurement report, and SRB1 for the rest of the cases. However, the new message UEInformationReponseSCG does not need to include logged measurement report, hence should only be sent via SRB1 or SRB3 (if configured).

A new message, e.g., UEInformationReponseSCG, is used to transfer the SN RACH report to SN.

- The legacy UEInformationResponse message can be sent via SRB1 or SRB2 (with the presence of logged measurement report), but the new message does not need to be sent via SRB2 as it does not include the logged measurement report.

The new message used to transfer the SN RACH report can be sent via SRB1 or SRB3 (if configured).

- Conclusion

In this paper, the following observations and proposal are given:

Observation 1	The availability indicator for the SgNB RACH report is not required, hence no issue is identified for the availability indication step.

Observation 2	Either to enhance UEInformationRequest message with SRB3 transmission, or to reuse the current procedure but to embed UEInformationResquest message in EUTRA (in case MN is an eNB) or NR DLInformationTransferMRDC (in case MN is a gNB) enables the interaction between SN and UE.

Observation 3	The latter option that UEInformationRequest message embedded in EUTRA/NR DLInformationTransferMRDC offers more flexibility to both UE and SgNB.

Observation 4	The SN RACH report cannot be sent to SN via the legacy UEInformationReponse message as the UEInformationReponse merely carries the MN RACH report and is terminated at MN.

Observation 5	The legacy UEInformationResponse message can be sent via SRB1 or SRB2 (with the presence of logged measurement report), but the new message does not need to be sent via SRB2 as it does not include the logged measurement report.

Proposal 1	The legacy UEInformationRequest message can be embedded in EUTRA/NR DLInformationTransferMRDC to enable the interaction between SN and UE.

Proposal 2	A new message, e.g., UEInformationReponseSCG, is used to transfer the SN RACH report to SN.

Proposal 3	The new message used to transfer the SN RACH report can be sent via SRB1 or SRB3 (if configured).

- References

- R3-205662, LS to RAN2 on RACH report for SgNB, RAN3, to RAN2, RAN3#109-e meeting, August 17th – 28th, 2020.