3GPP TSG-RAN2 Meeting #112-e	R2-2007526

# Online, 2nd NOV 2020 - 13th NOV 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0952 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 38.321 for HARQ processes handling of retransmission within a bundle(R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Repetition based retransmission can be configured to UE for transmitting one TB multiple times in order to enhance the transmission reliability. In the current TS 38.321-REL16, the description for retransmission with repetition method as following: ------------------------------------- From 38.321 g21----------------------------------------- New transmissions are performed on the resource and with the MCS indicated on PDCCH or indicated in the Random Access Response (i.e. MAC RAR or fallbackRAR), or signalled in RRC or determined as specified in clause 5.1.2a for MSGA payload. Retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH, or on the same resource and with the same MCS as was used for last made transmission attempt within a bundle, or on stored configured uplink grant resources and stored MCS when cg-RetransmissionTimer is configured. Retransmissions with the same HARQ process may be performed on any configured grant configuration if the configured grant configurations have the same TBS. ---------------------------------------From 38.321g21 ------------------------------------------ - Accoriding to the yellow highlighted wording in 38.321, it seems not correct to use the “same resource” to describe the retransmission with the repetition method because each retransmission in a bundle is for sure in different resources of time domain. For example, The time resouce allocation for repetition is specified in TS 38.214 as shown below: - -------------------------- From 38.214 g30 ------------------------------------------------- For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2. - <omit for short> For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, the UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213]. - -------------------------- From 38.214 g30------------------------------------------------ - Hence the correction on 38.321 for repetition transmission is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: UL Repetition transmission Inter-operability: If UE impments according to the CR but NW does not, NW cannot receive the retransmission within a bundle. If NW implements according to the CR but UE does not, UE will send the repetition transmission in a bundle with a wrong resources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will send the repetition transmission in a bundle within a wrong resources |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------- [Changes Start] ----------------------------

5.4.2.2	HARQ process

Each HARQ process is associated with a HARQ buffer.

New transmissions are performed on the resource and with the MCS indicated on PDCCH or indicated in the Random Access Response (i.e. MAC RAR or fallbackRAR), or signalled in RRC or determined as specified in clause 5.1.2a for MSGA payload. Retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH, or on the pre-defined resource and with the same MCS as was used for last made transmission attempt within a bundle according to the clause 6.1.2 of TS 38.214 [7], or on stored configured uplink grant resources and stored MCS when cg-RetransmissionTimer is configured. Retransmissions with the same HARQ process may be performed on any configured grant configuration if the configured grant configurations have the same TBS.

When cg-RetransmissionTimer is configured and the HARQ entity obtains a MAC PDU to transmit and LBT failure indication is received from lower layer, the corresponding HARQ process is considered to be pending. For a configured uplink grant, configured with cg-RetransmissionTimer, each associated HARQ process is considered as not pending when:

-	a transmission is performed on that HARQ process and LBT failure indication is not received from lower layers; or

-	the configured uplink grant is initialised and this HARQ process is not associated with another active configured uplink grant; or

-	the HARQ buffer for this HARQ process is flushed.

If the HARQ entity requests a new transmission for a TB, the HARQ process shall:

1>	store the MAC PDU in the associated HARQ buffer;

1>	store the uplink grant received from the HARQ entity;

1>	generate a transmission as described below.

If the HARQ entity requests a retransmission for a TB, the HARQ process shall:

1>	store the uplink grant received from the HARQ entity;

1>	generate a transmission as described below.

To generate a transmission for a TB, the HARQ process shall:

1>	if the MAC PDU was obtained from the Msg3 buffer; or

1>	if the MAC PDU was obtained from the MSGA buffer; or

1>	if there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer or the MSGA buffer:

2>	if there are neither transmission of NR sidelink communication nor transmission of V2X sidelink communication at the time of the transmission; or

2>	if the transmission of the MAC PDU is prioritized over sidelink transmission or can be simultaneously performed with sidelink transmission:

3>	instruct the physical layer to generate a transmission according to the stored uplink grant.

If a HARQ process receives downlink feedback information, the HARQ process shall:

1>	stop the cg-RetransmissionTimer, if running;

1>	if acknowledgement is indicated:

2>	stop the configuredGrantTimer, if running.

If the configuredGrantTimer expires for a HARQ process, the HARQ process shall:

1>	stop the cg-RetransmissionTimer, if running.

The transmission of the MAC PDU is prioritized over sidelink transmission or can be performed simultaneously with sidelink transmission if one of the following conditions is met:

-	if there are both a sidelink grant for transmission of NR sidelink communication and a configured grant for transmission of V2X sidelink communication on SL-SCH as described in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and neither the transmission of NR sidelink communication is prioritized as described in clause 5.22.1.3.1 nor the transmissions of V2X sidelink communication is prioritized as described in clause 5.4.2.2 of TS 36.321 [22]; or

-	if there are both a sidelink grant for transmission of NR sidelink communication and a configured grant for transmission of V2X sidelink communication on SL-SCH as described in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and the MAC PDU includes any MAC CE prioritized as described in clause 5.4.3.1.3 or the value of the highest priority of the logical channel(s) in the MAC PDU is lower than ul-PrioritizationThres if ul-PrioritizationThres is configured; or

-	if there are both a sidelink grant for transmission of NR sidelink communication and a configured grant for transmission of V2X sidelink communication on SL-SCH as described in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and the MAC entity is able to perform this UL transmission simultaneously with both the transmission of NR sidelink communication which is prioritized as described in clause 5.22.1.3.1a and the transmissions of V2X sidelink communication which are prioritized as described in clause 5.14.1.2.2 of TS 36.321 [22]; or

-	if there is only configured grant(s) for transmission of V2X sidelink communication on SL-SCH as described in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and either none of the transmissions of V2X sidelink communication is prioritized as described in clause 5.4.2.2 of TS 36.321 [22] or the MAC entity is able to perform this UL transmission simultaneously with the transmissions of V2X sidelink communication which are prioritized as described in clause 5.14.1.2.2 of TS 36.321 [22]; or

-	if there is only a sidelink grant for transmission of NR sidelink communication at the time of the transmission, and if the MAC PDU includes any MAC CE prioritized as described in clause 5.4.3.1.3, or the transmission of NR sidelink communication is not prioritized as described in clause 5.22.1.3.1a, or the value of the highest priority of the logical channel(s) in the MAC PDU is lower than ul-PrioritizationThres if ul-PrioritizationThres is configured, or there is a sidelink grant for transmission of NR sidelink communication at the time of the transmission and the MAC entity is able to perform this UL transmission simultaneously with the transmission of NR sidelink communication which is prioritized as described in clause 5.22.1.3.1a:

-	if there are both a sidelink grant for transmission of NR sidelink communication and a configured grant for transmission of V2X sidelink communication on SL-SCH as described in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and either only the transmission of NR sidelink communication is prioritized as described in clause 5.22.1.3.1a or only the transmissions of V2X sidelink communication is prioritized as described in clause 5.4.2.2 of TS 36.321 [22] and the MAC entity is able to perform this UL transmission simultaneously with the prioritized transmission of NR sidelink communication or V2X sidelink communication:

NOTE 1:	Among the UL transmissions where the MAC entity is able to perform the transmission of NR sidelink communication prioritized simultaneously, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed.

NOTE 2:	Among the UL transmissions that the MAC entity is able to perform simultaneously with all transmissions of V2X sidelink communication prioritized, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed.

NOTE 3:	Among the UL transmissions where the MAC entity is able to perform the transmission of NR sidelink communication prioritized simultaneously with all transmissions of V2X sidelink communication prioritized, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed.

NOTE 4:	If there is a configured grant for transmission of V2X sidelink communication on SL-SCH as described in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and the MAC entity is not able to perform this UL transmission simultaneously with the transmission of V2X sidelink communication, and prioritization-related information is not available prior to the time of the transmission due to processing time restriction, it is up to UE implementation whether this UL transmission is performed.

------------------------------- [Changes End] -----------------------------