3GPP TSG-RAN2 Meeting #112-e	R2-2009952

eMeeting, 2nd – 13th November, 2020

Agenda Item:	6.9.3 Control plane Corrections

Source: 	Ericsson

Title:  	Way forward relaxed RRM requirements in RAN2 and RAN4

Document for:	Discussion and Decision

# Introduction

The relaxed RRM measurement requirements specified in 38.304 and 38.133 are further discussed in this contribution.

# Background

## NB-IOT/MTC/CE

38.304:

The Measurement rules for cell re-selection are specified in section 5.2.4.2.

The Relaxed monitoring measurement rules (including Relaxed monitoring criterion) are specified in section 5.2.4.12.

38.133:

The measurement requirements for cell re-selection are specified in section 4.6.2.

For example, for intra-frequency measurement requirements (Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC) in section 4.6.2.2 it is referred to 38.304 for “relaxed monitoring criteria”:

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6.2.2-1 and Table 4.6.2.2-2.

## NR

38.304:

The Measurement rules for cell re-selection are specified in section 5.2.4.2.

The Relaxed measurement rules (including Relaxed monitoring criterion for low mobility/not at cell edge) are specified in section 5.2.4.9.

38.133:

The measurement requirements for cell re-selection are specified in section 4.6.2.

The measurement requirements for cell re-selection are specified in section 4.2.2.

For example, for intra-frequency measurement requirements (Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC) in section 4.6.2.2 it is referred to 38.304 for “relaxed monitoring criteria”:

##### 4.2.2.9.1 	Introduction

This section contains the requirements for measurements on intra-frequency NR cells when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.X.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

##### 4.2.2.9.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on intra-frequency NR carrier and

-	UE is configured with low mobility criterion, or

-	UE is configured with both low mobility criterion and not-at-cell edge criteria but has fulfilled only the low mobility criterion.

The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.1-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.1-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.1-1.

# Discussion

There is a common understanding that the same requirements should be not be duplicated in different specifications, especially when those specifications are maintained in different working groups.

For LTE the trigger conditions are captured in 38.304, while the measurement requirements are captured in 38.133. But for NR the trigger conditions to (to great extend) are captured in both 38.304 and 38.133, i.e. duplicated. This may lead to problems, when not exact the same wording is used in both specifications, and it complicates maintenance.

To avoid overlap in the future (e.g. when relaxed RRM is introduced for RedCap UE) it is proposed that some general rules are discussed and agreed about what requirements should be captured in 38.304 and what requirements in 38.133 for relaxed RRM measurements:

Proposal: For relaxed RRM measurements the trigger conditions are captured 38.304 and the measurement requirements are captured in 38.133.

# Summary

RAN2 is kindly asked to discuss how to avoid duplication of relaxed RRM measurement requirements in 38.304 and 38.133:

Proposal: For relaxed RRM measurements the trigger conditions are captured 38.304 and the measurement requirements are captured in 38.133.