| 3GPP TSG RAN WG2 Meeting #112-e | R2-2009972 |
| --- | --- |

e-Meeting, November 2nd  – November 13th, 2020

Agenda Item:	8.7.3.3.

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	NR Sidelink Relay (Re-)Selection Criterion and Procedure

Document for:	Discussion and Decision

# Introduction

A new study item for Rel-17 on NR sidelink relay has been approved at RAN#86 [1]. The objectives of the SI are as follows:

This study item targets to study single-hop NR sidelink-based relay.

- Study mechanism(s) with minimum specification impact to support the SA requirements for sidelink-based UE-to-network and UE-to-UE relay, focusing on the following aspects (if applicable)  for layer-3 relay and layer-2 relay [RAN2];

  - Relay (re-)selection criterion and procedure;

  - Relay/Remote UE authorization;

  - QoS for relaying functionality;

  - Service continuity;

  - Security of relayed connection after SA3 has provided its conclusions;

  - Impact on user plane protocol stack and control plane procedure, e.g., connection management of relayed connection;

- Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying, assuming no new physical layer channel / signal [RAN2];

NOTE 1: The study shall take into account of further input from SA WGs, e.g., SA2 and SA3, for the bullets above (if applicable).

NOTE 2: It is assumed that UE-to-network relay and UE-to-UE relay use the same relaying solution.

NOTE 3: Forward compatibility for multi-hop relay support in a future release needs to be taken into account.

NOTE 4: For layer-2 UE-to-network relay, the architecture of end-to-end PDCP and hop-by-hop RLC, e.g., as recommended in TR 36.746, is taken as starting point.

In this document, 1A from the points in the above-mentioned study item, i.e. relay (re-)selection criterion and procedure, will be discussed.

# Relay (Re-)Selection Criteria

As discussed in the email discussion [2], several criteria can be applied to trigger relay (re-)selection. Generally, the criteria can be classified into two main categories, namely Access Stratum (AS) criteria and higher layer criteria [2]. Both types might be applied in both L2 and L3 architectures [3,4].

The following AS layer criteria, as elaborated in the next subsections, might be applied in the relay (re-)selection procedure.

## RSRP

RSRP is an essential criterion in the Access Stratum to be considered for relay (re-)selection. The sidelink and Uu RSRP can be considered in the remote UE as well as a candidate relay for the relay (re-)selection procedure and was one of the options mentioned in the email discussion [2].

Several companies pointed out that the Uu RSRP would not be necessary as this would have already been a part of the criteria for a UE to serve as a relay, e.g., based on the lower and upper thresholds and hence would exclude links with poor a signal quality. However, we believe that providing the remote UE with the Uu RSRP, in addition to the PC5 RSRP, of all the potential relays can help the remote UE to make a better choice.

RAN2 to study the case where both PC5 and Uu RSRP are provided in a remote UE for the relay (re-)selection.

## Relay UE Capability

On the other hand, we also understand that solely having a good Uu RSRP does not result in a high throughput on the Uu interface. This would depend on the various capabilities of the relay UE like for e.g., Carrier Aggregation (CA) or Dual Connectivity (DC). Therefore, even if several relay UE(s) might satisfy the Uu RSRP criterion and might be able to provide the required QoS, it would be beneficial for the remote UE to make a decision also based on the capability of the relay UE, i.e. based on how the required QoS can be satisfied as well as whether the relay UE will be able to support mechanisms enabling high throughput.

## HARQ Feedback

Other AS criteria such as HARQ feedback can also be considered in the relay (re-)selection procedure. In case the number of HARQ NACK feedbacks exceeds a (pre-)configured threshold, the remote UE or gNB may start the relay (re-)selection procedure.

## Relay-UE Load

The load at the relay UE could be an additional criterion for the remote UE(s) to assess the capability of the relay UE to provide the required QoS.

An option at the relay UE could be that if the load of a relay UE exceeds a certain threshold, it should exclude certain services for relaying that it could have originally supported. The exclusion of these services can be done by not including the corresponding relay service code as a part of the discovery messages. There are multiple ways to quantify the load at the relay UE which are applicable to both U2N and U2U relays. For example:

- Number of PC5-RRC connections currently supported for relaying,

- How often the relay occupies the resource pool for itself and how many remote UEs it serves,

- How often the data arrives from the remote UE(s) within a particular time window,

- Data rate as could be measured at the different layers of the relay UE(s) for relaying data.

Option A might not be a good measure of the load because even though the relay might have multiple active connections, not all the remote UE(s) will be transmitting at the same time. Therefore, despite multiple connections, the relay UE might not be fully loaded. The problem with option A is that there is no information about the volume of the relaying data being transmitted. B, C, D are better options as they quantify in some ways the volume of data being relayed between the source and destination. Furthermore, specifying these parameters will enable the different vendors to assess the load statistics in a uniform manner. In addition, depending on the relaying solution, the gNB could also provide this information.

RAN2 to study the following additional criteria to be included as a part of relay (re-)selection and procedures to provide corresponding information of candidate relays to the remote UEs:

- a.	Relay UE capability,

- b.	HARQ feedback,

- c.	Relay UE load.

## Entity Triggering Relay (re-)Selection

The following entities in a network might trigger the relay (re-)selection decision:

- Autonomously by remote UE: In-coverage or out-of-coverage UEs may apply AS and higher layer criteria to (re-)select a relay UE out of possible relay UEs autonomously without any gNB assistance. This option is crucial for out-of-coverage remote UEs which have no communication with any gNB. For in-coverage UEs, the autonomous relay (re-)selection may reduce processing load in gNB. Regarding the relay UE architecture, this option is necessary with a L3 relay architecture where the remote UE is not visible to the gNB and L2 relay architectures in case which a remote UE is out-of-coverage.

- By gNB: This case is relevant for L2 relay architecture where the relay (re-)selection is  under control of a gNB or L3 in case of in-coverage remote UE using direct path to the gNB.

- Remote UE-assisted by gNB: In-coverage remote UEs may receive assistance information from gNB to do the relay (re-)selection. Out-of-coverage remote UEs may also receive assistance information from gNB via the relay node to trigger the (re-)selection procedure. The assistance information can be relevant for AS layer criteria for relay (re-)selection. This option is feasible again for L2 relay architecture and in case of in-coverage remote UE for L3 relay.

The above mentioned options are dependent on the architecture of a relay UE and coverage scenario of a remote UE.

## Impact of L2 or L3 Architecture on Relay (re-)Selection

The relay (re-)selection procedure and criteria should be applicable for both L2 and L3 relay architectures and UE-to-UE and UE-to-Network relays without need for any modification.

RAN2 to study the applicability of the relay (re-)selection procedure and criteria for both L2 and L3 relay architectures.

## Candidate Relay Set in Remote UEs

The (re-)selection procedure might be time consuming to find an appropriate relay UE meeting all AS and higher layer criteria when a re-selection criteria is met. All UEs which meet the relay selection criteria can be listed by the gNB. This list can be sent to a remote UE along with other information such as resource pool, thresholds for (re)-selection etc. The final relay (re-)selection can be left up to UE implementation. The candidate list assists a remote UE in a faster relay (re-)selection procedure.

The relay re-selection procedure might lead to higher latencies due to the need for discovery procedure, measurements on PC5 and Uu interfaces and checking for the higher layer criteria.

A list of candidate relay UEs stored in a remote UE might accelerate the relay re-selection procedure.

# Resources for Relay UEs

Two options might be studied to be considered for resources in a relay UE for transmission of messages between two remote UEs or between a remote UE and gNB. The options are follows:

- Dedicated resource pool: as relaying imposes extra load on a relay UE, additional resources might be needed if the relayed data flow requires high reliability or low latency. A dedicated resource pool can be configured by gNB or pre-configured in a relay UE to be employed in the relaying of messages. The separation of resources for relay functionality can improve the reliability and latency.

- Common resource pool: a relay UE may use the common resource pool for relaying purpose. However, if the load of the relay node and the congestion of common resource pool are not considered in the relay (re-)selection procedure, the relay node might be unable to provision the QoS requirements of data flows.

By using the common resource pool to relay messages, the relay UE might not be able to provision QoS requirements.

RAN2 to study the feasibility of a dedicated resource pool for relaying.

# Conclusions

The following observations and proposals have been made in this document:

- The above mentioned options are dependent on the architecture of a relay UE and coverage scenario of a remote UE.

The relay re-selection procedure might lead to higher latencies due to the need for discovery procedure, measurements on PC5 and Uu interfaces and checking for the higher layer criteria.

By using the common resource pool to relay messages, the relay UE might not be able to provision QoS requirements.

- RAN2 to study the case where both PC5 and Uu RSRP are provided in a remote UE for the relay (re-)selection.

- RAN2 to study the following additional criteria to be included as a part of relay (re-)selection and procedures to provide corresponding information of candidate relays to the remote UEs:

- a.	Relay UE capability,

- b.	HARQ feedback,

- c.	Relay UE load.

RAN2 to study the applicability of the relay (re-)selection procedure and criteria for both L2 and L3 relay architectures.

A list of candidate relay UEs stored in a remote UE might accelerate the relay re-selection procedure.

RAN2 to study the feasibility of a dedicated resource pool for relaying.

# References

[1] RP-193253, “New SID on NR sidelink relay”, LG Electronics, 3GPP RAN#86, Dec 2019.

[2] [Post111-e][622][Relay] “Relay selection and reselection”.

[3] TS 23.303, “Technical Specification Group Services and System Aspects; Proximity-based services (ProSe); Stage 2, (Release 16)”, V16.0.0, (2020-07).

[4] TR 36.746, “Study on further enhancements to LTE Device to Device (D2D), User Equipment (UE) to network relays for Internet of Things (IoT) and wearables”, V15.1.1, (2018-04).