3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2009973

Online, November 2nd - 13th, 2020

Agenda item:		8.6.5 Aspects specific to CG based schemes

Source:		NEC

Title:		Discussion on CG-based Small Data Transmissions

Document for:		Discussion

# 1. Introduction

During RAN2 #111-e, it was agreed that:

- Data volume threshold is used for the UE to decide whether to do SDT or not.   FFS how we calculate data volume.

- UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported

- When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant.  FFS on details and whether any indication to network is needed.

During email discussion [1], it was proposed:

Proposal 3: New TAT is defined for the TA maintenance in INACTIVE state for CG case. Detailed UE behavior upon expiry of this timer needs discussion (i.e. FFS whether to release CG resources or some other action) – (16/22)

In this contribution, we discuss the mechanism to choose between RACH- and CG-based SDT, configuration/reconfiguration/release of CG resources and CG maintenance/validity after the TAT expires.

# 2. Discussion

2.1 Prioritization of RACH- vs CG-based SDT

It was agreed that a data volume threshold would be used to trigger SDT, but it is not clear if it is applicable for CG-based SDT or not. This agreement was proposed under the UL small data transmissions for RACH-based schemes AI, and we think that the data volume threshold should also be used for CG-based SDT.

Proposal 1: RAN2 to confirm that the data volume threshold is also used for CG-based SDT.

Once the SDT threshold is passed, the UE must choose which scheme should be used if radio resource for both CG-based SDT and RACH-based SDT are available and valid (e.g. TAT is running). We think that CG-based scheme should always be selected, because it consists in less signalling and thus less battery consumption. It is also likely that the CG will better match the small data pattern and size and having a CG would otherwise be a waste.

Proposal 2: If CG for SDT is available and valid, CG-based SDT should be always selected with higher priority than RACH-based SDT.

2.2 CG configuration during INACTIVE state

As agreed during the RAN2 #111-e meeting, SDT in INACTIVE state should avoid switching to CONNECTED state, which represents a high energy and signalling overhead for UEs. Currently, CGs configured during CONNECTED state are released when transitioning to INACTIVE or IDLE state.

In this contribution, we discuss two cases:

- Case 1: Configuration of the CG for SDT when the UE is transitioning from CONNECTED state to INACTIVE sate

- Case 2: Reconfiguration/release of the CG for SDT during INACTIVE state.

Case 1: Configuration of CG for SDT when transitioning from CONNECTED to INACTIVE

There are two options to allow a UE to have a CG after switching to INACTIVE state:

- Reuse a CG that was configured and used during CONNECTED state

- Configure a new CG specific to SDT while transitioning to INACTIVE state

Option 1 is much simpler and has less signalling overhead. Considering the DRB characteristics (e.g. QoS, traffic pattern) may not change after switching to INACTIVE state, it is reasonable to reuse the CG configured in CONNECTED. On the other hand, option 2 gives more flexibility such that the gNB can configure more sparse radio resources (time and/or frequency), as the UE switches to INACTIVE state and this would mean fewer occasions to generate UL data. On the other hand, both options may be beneficial depending on the network or operator policy. For example, if the gNB configures the CG in CONNECTED and considers it can be reused for the SDT, the gNB can indicate to the UE that the CG is to be reused for SDT. Otherwise, the gNB can configure new CG for the SDT.

We further discuss how to achieve above options. In option 1, the gNB can just reuse the configured CG but needs to indicate to the UE whether it will be released or kept for the SDT. Regarding the timing of such indication, the RRC Release seems better suited than the RRC Reconfiguration in CONNECTED, because the gNB will decide if the SDT is allowed or not at later stage, e.g. when deciding the RRC Release procedure. In option 2, it is more straightforward to do it during the RRC Release. Thus, we think that the RRCRelease message is suitable to achieve both options. One option to achieve this is to add a SetupRelease type IE, which can be used to indicate to release any CG used in CONNECTED state or configure new CG for SDT.

Proposal 3: RAN2 to agree that the RRCRelease message is used to configure the CG for SDT while transitioning to RRC Inactive mode.

Case 2: Reconfiguration/release of CG for SDT while in INACTIVE

The gNB may want to reconfigure or release the CG for SDT while the UE is performing SDT in INACTIVE state. Since an RRC-based solution is considered as a baseline for SDT, the gNB would include a DL RRC message to respond to an UL RRC message or to end the SDT procedure, without switching to CONNECTED state. A solution to reconfigure or release the CG for SDT while in INACTIVE state could therefore be achieved with this RRC message during the SDT procedure.

One remark is that even though RAN2 agreed to apply the RRC-based solution for the SDT, the exact RRC message may not be the same as legacy RRC Resume Request or RRCRelease messages but a new RRC message similar to the RRCRelease message.

Proposal 4: RAN2 to agree that the RRCRelease message (or similar) sent during/at the end of SDT procedure is used to reconfigure or release the CG for SDT while in INACTIVE.

According to previous proposals, in both cases (configuration while in CONNECTED state and reconfiguration/release while  in INACTIVE state), a SetupRelease type IE can be used to release any CG used before or (re)configure a new CG used for SDT afterwards. If the RRC message sent during or at the end of SDT procedure is the RRCRelease message, this would be beneficial to use a common RRCRelease message for both cases.

Proposal 5: RAN2 to introduce a SetupRelease type IE in the RRCRelease (or similar) message to configure, reconfigure or release a CG for SDT when transitioning from CONNECTED to INACTIVE state.

Proposal 6: RAN2 to introduce a SetupRelease type IE in the RRCRelease (or similar) message to configure, reconfigure or release a CG for SDT during the Small Data Transmission procedure in INACTIVE state.

2.3 TA Maintenance and validity

Another issue of CG is the time alignment timer that expires fast, thus disabling CG. To that end, a multiplier was introduced for PUR that allows the timer to be longer, up to a day. It has been proposed during email discussion [1] that a new TAT would be introduced for SDT. This new SDT-TAT is expected to last longer than legacy TAT but we still need to discuss what happens when this timer expires and how to maintain it and the CG.

Once the SDT-TAT has expired, CG should not be used, however It would be beneficial to allow the UE to store the configured grant. Since the UE can only use RACH-based SDT, it will need to choose RACH-based SDT for the next small data transmission. At that time, a TA Command will be included in the RAR and time alignment will be reestablished. Hence the SDT-TAT could be restarted then, and the stored CG can be used again for the following small data transmissions. This would result in little to no signalling overhead as the UE would not periodically use RACH for the sole purpose of updating its TA but rather as part of a one-time RACH-SDT procedure when SDT is triggered and the TAT has expired.

Proposal 7: Upon expiry of its TAT (SDT-TAT), the UE shall store its CG configuration instead of releasing it.

Proposal 8: Upon receiving a TA Command, the UE shall (re)start its TAT(SDT-TAT) and its stored CG shall become valid/activated again.

# 3. Conclusion

In this contribution, we observed and proposed the following:

Proposal 1: RAN2 to confirm that the data volume threshold is also used for CG-based SDT.

Proposal 2: If CG for SDT is available and valid, CG-based SDT should be always selected with higher priority than RACH-based SDT.

Proposal 3: RAN2 to agree that the RRCRelease message is used to configure the CG for SDT while transitioning to RRC Inactive mode.

Proposal 4: RAN2 to agree that the RRCRelease message (or similar) sent during/at the end of SDT procedure is used to reconfigure or release the CG for SDT while in INACTIVE.

Proposal 5: RAN2 to introduce a SetupRelease type IE in the RRCRelease (or similar) message to configure, reconfigure or release a CG for SDT when transitioning from CONNECTED to INACTIVE state.

Proposal 6: RAN2 to introduce a SetupRelease type IE in the RRCRelease (or similar) message to configure, reconfigure or release a CG for SDT during the Small Data Transmission procedure in INACTIVE state.

Proposal 7: Upon expiry of its TAT (SDT-TAT), the UE shall store its CG configuration instead of releasing it.

Proposal 8: Upon receiving a TA Command, the UE shall (re)start its TAT(SDT-TAT) and its stored CG shall become valid/activated again.

# References

[1] Summary of email discussion [Post111-e][925][R17 Small Data] Agreeable details of RRC-based solution (RACH and CG), ZTE