3GPP TSG RAN WG2 Meeting #112-e                                                                  R2-2009992

Online, November, 2020

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Resource Allocation Enhancements

Agenda item:	8.15.3

Document for:	Discussion

# Introduction

The study item for NR V2X was approved in RAN#86 and revised in RAN#88e [1], and the following objectives were identified in relation to resource allocation enhancements:

The objective of this work item is to specify radio solutions that can enhance NR sidelink for the V2X, public safety and commercial use cases. 2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after [RAN#89].

This contribution discusses the resource allocation features of inter-UE coordination, and its ability to enhance reliability, reduce latency as well as the power saving aspect for Mode 2 UEs.

# Inter-UE Coordination

The WID [1] identifies inter-UE coordination as one of the possible resource allocation enhancement techniques to overcome the issue of unreliable or inadequate sensing results. This is due to known issues such as missing sensing results because of the half duplex constraint or partial sensing. Furthermore, the well-known hidden node problem increases the chances of resource allocation collisions. Enhancements should increase reliability and reduce latency on the sidelink.

Inter-UE coordination is achieved when a UE, e.g. UE-A, shares a set of resources with another UE, e.g. UE-B, as shown in Figure 1. This is done so that the UE-B can use this information in order to assist its process of resource selection. This was explored in depth during the study phase of Release 16, under the form of Mode 2(b) and Mode 2(d) [2]. The following sections highlight some of the features and implementation aspects of inter-UE coordination.

Figure 1: Inter-UE coordination: UE-A sends assistance information to UE-B.

Figure 2 shows an example of how inter-UE coordination can help when two UEs are outside each other’s range. Depicted here is the well-known hidden node problem, where UE-1 and UE-2 are too far apart and thus not within reception range of one another. Consequently, both UE-1 and UE-2 can select interfering resources for a simultaneous transmission to UE-3, resulting in a collision. Coordination of resources among these UEs can prevent such collisions and the RAN2 requirements for such a mechanism will be highlighted in the next sections.

Figure 2: Depiction of Resource Collisions due to the Hidden Node Problem.

## Content of Assistance Information

One of the defining aspects of inter-UE coordination are the set of resources that are transmitted by UE-A in order to assist UE-B as shown in Figure 1. There are two types of resource sets that are currently under consideration:

- a set of resources which is preferred for UE-B’s transmission,

- a set of resources which is preferred not to be used by UE-B.

In the case that the set of resources are preferred for UE-B’s transmission, UE-A sends a set of resources that it sees available during its sensing and resource selection process as shown in Figure 3. Based on the set of resources, UE-B can use these resources for its own intended transmissions.

Figure 3: Exemplary depiction of UE-B using a preferred set of resources from UE-A.

On the other hand, the set of resources that are preferred not to be used by UE-B are those resources that UE-A had detected to be occupied by other UEs for their respective transmissions. If UE-A is the intended recipient for a transmission from UE-B, UE-A can also indicate resources that UE-A already has scheduled and reserved for other transmissions. If UE-B transmits to UE-A using these resources, due to the half-duplex constraint, UE-A would not be able to receive the intended transmission from UE-B, and hence, these resources are not preferred, as shown in Figure 4.

Figure 4: Exemplary depiction of UE-B using a set of not preferred resources from UE-A.

The set of resources transmitted by UE-A can be combined with a candidate resource set or a set of selected resources on the PHY layer or the MAC layer of UE-B. If the assistance information is received in the MAC layer, it would be straightforward for the MAC layer to carry out the combination of the resource sets. This can for example be done by either finding common resources in both sets as shown in Figure 3, or excluding not preferred resources as shown in Figure 4. The detailed procedures for the combination of the resource sets needs to be discussed by RAN2.

Observation 1: Assistance information as a set of resources to use or avoid can be merged in the MAC layer with the candidate resource set given by the PHY.

Proposal 1: RAN2 should evaluate the procedures for merging assistance information in the MAC layer.

## Methods for transmitting Assistance Information

As sidelink transmissions are fixed in size across time and frequency, just transmitting assistance information by itself is very inefficient. Therefore, it is desirable to multiplex assistance information with other data transmissions or to multiplex several assistance information into a single transmission.

Observation 2: Sending assistance information on its own in L2 is very inefficient.

Adding assistance information to a unicast transmission as a MAC CE is a straight forward approach. However, message handling for groupcast and broadcast scenarios needs to be further looked at, especially when assistance information to different group members needs to be differentiated at the receiver side.

Alternatively, assistance information can be sent as a physical layer message, as also pointed out in our accompanying RAN1 TDoc [3].

Proposal 2: RAN2 should evaluate ways to reduce the impact on resource utilization caused by transmissions for assistance information.

## Relationship between Assistance Information and Resource Pools

The set of resources as discussed above is relevant for a given resource pool. It can be decided that the assistance message itself is transmitted using the resources of a given resource pool, with its content indicating assistance information regarding resources pertaining to the same resource pool. To increase signaling efficiency, it could also be possible for the assistance message to be transmitted in other resource pools. For this, the resource pool will need to be indicated in some way in order to link assistance information to a given resource pool.

Observation 3: Mapping of the received set of resources to a resource pool needs to be maintained.

## Setup and Conditions for Sending Assistance Information

The PHY sensing procedure differs depending on the transmission parameters. Here among others, the priority and packet delay budget play a role within the selection process. When UE-A selects a set of resources for UE-B, it should also perform the appropriate sensing procedure depending on the transmission parameters required by UE-B. Thus, the parameters used by UE-A in order to determine the set of resources will either need to be pre-configured or can be adapted, by UE-B sending a request for the assistance message. Depending on the expected content, the request will likely need to indicate how UE-B will utilize the assistance information, for UE-A to modify sensing procedures and to generate the set of resources. This is because the parameters related to the sensing procedure changes with priority, packet delay budget and feedback mode.

Observation 4: To generate the assistance information by another UE, parameters about its intended use will be needed.

There are two main scenarios where assistance information can increase performance. One is a periodic case, where a UE-A assists UE-B in resource selection and sends a selected set of resources periodically to UE-B. This case can be setup via PC-5 RRC. Another case is that UE-A is triggered to send assistance information to UE-B. An example of a trigger could be the CBR-level of a resource pool or alternatively, UE-A could be triggered to assist UE-B when it has consistent failed receptions from UE-B, for example, due to packet collisions with transmissions from another UE.

Proposal 3: We propose to consider the following design aspects of inter-UE coordination:

- Additional parameters needed to generate the assistance information,

- Actions that trigger the assistance information procedure

# Power Saving

In order to cater to power constrained UEs in LTE Rel-14, two procedures were introduced to reduce the power consumption – random resource selection and partial sensing. Although these procedures worked to a large extent for Rel-14, the introduction of support for aperiodic traffic, along with periodic traffic, coupled with the requirements for enhanced reliability and low latency would make the use of these procedures directly challenging. These topics are handled by RAN1, and are discussed in our accompanying contribution [4].

Since the WID considers new solutions for power saving, we explore options to make communications more power efficient by leveraging RAN2 mechanisms. This can be done with DRX and inter-UE coordination by reducing sensing and reception while maintaining reliability through assistance information.

However, especially for Pedestrian UEs (P-UE) that are not always in the vicinity of other vehicles, it makes sense to limit sidelink communication to times when they are vulnerable road users. When people spend their time in buildings or away from roads, power saving can be achieved by turning off vehicular communications. The following section explores this approach.

Observation 5: P-UEs can save power by only transmitting when in the vicinity of other vehicles.

## Location-based Power Saving

Rel-17 is targeted at P-UEs, public safety UEs and commercial UEs that require power consumption to be minimized. These particular UEs actively transmit and receive only when in certain defined locations. For example, pedestrian UEs are required to be active while close to roads and intersections, and not when indoors. Similarly, public safety UEs need to be active only when at emergency locations, and not otherwise as shown in Figure 5.

In order to take advantage of this feature, it is possible to restrict a power saving UE’s activity based on its location. By defining pre-configured locations or regions, these UEs need to wake up and carry out transmissions only when they are within these regions. It is also possible to utilize the DRX feature to wake up more often in these regions, as well as combine it with assisted resource allocation, in order to maximize the power saving capabilities.

Inactive P-UEs

Active P-UEs

Figure 5: Depiction of active and inactive P-UEs based on their location – active outdoors, inactive indoors.

Proposal 4: Since power saving UEs are required to be active based on their location, we propose to evaluate how to enable these UEs to wake up and carry out transmissions only when they are in these regions.

# Conclusion

The following observations have been made in this document:

Observation 1: Assistance information as a set of resources to use or avoid can be merged in the MAC layer with the candidate resource set given by the PHY.

Observation 2: Sending assistance information on its own in L2 is very inefficient.

Observation 3: Mapping of the received set of resources to a resource pool needs to be maintained.

Observation 4: To generate the assistance information by another UE, parameters about its intended use will be needed.

Observation 5: P-UEs can save power by only transmitting when in the vicinity of other vehicles.

Based on the observations, the following are the proposals made:

Proposal 1: RAN2 should evaluate the procedures for merging assistance information in the MAC layer.

Proposal 2: RAN2 should evaluate ways to reduce the impact on resource utilization caused by transmissions for assistance information.

Proposal 3: We propose to consider the following design aspects of inter-UE coordination:

- Additional parameters needed to generate the assistance information,

- Actions that trigger the assistance information procedure

Proposal 4: Since power saving UEs are required to be active based on their location, we propose to evaluate how to enable these UEs to wake up and carry out transmissions only when they are in these regions.

# References

- RP-201385, “WID revision: NR sidelink enhancement”, LG Electronics, 3GPP RAN#88e, June-July 2020.

- TR 38.885 v16.0.0, “Study on NR Vehicle-to-Everything (V2X) (Release 16)”, June 2020.

- R1-2008757, “Resource Allocation Enhancements for Mode 2”, Fraunhofer HHI, Fraunhofer IIS, RAN1#103e, October-November 2020

- R1-2008817, “NR Sidelink resource allocation for UE power saving”, Fraunhofer IIS, Fraunhofer HHI, RAN1#103e, October-November 2020.