3GPP TSG-RAN WG2 Meeting #112e	R2-2010006

Online, November 2nd - 13th, 2020

Agenda item:	8.6.4

Source:	Qualcomm Incorporated

Title:	Discussion on RACH based NR small data transmission

WID/SID:	NR_SmallData_INACTIVE-Core – Release 17

Document for:	Discussion and Decision

# Introduction

In RAN2 #111e meeting, several agreements were made related to the RACH based NR small data transmission.

Agreements

1 	Small data transmission with RRC message is supported as baseline for RA-based and CG based schemes

2	RRC-less can be studied for limited use cases (e.g. same serving cell and/or for CG) with lower priority

3	Context fetch and data forwarding with anchor re-location and without anchor re-location will be considered.   FFS if there are problems with the scenario “without anchor relocation”.

4	From RAN2 perspective, stored “configuration” in the UE Context is used for the RLC bearer configuration for any SDT mechanism (RACH and CG).

5	The 2-step RACH or 4-step RACH should be applied to RACH based uplink small data transmission in RRC_INACTIVE

6	The uplink small data can be sent in MSGA of 2-step RACH or msg3 of 4-step RACH.

7	Small data transmission is configured by the network on a per DRB basis

8	Data volume threshold is used for the UE to decide whether to do SDT or not.   FFS how we calculate data volume.

FFS if an “additional SDT specific” RSRP threshold is further used to determine whether the UE should do SDT

9	UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported

10	When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant.  FFS on details and whether any indication to network is needed.

During email discussion, there are still some remaining issues that needs more discussion. In this paper, we would like to discuss more details on RACH based small data transmission scheme and provide our views.

# Discussion

Separate RACH resource for SDT and legacy user

In the email discussion [Post111-e][925][R17 Small Data], how to configure RACH resource for the small data transmission (SDT) users and legacy RACH users has been discussed. It seems that companies might have different understanding on the configuration for RACH resource.

In our view, the configuration for RACH resource includes the PRACH occasions configuration and preamble configuration. It could be beneficial to configure PRACH occasions separately so that UE could perform RACH from the separate resources for different reasons, especially if there are a large number of users performing RACH for one reason in one cell. Thus, separate PRACH occasions configuration should be supported for SDT and legacy users. On the other hand, shared PRACH occasion configuration could avoid capacity loss in high cell load. Then the preamble partitioning is needed for SDT and legacy users to enable the UE to differentiate the MSGB or Msg2 reception. Therefore, both options should be supported, and it is up to network to configure which option in different scenarios.

Proposal 1: For RACH based small data transmission, both options should be supported for small data user and legacy RACH user, i.e. separate PRACH occasion or shared PRACH occasion with preamble partitioning.

Subsequent small data transmission

In the last meeting, it has been agreed that the data volume threshold is used for the UE to decide whether to do SDT or not. Thus, if network allows UE to transmit small data in RRC_INACTIVE and UE potential uplink data is smaller than the configured data volume threshold, such as a TB size threshold, UE initiates uplink small data transmission by selecting a suitable grant size which can accommodate the user’s small data. However, in some cases, UE could not finish small data transfer within RACH procedure. For example,

- If there is no suitable grant resource for UE to transmit all data, UE can only send the regular CCCH message together with the partial or segmented user uplink data in the selected resource.

- Network could not know what type of small data traffic patten UE may have in the near future, for instance, either an one-shot small data traffic or a multi-shot small data traffic, or the periodicity of traffic arrival as well as the amount of data that UE may have each time. For the multi-shot traffic pattern, the traffic modelling can be like this. UE sends the first object in the first uplink small data transmission, and UE’s buffer becomes empty. Then network sends ACK in response to the UE first uplink data transmission. After a gap, UE’s application layer may generate more objects to send. In multi-shot traffic, after UE first uplink data transmission in RACH, network does not know whether UE may have more data to send after network sends the responses.

- The PUSCH resource is contention based in the configured CBRA resource. If network could configure the dedicated PUSCH resource to UE for small data transfer, it could reduce the transmission error for the user data, and further reduce the latency and save the radio resource.

Observation 1: The buffer status report information reflects the current amount of data that UE may have.

Observation 2: After UE finishes the first uplink data transmission during RACH procedure, it is difficult for network to know whether UE has more uplink data to send without UE assistance information.

Therefore, it is beneficial to allow UE to send UE assistance information (i.e. small data request message) in MSGA or Msg3 in RACH procedure. The small data request message can be used to indicate the UE’s current buffer status, as well as to provide traffic pattern information to assist network to make decision to allow UE to continue the subsequent uplink data transmission or to transit UE to RRC_CONNECTED state.

Observation 3: The UE assistance information provides traffic pattern information which could assist network to make decision allowing UE to continue the subsequent uplink data transmission or to transit UE to RRC_CONNECTED.

When network successfully receives the UE assistance information in MSGA or Msg3, network could schedule UE with dynamic grant with appropriate granted resource for one-shot small data transfer. Network could also configure dedicated PUSCH resource with multiple resource occasions (i.e. configured grant) for UE subsequent data transfer in RRC_INACTIVE.

Based on the UE assistance information, it would be up to network to allow the UE to continue to operate in RRC_INACTIVE or to transit to RRC_CONNECTED if network believes the future data traffic pattern is a large data transmission which is not suitable to be handled in RRC_INACTIVE.

The UE assistance information could be sent along with the CCCH message as well as the optional segmented user data if the whole MAC PDU can be accommodated into the granted resource in MSGA or Msg3.

Proposal 2: UE is allowed to send UE assistance information during RACH procedure to request network to configure dedicated PUSCH resource for UE subsequent small data transfer.

After network receives UE assistance information, network configures the dedicated PUSCH resource (such as configured grant). The configured grant configuration could be contained in the RRC release message transmitted in MSGB or Msg4 together with other downlink data. If UE receives the configured grant resource from network, UE continues to transmit the subsequent uplink data on the configured grant resource w/o moving to RRC_CONNECTED. UE transmits the data of multi-shot traffic in the multiple configured PUSCH resources. The rest procedure by using configured grant resource should be quite similar or the same as the CG based small data transmission scheme.

Proposal 3: When UE receives the dedicated PUSCH resource configuration during RACH procedure, UE sends the small data on the dedicated PUSCH resource.

Figure 1 and Figure 2 show the examples of call flows on RACH based small data transmission via 2-step or 4-step RACH with subsequent uplink data transmission respectively.

Figure 1 UL small data via 2-step RACH with subsequent UL data transmission

Figure 2 UL small data via 4-step RACH with subsequent UL data transmission

In legacy RACH procedure, upon contention resolution, UE acquires C-RNTI from either the successRAR in 2-step RACH or from the value of TC-RNTI in 4-step RACH. Thus, after RACH procedure is successful, UE in RRC_INACTIVE could be able to monitor the PDCCH addressed to C-RNTI receiving the further UL and/or DL dynamic scheduling. In that case, network should configure corresponding search space, i.e. a common search space or UE-specific search space to UE for PDCCH monitoring. Because it is not a usual case to allow a UE in RRC_INACTIVE to monitor PDCCH addressed by C-RNTI in common search space. Hence, we think the details about the search space configuration for PDCCH monitoring in the SDT scenario should be discussed, and it should be decided by RAN1.

On the other hand, it is not good enough for UE to only rely on dynamic grant for subsequent data transmission. Dynamic grant could only handle the data stored in UE buffer currently, i.e. one-shot traffic. Once UE’s buffer becomes empty and more data is arrived with a gap, UE has no chance to send SR to request network scheduling more uplink resource in RRC_INACTIVE. UE has to perform RACH procedure for SDT again to transmit the new arrived data. It is obviously inefficient to repeat RACH procedure to handle additional small data in multi-shot traffic case.

Taking the above analysis into account, we think other solutions for subsequent small data transfer should also be considered for RACH based solution. That is network provides the dedicated PUSCH resource based on UE assistance information, i.e. configured grant for subsequent small data transmission.

Proposal 4: UE monitors PDCCH addressed to C-RNTI in a configured search space for DL and/or UL dynamic scheduling for subsequent small data transmission.

Proposal 5: The configuration of search space for PDCCH monitoring for the subsequent small data transmission should be decided by RAN1.

UE assistance information

The UE assistance information (also called small data request message) can be an RRC message or new RRC parameters in the existing RRC message or a MAC CE. The UE assistance information message should be able to provide network more UE information such as the amount of data in the buffer and/or the coming small data traffic pattern. The following contents should be considered in the UE assistance information.

- Buffer status information, to indicate the amount of data UE may already have in the buffer;

- Small data traffic pattern, to indicate the type of small data traffic either one-shot traffic or multi-shot traffic so that network may provide a set of dedicated PUSCH resource. The traffic pattern can also optionally include the periodicity of traffic arrival and the possible amount of data for each traffic occasion in case of a multi-shot traffic.

Proposal 6: The UE assistance information at least contains current buffer status, traffic pattern indication with one-shot or multi-shot, periodicity of traffic arrival and estimated amount data of each shot.

Proposal 7: The UE assistance information can be RRC message or MAC CE.

# Conclusion

We have the following observations:

Observation 1: The buffer status report information reflects the current amount of data UE may have only.

Observation 2: After UE finishes the first uplink data transmission during RACH procedure, it is difficult for network to know whether UE has more uplink data to send without UE assistance information.

Observation 3: The UE assistance information provides traffic pattern information which could assist network to make decision allowing UE to continue the subsequent uplink data transmission or to transit UE to RRC_CONNECTED.

We’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: For RACH based small data transmission, both options should be supported for small data user and legacy RACH user, i.e. separate PRACH occasion or shared PRACH occasion with preamble partitioning.

Proposal 2: UE is allowed to send UE assistance information during RACH procedure to request network to configure dedicated PUSCH resource for UE subsequent small data transfer.

Proposal 3: When UE receives the dedicated PUSCH resource configuration during RACH procedure, UE sends the small data on the dedicated PUSCH resource.

Proposal 4: UE monitors PDCCH addressed to C-RNTI in a configured search space for DL and/or UL dynamic scheduling for subsequent small data transmission.

Proposal 5: The configuration of search space for PDCCH monitoring for the subsequent small data transmission should be decided by RAN1.

Proposal 6: The UE assistance information at least contains current buffer status, traffic pattern indication with one-shot or multi-shot, periodicity of traffic arrival and estimated amount data of each shot.

Proposal 7: The UE assistance information can be RRC message or MAC CE.

# References

- RP-193252, New Work Item on NR small data transmissions in INACTIVE state, ZTE.

# Annex