3GPP TSG-RAN WG2 #112-e	Tdoc R2-2010050

Electronic meeting, November 2nd – 13th, 2020

Agenda Item:	6.1.2

Source:	Ericsson

Title:	Correction for SPS capability

Document for:	Discussion, Decision

# Introduction

This paper discusses a correction for SPS capability.

# Discussion

From 38.331 V15.11.0

### 6.3.2 Radio resource control information elements #### – SPS-Config The IE SPS-Config is used to configure downlink semi-persistent transmission. Downlink SPS may be configured on the SpCell as well as on SCells. The network ensures SPS-Config is configured for at most one cell in a cell group.

From 38.306 V15.11.0

#### 4.2.7.10 Phy-Parameters Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. downlinkSPS Indicates whether the UE supports PDSCH reception based on semi-persistent scheduling.; UE; No; No; No.

- The yellow-highlighted description related with the network configuration in RRC is a UE capability. There may be no point for UE to implement more than one cell in a cell group, if network cannot configure so.

- Observation 1	Rel-15 SPS capability is partially captured in the TS 38.331 RRC spec.

- In Rel-16, it is agreed to support up-to 8 SPS configurations per BWP and RAN1 further introduces a restriction of up-to 32 SPS configurations in a cell group. Multiple SPS configurations per BWP is configured by the following toAddModList in BWP-DownlinkDedicated.

SPS-ConfigToAddModList-r16 ::=          SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config

- The above yellow-highlighted description is not anymore appropriate, and it is removed and revised as shown below:

From 38.331 V16.2.0

### 6.3.2 Radio resource control information elements #### – SPS-Config The IE SPS-Config is used to configure downlink semi-persistent transmission. Multiple Downlink SPS configurations may be configured in one BWP of a serving cell.

- However, the UE capability in 38.306 is not updated.

From 38.306 V16.2.0

#### 4.2.7.2 BandNR parameters Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. sps-r16 Indicates whether the UE support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group. This field includes the following parameters: - maxNumberConfigsPerBWP-r16 indicates the maximum number of active SPS configurations in a BWP of a serving cell. - maxNumberConfigsAllCC-r16 indicates the maximum number of active SPS configurations across all serving cells in a MAC entity. The UE can include this feature only if the UE indicates supports of downlinkSPS.; Band; No; N/A; N/A. #### 4.2.7.10 Phy-Parameters Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. downlinkSPS Indicates whether the UE supports PDSCH reception based on semi-persistent scheduling.; UE; No; No; No.

- One can understand sps-r16 is related with the support of multiple SPS in one BWP. Thus, if UE indicates the support of downlinkSPS but not sps-r16, it is not clear if UE can support more than one SPS-Config in a cell group (but only one SPS-Config per cell) which is now allowed in the RRC signalling in Rel-16. The root cause of the confusion is that an RRC description captures a UE capability (see Observation 1) and this RRC description is subsequently updated in the later releases.

- It is our understanding that, for the same capability bit, we should keep the same understanding in Rel-16 as in Rel-15.  Therefore, we propose to update the field description of downlinkSPS in TS 38.306 (both Rel-15 and Rel-16) to include what has been written in Rel-15 RRC spec.

- Proposal 1	Clarify in TS 38.306 (Rel-15 and Rel-16) that, for capability downlinkSPS, UE supports one SPS config configuration per cell group.

The TP can be found in the Annex 4.

# Conclusion

In the previous section, we have the below observation:

- Observation 1	Rel-15 SPS capability is partially captured in the TS 38.331 RRC spec.

In the previous section, we made the following proposals:

- Proposal 1	Clarify in TS 38.306 (Rel-15 and Rel-16) that, for capability downlinkSPS, UE supports one SPS config configuration per cell group.

# TP

38.306 V16.2.0

#### 4.2.7.10	Phy-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| downlinkSPS Indicates whether the UE supports PDSCH reception based on semi-persistent scheduling. One SPS configuration is supported per cell group. | UE | No | No | No |

38.306 V15.11.0

#### 4.2.7.10	Phy-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| downlinkSPS Indicates whether the UE supports PDSCH reception based on semi-persistent scheduling. One SPS configuration is supported per cell group. | UE | No | No | No |