3GPP TSG-RAN WG2 Meeting # 112e	R2-2010070

# Electronic Meeting,  Nov 2-13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0040 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing Updates for Aperiodic UL SRS Support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Pos-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Aperioidc UL SRS support has been agreed in RAN3. The stage 2 support for UL-SRS is currently missing. Alignments with stage 3 NRPPA TS 38.455 for the support of Aperiodic positioning |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aperiodic positioning support has been added to procedures that use aperiodic UL SRS. Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: Aperiodic UL SRS Transmission Inter-operability: If the NW implements this CR but the UE does not: - Aperiodic UL SRS may not be transmitted If the UE implements this CR but the NW does not: - Aperiodic UL SRS may not be configured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality for aperiodic UL SRS transmission |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.2.4, 8.13.2.3, 8.13.3.3a, 8.14.2.3, 8.14.3.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

#### 8.10.2.4	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-1.

Table 8.10.2.4-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Number of requested SRS resource sets and SRS resources per set

Pathloss reference: - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

The TRP measurement request information that may be signalled from the LMF to the gNBs is listed in Table 8.10.2.4-2.

Table 8.10.2.4-2: TRP Measurement request information that may be transferred from LMF to gNBs.

Information

TRP ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-3.

Table 8.10.2.4-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger - Activation time

Next Change

##### 8.10.3.2.3	Positioning Activation/Deactivation Procedure

The purpose of this procedure is to enable the LMF to request activation and deactivation of UL-SRS transmission of the target UE.

Figure 8.10.3.2.3-1 shows the messaging between the LMF and the gNB to perform this procedure.

Figure 8.10.3.2.3-1: Positioning Activation/Deactivation Procedure.

(1)	The LMF sends the NRPPa Positioning Activation Request message to the serving gNB of the target UE to request UL-SRS activation for the target UE. For a semi-persistent UL-SRS, the message includes an indication of an UL-SRS resource set to be activated and may include information that indicates the spatial relation for the semi-persistent UL-SRS resource to be activated, as listed in Table 8.10.2.4-3. For an aperiodic UL-SRS, the message includes a DCI code point indication for aperiodic UL-SRS transmission.

(2)	For semi-persistent UL-SRS, the serving gNB may then activate the configured semi-persistent UL-SRS resource sets by sending the SP Positioning SRS Activation/Deactivation MAC CE command as specified in TS 38.211 [39]. For aperiodic UL-SRS, the serving gNB may then DCI trigger as specified in TS 38.211 [39].
If the UL-SRS has been successfully activated as requested in step 1, the gNB sends the NRPPa Positioning Activation Response message to the LMF with a time indication. If the serving gNB is not able to fulfil the request from step 1, it returns the Positioning Activation Failure message indicating the cause of the failure.

(3)	If a previously activated UL-SRS should be deactivated, the LMF sends the NRPPa Positioning Deactivation message to the serving gNB of the target device to request deactivation. This message includes an indication of the UL-SRS resource set to be deactivated.

Next Change

#### 8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger - Activation time

Next Change

#### 8.13.3.3a	Positioning Activation/Deactivation Procedure

The purpose of this procedure is to enable the LMF to request activation and deactivation of UL-SRS transmission of the target UE.

Figure 8.13.3.3a-1 shows the messaging between the LMF and the gNB to perform this procedure.

Figure 8.13.3.3a-1: Positioning Activation/Deactivation Procedure.

(1)	The LMF sends the NRPPa Positioning Activation Request message to the serving gNB of the target UE to request UL-SRS activation for the target UE. For a semi-persistent UL-SRS, the message includes an indication of an UL-SRS resource set to be activated and may include information that indicates the spatial relation for the semi-persistent UL-SRS resource to be activated, as listed in Table 8.13.2.3-3. . For an aperiodic UL-SRS, the message includes a DCI code point indication for aperiodic UL-SRS transmission.

(2)	For semi-persistent UL-SRS, the serving gNB may then activate the configured semi-persistent UL-SRS resource sets by sending the SP Positioning SRS Activation/Deactivation MAC CE command as specified in TS 38.211 [39]. For aperiodic UL-SRS, the serving gNB may then DCI trigger as specified in TS 38.211 [39].
If the UL-SRS has been successfully activated as requested in step 1, the gNB sends the NRPPa Positioning Activation Response message to the LMF with a time indication. If the serving gNB is not able to fulfil the request from step 1, it returns the Positioning Activation Failure message indicating the cause of the failure.

(3)	If a previously activated UL-SRS should be deactivated, the LMF sends the NRPPa Positioning Deactivation message to the serving gNB of the target device to request deactivation. This message includes an indication of the UL-SRS resource set to be deactivated.

Next Change

#### 8.14.2.3	Information that may be transferred from the LMF to gNB

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.14.2.3-1.

Table 8.14.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Number of requested SRS resource sets and SRS resources per set

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.14.2.3-2.

Table 8.14.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.14.2.3-3.

Table 8.14.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger - Activation time

Next Change

#### 8.14.3.3a	Positioning Activation/Deactivation Procedure

The purpose of this procedure is to enable the LMF to request activation and deactivation of UL-SRS transmission of the target UE.

Figure 8.14.3.3a-1 shows the messaging between the LMF and the gNB to perform this procedure.

Figure 8.14.3.3a-1: Positioning Activation/Deactivation Procedure.

(1)	The LMF sends the NRPPa Positioning Activation Request message to the serving gNB of the target UE to request UL-SRS activation for the target UE. For a semi-persistent UL-SRS, the message includes an indication of an UL-SRS resource set to be activated and may include information that indicates the spatial relation for the semi-persistent UL-SRS resource to be activated, as listed in Table 8.14.2.3-3. For an aperiodic UL-SRS, the message includes a DCI code point indication for aperiodic UL-SRS transmission.

(2)	For semi-persistent UL-SRS, the serving gNB may then activate the configured semi-persistent UL-SRS resource sets by sending the SP Positioning SRS Activation/Deactivation MAC CE command as specified in TS 38.211 [39]. For aperiodic UL-SRS, the serving gNB may then DCI trigger as specified in TS 38.211 [39].
If the UL-SRS has been successfully activated as requested in step 1, the gNB sends the NRPPa Positioning Activation Response message to the LMF with a time indication. If the serving gNB is not able to fulfil the request from step 1, it returns the Positioning Activation Failure message indicating the cause of the failure.

(3)	If a previously activated UL-SRS should be deactivated, the LMF sends the NRPPa Positioning Deactivation message to the serving gNB of the target device to request deactivation. This message includes an indication of the UL-SRS resource set to be deactivated.

End of Changes