3GPP TSG RAN WG2 Meeting #112e           		      	    	      R2-2010104

Electronic meeting, Nov 2-13, 2020

Agenda item:	8.7.3.4

Source:		Intel Corporation

Title:	 Release procedure for SL Relaying support

Document for:	       Discussion

# 1	Introduction

RAN2#111e kicked off the study item on Sidelink Relay [1] in NR with several agreements related to scope, discovery, L2 and L3 relaying concepts. Multiple detailed post email discussions were held to cover the different topics. In this contribution, we plan to discuss the service continuity or path switching procedure and primarily focus on graceful release to support SL Relaying.

# 2	Discussion

As part of the post-meeting R2#111e email discussion#621, service continuity was discussed in detail for both L2 and L3 Relaying including details on requirements, scenarios (mobility) and related solutions. There are two cases considered for service continuity including switching from Relay link (indirect path) to Uu cell (direct path) and switching from Uu cell (direct path) to Relay link (indirect path). Intra-gNB scenario is to be prioritized for L2 relaying with inter-gNB and group mobility to be studied in WI phase, if needed. Further details about service continuity are covered in our companion paper [2].

## 2.1	Path switching and PC5 link release for L2 relaying

When the procedure for the UE switching from relay/indirect path to Uu cell was put forth, it was suggested that the PC5 link path would be released after the UE switches to the Uu link. PC5-RRC connection is a logical connection between two UEs (with source and destination layer-2 IDs) which is considered established upon corresponding PC5 unicast link establishment in upper layer (e.g. V2X). As per TS 23.287, PC5 link release is supported at upper layer as shown in figure 2.1.1. Therefore, when the UE switches to the Uu link, the PC5 ‘connection’ release to help UE exit the relaying link could potentially be done at upper layer. However, it is to be noted that in general, this link release is triggered at the upper layer and informed to the lower layers.

Figure 2.1.1. Figure 6.3.3.3-1: Layer-2 link release procedure [TS 23.287]

Observation 2.1.1: PC5 link release is supported in upper layer as per TS 23.287.

During the email discussion, the first figure (figure 5.1) as shown in annex is proposed which includes a step (i.e. step 7) to introduce PC5 link release, if needed. Since all the steps in the procedure are AS-layer steps, we wonder if the step 7 corresponds to AS layer action as well. Considering this, it would be good to clarify whether we want to introduce ‘PC5 link release’ as an explicit PC5-RRC message in AS layer when we do not have a corresponding PC5 link establishment/setup request in AS layer. It could be that it is an indication from AS layer that can be sent to upper layer which can in turn initiate the link release procedure as already supported in the SA2 specifications. This indication can also act as a trigger for the UE to switch the data path to ensure service continuity.

Proposal 2.1.1: Clarify whether we can reuse layer-2 link release as supported in sidelink in upper layer for path switching procedure in SL relaying.

Proposal 2.1.2: Provide an indication from AS layer to perform layer-2 link release at upper layer upon direct link establishment.

## 2.2	Path switching and Uu link release for L3 relaying

In the email discussion (#621), the path switching procedure when the UE switches from the direct path to indirect relaying path is also discussed and it proposes the procedure as shown in the figure 5.2 in annex. As can be seen, UE’s release from Uu link is not considered here. This is because for L2 relaying, the switching can be kept seamless as the network is involved. On the other hand, while a make-before-break can be performed for L3 relaying once the relay link is established, it would be beneficial to break the Uu link in a graceful manner. This trigger point is shown in figure 2.1 below as step 7.

Figure 2.1 L3-relaying based path switching from Uu link to relay link

While it can be assumed that the network would eventually release the UE upon inactivity or the UE will move out-of-coverage of the network, we feel that a release message from the network would enable the UE to receive appropriate release parameters before losing the connection. Furthermore, it may aid service continuity, by being a trigger to switch the paths before the UE moves out-of-coverage (and receiving those parameters). Upon switching to the relay link, we think the UE may request the gNB to release its Uu link appropriately. This particular aspect is actually being discussed as part of Multi-SIM for LTE/NR (MUSIM) work item wherein UEs may have multiple SIMs and move from one network to another. Coordinated leaving is a key issue within the realm of this work wherein UE assistance information message is proposed as one of the solutions to be considered for UE-initiated leaving.

Observation 2.2.1: The timely release of Uu link via UE-initiated mechanism may be beneficial for L3-based relaying while performing path switching from Uu path to relay path to enhance service continuity.

Path switching in L3-based sidelink relaying can thus benefit from the solution identified for coordinated leaving in MUSIM in that the UE may request the network to release its Uu connection while informing that it is switching to the relaying path.

Proposal 2.2.1: Consider UE-initiated Uu link release for support of L3 relaying while path switching from Uu path to relay path to enable service continuity.

- 3	Conclusions

In this contribution, we provide our views on the release procedure for performing path switching in sidelink relaying and have the following observations and proposals:

Observation 2.1.1: PC5 link release is supported in upper layer as per TS 23.287.

Observation 2.2.1: The timely release of Uu link via UE-initiated mechanism may be beneficial for L3-based relaying while performing path switching from Uu path to relay path to enhance service continuity.

Proposal 2.1.1: Clarify whether we can reuse layer-2 link release as supported in sidelink in upper layer for path switching procedure in SL relaying.

Proposal 2.1.2: Provide an indication from AS layer to perform layer-2 link release at upper layer upon direct link establishment.

Proposal 2.2.1: Consider UE-initiated Uu link release for support of L3 relaying while path switching from Uu path to relay path to enable service continuity.

- 4	References

[1] RP-193253, SID for Sidelink Relay

[2] R2-2009271, Further details on service continuity for Relaying, Intel corporation, RAN2#112e

- 5	Annex

Figure 5.1. Figure 2-2: Procedure for remote UE switching to direct Uu cell

[Reference 111e#621 email discussion]

Figure 5.2. Figure 2-3: Procedure for remote UE switching to indirect relay UE

[Reference 111e#621 email discussion]