3GPP TSG-RAN WG2 #112-e	Tdoc R2-2010159

Online, November 2nd – 13th 2020

Agenda Item:	8.4.2

Source:	Ericsson

Title:	On Topology-wide Fairness, Multi-hop Latency and Congestion Mitigation

Document for:	Discussion, Decision

# 1	Introduction

To clarify the scope of “Topology, routing and transport enhancements” objective of Rel-17 IAB WI and determine technical issues, RAN2 has started an email discussion ([Post111e#902][eIAB]).

This paper discusses the issues highlighted by different companies under the scope of Post111e#902 and presents our viewpoint on these issues.

# 2	Discussion

This section provides an overview of each topic covered in the email discussion Post111e#902 and our perspective on these topics.

## 2.1	Topology-wide fairness

The issue of fairness for multi-hop IAB network was discussed during the IAB SI phase and some options for how to handle fairness are included in TR 38.874.

Regarding how to enable fairness in IAB network, two approaches seem to emerge from the RAN2 email discussion on topology-wide fairness:

- The first approach discussed is to first determine to what extent the Rel-16 specs can already today support topology-wide fairness and what aspects of topology-wide fairness.

- The second approach discussed is to specify new procedure/signalling in IAB Rel-17 for enabling topology-wide fairness since Rel-16 specs cannot support topology-wide fairness.

In our view, RAN2 should follow the first approach by discussing what level of fairness can already be achieved by using Rel-16 specs and then focus on what needs to be introduced in Rel-17 for fairness, which will help in minimizing the specification efforts.

When considering the second approach, there are several ways of acquiring the information needed to enable topology-wide fairness in an IAB network. 
In some papers, e.g. [1], it is claimed that knowing the number of UEs/DRBs conveyed in the BH RLC channel can be helpful, at least in the N:1 DRB-to-BH RLC channel mapping. The BH RLC channel that conveys more bearers should be treated differently by the scheduler than the bearers that convey little traffic. The rationale is to eventually provide fair treatment of the individual UEs/DRBs, irrespective of whether they are served by a BH RLC channel that may convey a lot of UEs/DRBs or a little. It is then proposed that the CU should provide the DU with information on the average or actual number of DRBs conveyed in the BH RLC channel. 
At a first glance, it seems like a sensible approach, however, if the CU is required to provide the various IAB nodes with such information, whenever there is an update in the number of DRBs/UE, that might require significant signalling over the F1. As such, it is not clear what is the advantage of this approach compared with an approach in which the CU simply reconfigures via F1 the BH RLC channels, whenever there is a significant imbalance in the traffic load (i.e., number of UE DRBs) carried by such BH RLC channels. Additionally, taking into account the overall (actual or average) number of UEs/DRBs that may be conveyed in one BH RLC channel is not entirely correct. The traffic patterns of such UEs/DRBs might be very much different from each other, depending not only on the QoS of the traffic, but also on the traffic model generated by the end-user, the radio conditions, the congestion situations at the various intermediate IAB nodes, the number of hops apart, etc. Assuming that a BH RLC channel is always at “full load”, i.e. all packets of this BH RLC channel always convey data for all the UEs/DRBs associated with this BH RLC channel, is not a correct assumption in a real-world scenario.

Providing the IAB node with information related to the DRBs/UEs served by a BH RLC channel might require frequent signalling updates. It is not clear the advantage of such approach compared with an approach in which the CU simply reconfigures the BH RLC channels when there is a high imbalance in the traffic they are handling.

Assuming that a BH RLC channel is always at “full load”, i.e. the BH RLC channel continuously conveys all the traffics for all DRBs/UEs, is not realistic. The traffic conveyed by one BH RLC channel for the various UEs/DRBs may vary over time depending on a number of factors.

Therefore, as mentioned, topology-wide fairness can be achieved by legacy tools, at least to some extent, e.g. by simply letting the CU to properly balance the traffic across the available BH RLC channels. If more granularity is required, RAN2 could consider to more efficiently utilize the extended LCID space (i.e., 16 bits) for IAB networks. For example, establishing several BH RLC channels rather than one BH RLC channel for carrying all UE bearers traffic served by an IAB node. In addition, the IAB-donor CU can map the same number of UE bearers to each BH RLC channel and can then maintain/update this mapping whenever the network status changed due to either new UEs connecting to or some existing UEs depart from the network.

Existing Rel-16 specs allow a certain degree of topology-wide fairness in an IAB network.

RAN2 to discuss which legacy Rel-16 tools can be adopted by the network to enable topology-wide fairness, before discussing an enhancement in Rel-17.

Regarding the metrics to be used for quantifying the topology-wide fairness, in our view, the 5G QoS framework already provides all the necessary metrics for enforcing the QoS (over RAN) and proper handling of the different services. Hence, RAN2 does not need to define/introduce new KPIs for fairness or QoS fulfillment. Instead, RAN2 should focus on specifying tools to facilitate the fulfillment of fairness and legacy QoS requirements in the IAB network.

RAN2 to first identify gaps (if any) in the existing legacy and if needed specify tools to facilitate the fulfillment of fairness and legacy QoS requirements, rather than introducing new KPIs for IAB network.

In any case, if RAN2 eventually decides to pursue some enhancements to the current framework, such enhancements should rely as little as possible on F1 signalling, because as observed in Observation 1 and Observation 2, that might not be very efficient, and would not give significant gains compared with existing techniques.

## 2.2	Multi-hop latency

Several issues related to minimizing multi-hop latency have been raised in RAN2 email discussion Post111e#902. Some of these issues seem quite complex while the potential benefits are not clear. Besides, some of these issues such as “enhancement in pre-emptive BSR procedure” have been thoroughly discussed in Rel-16 and it is not clear what additional improvement can be made for Rel-17 IAB network. Ideally, the pre-BSR could achieve minimum latency for packets since the next nodes in the upstream direction know exactly the amount of data that is coming.

In general, the network provides a configuration to meet the required QoS on a bearer basis. The Rel-16 IAB-donor CU knows the network topology under its domain and each backhaul channel and each route that is set up may have different levels of QoS, and hence can lead to different latency. When routing packets, the network may choose an appropriate route to meet the QoS requirements including latency requirements. In addition, the pre-BSR might be able to assist the network to minimize UL latency. Moreover, the network nodes, e.g., intermediate IAB nodes could discard a packet if they cannot commit to the PDB or other QoS parameters.

PDB management is part of the network functions to commit and deliver the QoS of a service.

Proper configuration of the BH bearers and routes are tools to deliver the committed QoS parameters and an ideal pre-emptive BSR can provide minimum UL latency.

Additionally, for ensuring PDB management, according to Rel.16 specifications, it is already possible for the CU to configure via F1 the IAB node with a PDB requirement per BH RLC channel. Since the Rel-16 IAB-donor CU knows the network topology under its domain, it can configure the PDB HbH at each node along the path towards an access IAB node. On the basis of this information, a network node can discard a packet if its scheduler cannot satisfy the PDB for the associated BH RLC channel, without the need to specify any specific discarding rule. Hence, before discussing enhancements to PDB management, RAN2 should first discuss what can be already achieved with Rel-16 IAB node(s)

In Rel.16, the CU can configure an intermediate IAB node with a per-hop PDB per BH-RLC channel. The IAB node can apply packet discarding policies on the basis of this information and scheduling decisions.

RAN2 to first discuss which legacy Rel-16 tools can be adopted by the network to take into account hop-by-hop latency, before discussing an enhancement in Rel-17.

If RAN2 identifies the need to enhance the current ways to handle hop-by-hop latency, it could for example extend the current PDB configuration, such that it is possible for the CU to indicate to the IAB node the PDB per BAP destination, rather than only per BH RLC channel as it is today. However, this information should be conveyed by F1 signalling, and it is up to RAN3 to decide on it. More related to RAN2, it can be discussed, if needed, whether information related to the remaining PDB for a given packet should be transferred from one node to another. However, RAN2 should aim at minimum specification changes, and avoid specifying unnecessary procedures that can be left to the IAB node implementation, e.g. whether to discard a packet or not on the basis of PDB fulfillment.

If RAN2 identifies the need to enhance latency handling, it should aim at minimum specification changes, and avoid specifying solutions that can be handle by the IAB node implementation, e.g. packet discarding.

Finally, the topic of “enhancement of LCGs per BH link” was briefly discussed during Rel-16 but was postponed to Rel-17. In our view, this topic can be discussed in the context of ensuring QoS requirements for UL traffic in a multi-hop IAB network.

The topic of “enhancement of LCGs per BH link” can be discussed in the context of ensuring QoS requirements for UL traffic in a multi-hop IAB network.

RAN2 to discuss whether LCG ID extension is needed to enhance the pre-emptive BSR operation.

## 2.3	Hop-by-hop congestion mitigation

For congestion mitigation, few topics are highlighted in the email discussion Post111e#902 for further consideration. For instance, some companies argued that local (re)routing be allowed for congestion-mitigation (apart for RLF supported by Rel-16 specs), and hence, should be discussed in RAN2. Besides, some companies made suggestions in favour of enhancements to Rel-16 HbH flow control.

Before making a hasty decision on local routing, RAN2 should discuss its potential benefits, as local decisions are made based on limited information and can lead the overall network to a suboptimal state. On the topic of enhancements to Rel-16 HbH flow control, our viewpoint is that HbH flow control was extensively discussed during Rel-16 and there was a consensus in RAN2 that HbH can only solve the temporary congestion on a local BH link, not long-term congestion. For long-term congestion, E2E flow control mechanism can be used (and improved if needed), which fell under RAN3 domain. Additional enhancements in HbH flow control might bring some minor improvement in congestion avoidance, which certainly is not worth the efforts. Rather RAN2 should prioritize its limited time units for other important topics mentioned in IAB Rel-17 WID.

HbH flow control was extensively discussed during Rel-16, and there was a consensus in RAN2 that HbH can only solve the temporary congestion on a local BH link, not long-term congestion.

Additional enhancements in HbH flow control might bring some minor improvement in congestion avoidance, which certainly is not worth the efforts.

Local routing decisions are made on limited information and hence can lead the overall network to a suboptimal state.

RAN2 to discuss the pros and cons of local routing before making any decision on this topic.

RAN2 to deprioritize enhancement to HbH flow control in Rel17.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Providing the IAB node with information related to the DRBs/UEs served by a BH RLC channel might require frequent signalling updates. It is not clear the advantage of such approach compared with an approach in which the CU simply reconfigures the BH RLC channels when there is a high imbalance in the traffic they are handling.

Observation 2	Assuming that a BH RLC channel is always at “full load”, i.e. the BH RLC channel continuously conveys all the traffics for all DRBs/UEs, is not realistic. The traffic conveyed by one BH RLC channel for the various UEs/DRBs may vary over time depending on a number of factors.

Observation 3	Existing Rel-16 specs allow a certain degree of topology-wide fairness in an IAB network.

Observation 4	PDB management is part of the network functions to commit and deliver the QoS of a service.

Observation 5	Proper configuration of the BH bearers and routes are tools to deliver the committed QoS parameters and an ideal pre-emptive BSR can provide minimum UL latency.

Observation 6	In Rel.16, the CU can configure an intermediate IAB node with a per-hop PDB per BH-RLC channel. The IAB node can apply packet discarding policies on the basis of this information and scheduling decisions.

Observation 7	The topic of “enhancement of LCGs per BH link” can be discussed in the context of ensuring QoS requirements for UL traffic in a multi-hop IAB network.

Observation 8	HbH flow control was extensively discussed during Rel-16, and there was a consensus in RAN2 that HbH can only solve the temporary congestion on a local BH link, not long-term congestion.

Observation 9	Additional enhancements in HbH flow control might bring some minor improvement in congestion avoidance, which certainly is not worth the efforts.

Observation 10	Local routing decisions are made on limited information and hence can lead the overall network to a suboptimal state.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss which legacy Rel-16 tools can be adopted by the network to enable topology-wide fairness, before discussing an enhancement in Rel-17.

Proposal 2	RAN2 to first identify gaps (if any) in the existing legacy and if needed specify tools to facilitate the fulfillment of fairness and legacy QoS requirements, rather than introducing new KPIs for IAB network.

Proposal 3	RAN2 to first discuss which legacy Rel-16 tools can be adopted by the network to take into account hop-by-hop latency, before discussing an enhancement in Rel-17.

Proposal 4	If RAN2 identifies the need to enhance latency handling, it should aim at minimum specification changes, and avoid specifying solutions that can be handle by the IAB node implementation, e.g. packet discarding.

Proposal 5	RAN2 to discuss whether LCG ID extension is needed to enhance the pre-emptive BSR operation.

Proposal 6	RAN2 to discuss the pros and cons of local routing before making any decision on this topic.

Proposal 7	RAN2 to deprioritize enhancement to HbH flow control in Rel17.

# 4	References

R2-2006965, Simulations on fairness support in IAB topology, Qualcomm Incorporated, RAN2#111

R2-2007659, User plane Latency in Multi-hop IAB Systems, Ericsson, RAN2#111