3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2010184

Online, November 2nd – 13th, 2020

Title:	Draft reply LS on restricting the rate per UE per network slice

Response to:	R2-2010694/S2-2007946

Release:	Rel-17

Work Item:	FS_eNS_Ph2

Source:	Huawei [will be RAN2]

To:	SA2

Cc:	RAN3

Contact Person:

#### Name:	Jun Chen

E-mail Address:	jun.chen@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1  Overall description

RAN2 thanks SA2 for their LS on restricting the rate per UE per network slice.

RAN2 discussed listed 3 solutions and replies as follows.

- For Solution #22:

The purpose of SMBR signalled over the NG interface from AMF to RAN is for SMBR enforcement. From RAN2 perspective, RAN is able to perform DL SMBR enforcement by scheduling. However, it may need some enhancement for RAN to perform UL SMBR enforcement due to RAN’s unawareness of the UL data volume of the UE in a certain slice and the UE-level UL grant allocation currently. We expect it to be further evaluated by RAN in a future SLA related topic.

- For Solution #37:

According to the definition of SMBR, the session AMBR is calculated based on the SMBR. UE AMBR accounts for the sum of all session AMBR of all PDU sessions. Therefore, it is useless providing SMBR to RAN for UE AMBR calculation in this solution.

- For Solution #43:

This solution is for KI #4 “Support for network slice quota event notification in a network slice”, not for KI #3.

Solution #43 is based on solution#22, i.e., SMBR should first be signalled to RAN over NG interface. After that, the notification of the reached SMBR from RAN to AMF is feasible, which will also have RAN3 impact.

The frequency of the SMBR notification depends on the network slice planning and the traffic pattern of the UE.

2  Actions

To SA2:

ACTION: 	RAN2 kindly asks SA2 to take the above information into account.

3  Date of Next TSG-RAN WG2 Meetings:

RAN2#113-e             Electronic meeting		 25th Jan – 5th Feb, 2021

RAN2#113-b-e          Electronic meeting	12th Apr – 20th Apr, 2021